SA WG2 Meeting #171	 S2-2508704

13 – 17 Oct, 2025, Wuhan, China	(revision of S2-2508110)

Source:	Samsung

Title:	[General aspects] Architectural Assumptions for 6G

Document for:	Approval

Agenda Item:	20.6.0

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes Architectural Assumptions on top of the postponed the paper (S2-2508110)

**** First Change (all text new) ****

3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

6G Core Network (6G CN): In this study, it refers to the Core Network for the 6G System.

NOTE: Whether 6G CN can be considered as evolution of 5GC or as a new Core Network will be determined during the study.

6G RAN: In this study, it refers to the radio access network for 6G radio access technology (6GR) being studied by 3GPP RAN WGs [ref: RP-251881].

6G System (6GS): 3GPP System consisting of 6G RAN, non-3GPP access, 6G Core Network and UE.

**** Next Change (all text new) ****

4.1	Architectural Assumptions

Editor's Note: This clause documents the common architecture assumptions identified for the study.

The following is assumed for 6GS architecture:

1.	The framework of SBA specified for 5GC is assumed as a starting point for discussion.

2. 	This study assumes that control and user plane of the 6G RAN connects to a single core network. It is assumed that the 6G RAN is a single technology framework based on a stand-alone architecture.

3.	The 6G system is assumed to maintain the existing RAN and CN functionality split as supported in 5GS, and avoid duplication of functionality in 6G RAN and CN.

NOTE X1: SA2 System Architecture decision on RAN-CN Interface (e.g., whether P2P or SBI) will be determined before RAN check point (i.e., TSG#112 June 2026).

4.	Support of multi-media services such as voice, video (essential and regulatory services) will be using IMS.

5. 	It is assumed the NFs in 6G CN may be newly defined or re-designed or extend 5GC NFs. In this case, the stand-alone architecture of NG-RAN as defined in TS 23.501 [x] can be connected to the 6G CN.

NOTE X2: What parts of 5GC can be re-used or require re-design is to be determined as part of this study.

NOTE X3:  SA2 System architecture decision on whether NG-RAN can be connected to the 6G CN will be determined before RAN check point (i.e. TSG#112 June 2026).

**** End of Changes ****