3GPP TSG-SA WG2#171	S2-2508752

Wuhan, China, 13-17 October 2025

Source:	Nokia

# Title:	KI#3: Support of IMS Emergency Call over GEO

# Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This contribution proposes how IMS emergency calls could be supported over NB-IoT NTN via GEO satellite for FS_5GSAT_Ph4_ARC.

# 1. Discussion

The KI#3 Support of IMS emergency call over NB-IoT NTN via GEO satellite connecting to EPC in 23.700-19 states that the following aspect is to be studied:

-	How to support IMS emergency call over NB-IoT NTN via GEO satellite connecting to EPC to comply with the relevant requirements defined in TS 22.261 [8] and TS 22.101 [9].

# 2. Text Proposal

It is proposed to capture the following changes for KI#3 into the FS_5GSAT_Ph4_ARC TR 23.700-19.

# >>>>BEGINNING OF CHANGES<<<<

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 |
| X |  |  | X |  |  |
| - |  |  |  |  |  |
| - |  |  |  |  |  |
| - |  |  |  |  |  |
| - |  |  |  |  |  |

# >>>>NEXT CHANGE (new text)<<<<

## 6.X	Solution #X: IMS Emergency Call over GEO

### 6.X.0	High-level solution Principles

This solution aims to resolve Key Issue #3, i.e. " Support of IMS emergency call over NB-IoT NTN via GEO satellite connecting to EPC " and comprises the following principles:

-	MME shall be able to indicate support for emergency bearer services using the Emergency Service Support indicator in the Attach and TAU procedures to a UE that accesses the network using NB-IoT via a GEO satellite.

-	The NB-IoT cell of the GEO satellite shall be able to indicate support for emergency services in its broadcast information.

-	The procedures for UE Detectable Emergency Session and non-UE Detectable Emergency Session described in TS 23.167 [7] can also be used for the case that the emergency voice calls are established using NB-IoT over a GEO satellite.

-	The enhanced interface Gm*, described in the solution 19 of TR 23.700-19, can also be used for the procedures related to the establishment of IMS emergency session, which are specified in section 7 of 3GPP TS 23.167 [7].

### 6.X.1	Description

According to TS 23.401 [5] and specifically the clause “4.3.12 IMS Emergency Session Support”, the NB-IoT does not natively support emergency voice calls, as shown below:

- “Support for emergency bearer services is not available when the UE is using NB-IoT, i.e. the MME shall not indicate support for emergency bearer services using the Emergency Service Support indicator in the Attach and TAU procedures to a UE that accesses the network using a RAT Type equal to NB-IoT, and an NB-IoT cell shall not indicate support for emergency services in any broadcast information in AS.”

However, according to solutions under discussion in Key Issue #1 and Key Issue #2, the above restriction can be removed, and IMS Emergency sessions can be supported using NB-IoT over a GEO satellite connected to the EPC. Hence, more specifically, the MME shall be able to indicate support for emergency bearer services using the Emergency Service Support indicator in the Attach and TAU procedures to a UE that accesses the network using a RAT Type equal to NB-IoT. And the NB-IoT cell of the GEO satellite shall be able to indicate support for emergency services in the broadcast information in the access stratum.

At the IMS signalling, the solutions for UE Detectable Emergency Session and non-UE Detectable Emergency Session described in TS 23.167 [7] could also be used for the case where emergency voice calls are established using NB-IoT over a GEO satellite. More specifically, considering whether the UE can detect the emergency session and/or whether there is already a prior IMS registration, the following options can be considered.

- UE is already IMS Registered: The UE can send the IMS emergency session request, as specified in clause 7.3 of TS 23.167 [7].

- However, in the case where the UE is roaming, it would be beneficial to avoid rejecting the IMS emergency session request, as it is indicated in clause 7.3 of TS 23.167 [7], which will increase the emergency call delay (i.e., since the P-CSCF shall request that the UE perform an IMS emergency registration). Hence, it is suggested to follow the procedure described in clause 7.4 of TS 23.167. If the UE is connected via GEO, the UE will send an emergency request to the P-CSCF of the visited PLMN, and the emergency request will be forwarded to the E-CSCF of the visited PLMN.

- UE is not IMS Registered: There are two options to facilitate the establishment of the emergency voice call using NB-IoT over a GEO satellite:

- the UE firstly initiates the IMS Emergency registration procedure as described in clause 7.2 of TS 23.167 [7], and then the UE sends the IMS emergency session request, which increases the delay for establishing the emergency call.

- the UE can use the IMS Emergency Session Establishment without Registration procedure as described in clause 7.4 of TS 23.167 [7].

- Non-UE Detectable Emergency Session: In case the UE cannot detect the emergency session, the non-UE detectable emergency sessions can be handled as described in Clause 7.1.2 of TS 23.167 [7].

### 6.X.2	Procedures

#### 6.X.2.1	IMS Emergency session via Gm*

The enhanced interface Gm*, as it is described in the solution 19, can also be used for the procedures related to the establishment of IMS emergency session, as they are specified in section 7 of TS 23.167 [7].

In the following example, it is assumed that the UE is registered to the IMS and an Emergency Call using NB-IoT over GEO shall be established.

Figure 1: Procedure for Emergency Call using NB-IoT over GEO via Gm*

- The UE detects the request for the establishment of an emergency session.

- The UE requests emergency PDN connection for emergency service to the MME and the latter selects the appropriate P-GW based on the emergency configuration data. The MME sends a create session request message to the selected P-GW. The P-GW allocates the UE's IP address and establishes an IP-CAN session with the PCRF, where the UE's IP address, IMSI/IMEI/MSISDN, are associated with the PDN connection for emergency purposes.

The EPS bearer (CN and RAN) is established using existing procedures.

- The UE, after establishing the PDN for emergency, sends a Gm* INVITE message with an emergency indication to the P-CSCF, including its IP address, its location (if available), and the IMPU.

The P-CSCF can identify that this is an emergency session request from the dialled number. The P-CSCF contacts the PCRF for UE Authentication by mapping UE’s IP address with IMSI/MSISDN. The UE location may also be validated.

NOTE: the UE location can be included in the Gm* INVITE message. Or it can be retrieved by the network via UE location reporting (e.g., using GNSS or RAN-assisted NTN positioning means as these may be defined in RAN WGs).

The IMS network uses the location information to route the emergency session request towards the appropriate PSAP. The P-CSCF transforms the Gm* message into a regular SIP signalling message and sends the INVITE message with the emergency indication to the appropriate PSAP.

The PSAP sends a SIP 200 OK to the IMS core, which is then sent to the UE via the Gm* interface.

- The emergency call is established using NB-IoT via the GEO satellite.

### 6.X.3	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

UE:

- The UE requests emergency PDN connection for emergency service to the MME using NB-IoT via GEO satellites that indicate support for emergency services in broadcast information.

- UE to support IMS emergency session procedures using Gm* interface.

RAN

- Broadcast support of emergency services using NB-IoT via GEO satellite.

MME:

- The MME needs to indicate support for emergency bearer services using the Emergency Service Support indicator in the Attach and TAU procedures using NB-IoT via GEO satellite.

P-CSCF:

-	Support receiving from the UE using Gm* interface information required for IMS emergency session

# >>>>END OF CHANGES<<<<