3GPP TSG-SA WG2#171	S2-2508753

Wuhan, China, 13-17 October 2025	(was S2-2506435)

Source:	Nokia

# Title:	New Solution for Support of Short Message to Emergency Response Centre via SMS over IP

# Document for:	Approval

Agenda Item:	20.8.1

Work Item / Release:	FS_SMS2EC_ARC / Rel-20

Abstract of the contribution: This contribution proposes a new solution for FS_SMS2EC_ARC.

# 1. Discussion

The KI#3 SMS over IP Routing to Appropriate PSAP in 23.700-65 states the following:

The following aspects are to be studied:

-	how and when to trigger location determination and location data sharing when sending short messages using SMS over IP for routing to Emergency Response Centre, considering both 3GPP and non-3GPP access scenarios;

-	how to route short messages using SMS over IP to Emergency Response Centre serving the UE’s location, according to the local regulations (in roaming case, the regulations applicable to the UE’s location);

-	how to support PSAP callback capabilities for sending and routing regular short messages using SMS over IP.

The key issue covers SMS delivery via IP both in EPS and 5GS.

NOTE 1:	The solutions for EPS and 5GS are based on same principles.

The key issue covers both roaming and non-roaming scenarios.

The proposed solution solves the short messages using SMS over IP to Emergency Response Centre to PSAP based on the emergency service type and location.

# 2. Text Proposal

It is proposed to capture the following changes into TR 23.700-65.

# * * * * First change (all new text) * * * *

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 |
| X |  |  | X |
| - |  |  |  |
| - |  |  |  |
| - |  |  |  |
| - |  |  |  |

## 6.X	Solution #X: Support of Short Message Service to Emergency Response Centre via SMS over IP

### 6.X.0	High-level solution Principles

This solution aims to resolve Key Issue #2, i.e. "Support of Short Message Service to Emergency Response Centre via SMS over IP". The solution principles shall follow the architectural assumptions as described in clause 4.

To address the issue described in WT-1, the following solutions are proposed:

-	UE sends a new indication during Attach/Registration procedure with EPS/5GS, whether the UE supports emergency SMS over IP and/or emergency SMS over NAS.

-	When the serving network receives the above indication from the UE, the serving network sends an indication in response to Attach/Registration procedure whether the serving PLMN supports emergency SMS over IP and/or emergency SMS over NAS.

-	A UE receiving an indication that emergency SMS over IP is supported in a serving PLMN but failed to send the SMS over IP attempts emergency SMS over NAS if configured in the UE, unless emergency SMS over IP is supported over Non-3GPP access. In this case, the UE attempts to send the emergency SMS over IP via Non-3GPP access first, prior to using emergency SMS over NAS.

-	If both emergency SMS over IP and emergency SMS over NAS are not supported in the serving network, it is up to the UE implementation how to proceed. The UE can, for example try to choose another network to send the emergency SMS over IP or SMS over NAS. The UE can also try to establish an emergency call or indicate to the user that emergency SMS over IP or SMS over NAS is not possible.

To address the issue described in WT-2, the following solutions are proposed:

-	The UE uses the same mechanisms (local configuration and handling of emergency Number/emergency service and sub-service type received over NAS during attach/registration procedure).

The UE detects emergency SMS over IP and/or SMS over NAS based on used emergency numbers. Provisioning of emergency numbers and associated types or URN information as specified in clause 4.1 bullets 2a, 2b) and 2c) in 3GPP TS 23.167 applies.

To address the emergency SMS over IP described in WT-3, the following solutions are proposed:

-	When UE detects the sos emergency number and it does not have emergency IMS registration for SMS then UE shall complete emergency IMS registration for SMS. After completing the emergency IMS registration procedure, the UE sends an SMS as a SIP MESSAGE containing emergency SMS in the payload to the P-CSCF in the serving PLMN. The P-CSCF forwards the SIP MESSAGE to an E-CSCF. The E-CSCF may optionally query the LRF to obtain the UE’s location information. The E-CSCF then forwards the SIP MESSAGE, including the emergency short message, to an IP-SM-GW capable of handling emergency SMS. The IP-SM-GW selects an appropriate SMS-SC and forwards the emergency SMS to it. The SMS-SC selects an appropriate PSAP and delivers the emergency SMS to the selected PSAP based on the emergency service type.

-	Any SMS sent back by the PSAP to the UE is handled as a regular SMS.

### 6.X.1	Description

This solution addresses Key Issue #2.

-	UE performs emergency IMS registration and sends SIP MESSAGE containing an R-URI set to the emergency number or sos-URN. The payload of the SIP MESSAGE contains the emergency SM.

-	If E-CSCF queries the LRF and forwards the SIP MESSAGE to an IP-SM-GW able to handle emergency SMS.

### 6.X.2	Procedures

#### 6.X.2.1	Short Message routing to Emergency Response Centre via SMS over IP

During EPS/5GS registration, the UE indicates its capability to support SMS over IP and/or SMS over NAS. The serving network also indicates its capability to support these services in the registration response.

If the serving network has provided the above-mentioned SMS over IP indication, then UE invokes the following procedure to send a Short Message to an Emergency Response Centre.

Figure 1: Procedure for Short Message to Emergency Response Centre via SMS over IP

- UE emergency IMS registration follows the procedure as described in clause 7.2 of 3GPP TS 23.167.

- If the UE decides to send SMS over IP, then it sends a SIP MESSAGE to the P-CSCF in the serving network containing an R-URI set to SOS URN or an emergency number and shall include the location of the UE. The SIP MESSAGE contains the emergency short message.

- P-CSCF forwards the SIP MESSAGE to an E-CSCF that is capable of handling emergency SMS over IP.

- E-CSCF optionally queries the LRF to get user location information and determines a suitable PSAP address.

- E-CSCF includes the PSAP address in the SIP MESSAGE and forwards it to the IP-SM-GW configured in the E-CSCF.

5a.	If the E-CSCF sends the PSAP address to the IP-SM-GW forwards the emergency short message to the PSAP address received in step-4.

5b.	If the IP-SM-GW receives the PSAP address from E-CSCF and can indicate the same to SMSC, then it will replace the destination address and send the SMS payload to SMSC.

6.	SMSC sends the emergency SMS to local PSAP, this step is executed only if the step 5b is used to deliver emergency SMS via SMSC. 7.	PSAP may send acknowledgement to the IP-SM GW.

8-10.	 The successful response code 202 Accepted is sent from IP-SM-GW to E-CSCF -> P-CSCF -> UE.

### 6.X.3	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

UE:

- The UE needs to support sending and receiving feature support flag for emergency SMS over IP and/or SMS over NAS.

P/S/E-CSCF:

- P-CSCF needs to select E-CSCF during IMS emergency registration and route emergency SMS.

IP-SM-GW:

- IP-SM-GW needs to have the capability to forward emergency SMS to the local PSAP or local SMS-SC.

# * * * * End of change * * * *