3GPP TSG-WG SA2 Meeting #171	S2-2508757

# Wuhan, CN, 13th Oct – 17th Oct, 2025		(revision of S2-250xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6411 | rev | 1 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on Energy Consumption calculation in case of redundant transmission or PDU Session with multiple PDU Session Anchors |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys |  |  |  |  |  | Date: |  |  | 2025-10-3 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | As per TS 23.501 clause 5.51, energy consumption of a PDU Session or Service Data Flow at UPF(s)/gNB(s) is proportional to the data volume it consumed. SMF provides the data volume reported by PSA UPF, along with gNB ID(s) and (I-)UPF ID(s) to EIF for energy consumption calculation. However, significant inaccuracy may occur under following scenarios: 1. When redundant transmission is used for a PDU Session or Service Data Flow, redundant packets are not included in Usage Reports during N4 reporting procedure as specified in TS 29.244: “The following requirements shall apply when using Redundant Transmission on N3/N9 interfaces or Redundant Transmission at transport layer: - the UPF shall not count redundant packets in Usage Reports (e.g. Volume Measurement), i.e. it shall count the traffic only once in Usage Reports;” In this case, the data volume reported by UPF is not reflecting the actual value it handled, which will lead to energy consumption calculation error in EIF in case of redundant transmission. 2. In case of a PDU Session with multiple PDU Session Anchors, the data volume handled by gNB and I-UPF is equal to the sum of data volume of multiple PDU Session Anchors. Simply providing the gNB(s) ID and (I-)UPF ID(s) along with the data volume reported by PSA UPF may lead to misunderstanding of data volume handled by gNB and I-UPF in EIF. This CR proposes to fix the energy calculation error in aforementioned scenarios. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the description on how does the SMF handle data volume reported from UPF in case of redundant transmission or PDU Session with multiple UPF Anchors and provide it to EIF. E.g., when redundant transmission is used, the data volume provided to EIF shall also include the duplicated traffic. Add an optional entry NF specific UL and/or DL Data Volume in the Information from SMF for user-plane energy consumption calculation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect Energy Consumption calculation at EIF and may lead to inappropriate policy decision based on it. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.51.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.51.2.2	Energy Consumption information collection

##### 5.51.2.2.1	General

The Energy Information Function (EIF) collects the UE related Energy Consumption information including Node-level energy consumption information, Node-level data volume from OAM and data volume of the required granularities (i.e. S-NSSAI, PDU Session and/or Service Data Flow) from UPF (via SMF). The EIF can expose the energy consumption at the requested granularity over a certain time window (i.e. the total energy consumed from a start time to an end time) or periodically (i.e. every reporting period of e.g. 10 minutes).

Node-level energy consumption is collected network-wide at a configurable starting time with interval T such that the Energy Consumption information reported to the EIF is time-aligned.

The reporting period for periodic reporting by the EIF to its consumers can only be a multiple of this time interval T and is time-aligned to the node-level energy consumption reporting occurring at every time interval T configured in the network.

NOTE:	If the reported Energy Consumption information is not time-aligned, it cannot be aggregated.

##### 5.51.2.2.2	Energy Consumption information collection from SMF

The serving SMFs are retrieved by the EIF from the UDM of the UE based on the provided input parameters including the UE ID and optional S-NSSAI or (S-NSSAI, DNN). Also, the EIF subscribes to the UDM to be notified with the applicable SMFs.

The EIF invokes Nsmf_EventExposure_Subscribe as defined in TS 23.502 [3] with the required granularities (UE ID, DNN/S-NSSAI, application information (e.g. Application Identifier, or Packet Filters)) to retrieve the information from SMF, which is shown in Table 5.51.2.2.2-1. The SMF receives the data volume of the required granularities from the PSA UPF. Whenever necessary for describing a specific data path scenario of a PDU Session, the SMF can generate relevant an NF specific UL and/or DL Data Volume (see Table 5.51.2.2.2-2) based on the data volume reported by the PSA UPF and provide it to the EIF as well, e.g., in case of the scenarios described in clause 5.6.4, 5.33.2.2 and 5.33.2.3.

NOTE x:	In case of redundant PDU Session, SMF uses the data volume reported by PSA UPF as Common DV UL and DL Data Volume (see Table 5.51.2.2.2-2) to represent the actual data volume handled by gNB and by I-UPF while NF specific UL and/or DL Data Volume is used to represent the actual data volume handled by PSA UPF so that both original and duplicated data volume can be represented. In case of PDU Session with multiple PDU Session Anchors, SMF uses the sum of the data volumes reported by PSA UPFs as Common UL and DL Data Volume to represent the actual data volume handled by gNB and by I-UPF while NF specific UL and/or DL Data Volumes are used for each PSA UPF.

NOTE y:	The data volumes provided by the SMF refer to the “access side” of the NFs, i.e., Uu interface for gNB, N3 interface for I-UPF and N3/N9 interfaces for PSA UPF, in order to keep them aligned with OAM measurement on Node-level data volume (see 5.51.2.2.3-1) which is used for the Energy Consumption calculation.

The SMF derives the gNB ID(s) from the AMF provided ULI as described in clause 4.3.2. If the I-SMF is involved in the PDU session, the SMF of PDU session requests I-SMF to report the current I-UPF ID and ULI and subsequent change of these entities.

The SMF then sends the data volumes, along with the serving gNB ID(s) and (I-)UPF ID(s) to EIF for energy consumption calculation. And the information collected from SMF by EIF, is shown in Table 5.51.2.2.2-2. Upon release of a PDU session, the SMF terminates subscriptions related to the PDU session. The SMF periodically notifies EIF with the information at the end of each time interval T until the EIF unsubscribes as defined in TS 23.503 [3]. The SMF shall collect data volume only for the PDU session carried over the 3GPP Access. When the PDU session is over Non-3GPP Access type the SMF reports UL/DL Data volume in the measurement period equal to 0 (zero) and the gNB ID is not provided.

NOTE 1:	The SMF can derive gNB ID(s) according to the AMF provided ULI as described in clause 4.3.2 of TS 23.502 [3].

NOTE 2:	In the case of MA PDU session, the SMF does report the UL/DL Data volume related to the traffic over the Non-3GPP access leg and the serving gNB ID refers to the traffic over the 3GPP leg.

When the SMF receives the Nsmf_EventExposure_Subscribe for a Service Data Flow from the EIF, the SMF generates a URR for the Data Volume counting with a Periodic measurement threshold set according to the Timer interval T and associates the subscription from EIF to the generated URR. The SMF is then associating the generated URR to an appropriate PDR. SMF logic ensures that the PDRs and their precedence values are configured appropriately in the UPF so that both, the URRs for Data Volume counting for EIF and the traffic handling instructions derived from the existing PCC rules can co-exist and the traffic of the Service Data Flow is treated in the same way as before. For example, the SMF may behave as follows:

-	If there is a PCC rule existing for that Service Data Flow, the SMF associates the generated URR with the PDR of that PCC rule.

-	If there is no PCC rule for the Service Data Flow, the SMF generates a PDR for the Service Data Flow and associates the generated URR with that PDR. All other instructions of the PDR related to the match-all PCC rule are copied (so that the traffic of the Service Data Flow is treated in the same way as before).

-	In all other cases, e.g. if the Service Data Flow is partially overlapping with an existing PCC rule (which can only occur when the subscription contains Packet Filters), the SMF may need to generate PDR(s), associate the generated URR with the(se) PDR(s) and copy all other instructions of the PDR related to the existing PCC rule so that both, the Data Volume counting for EIF and the traffic handling instructions derived from the existing PCC rule can be realized.

If the PCC rules are changed, the SMF shall again perform the above checks and actions.

When the SMF receives the Nsmf_EventExposure_Subscribe for the PDU Session from the EIF, the SMF associates the subscription from EIF to the PDU Session, generates a URR for the Data Volume counting (with a Periodic measurement threshold set according to the Timer interval T) and associates it with all PDRs of the PDU Session.

In both cases, the UPF will send the collected data volume information of this URR at the end of each Time interval to the SMF in a Usage Report (see clause 4.4.2.2 of TS 23.502 [3]). If there is a change in the user plane path (e.g. handover between gNBs or insertion/removal of I-UPF) during the Time interval, the SMF shall request the collected data volume information of this URR from the UPF. If this happens, the UPF will immediately send the collected data volume information to the SMF in a Usage Report (see clause 4.4.2.2 of TS 23.502 [3]) and the SMF shall then store this information together with the gNB ID and I-UPF ID that belong to the user plane path before the change happened.

Table 5.51.2.2.2-1: Information to SMF for user-plane energy consumption calculation

| Information | Description |
| --- | --- |
| UE ID | SUPI. |
| S-NSSAI +DNN | Slice and DNN applicable to a PDU session. |
| IP 5-Tuple | IP-5-tuple. |

NOTE 3:	The user-plane energy consumption information reporting interval from the SMFs is the network-wide configurable starting time and interval T.

Table 5.51.2.2.2-2: Information from SMF for user-plane energy consumption calculation

| Information | Description |
| --- | --- |
| UE IP address | UE IP address. |
| UE ID | SUPI. |
| S-NSSAI | Network Slice applicable to a UE |
| S-NSSAI +DNN | Slice and DNN applicable to a PDU session. |
| Packet Filters | Packet Filters as in clause 5.7.6 for IP or Ethernet traffic. |
| Application Identifier | Identification for the traffic of the service data flow. |
| List of Data Volume information | The data volume and the associated UPF(s) and gNB(s) serving the UE within the time interval. |
| > Common UL and DL Data Volume | The UL and DL Data Volume of a PDU Session identified by (UE-ID, S-NSSAI/DNN) or a Service Data flow (UE ID, S-NSSAI, DNN, Packet Filters/Application Identifier) handled by Network Functions. |
| > List of (I-)UPFs | The (I-)UPF(s) associated to the reported data volume. |
| >> (I-)UPF ID | Identifier of an (I-)UPF associated to a reported data volume used by a PDU Session identified by (UE-ID, S-NSSAI/DNN) or a Service Data flow (UE ID, S-NSSAI, DNN, Packet Filters/Application Identifier). |
| >> NF specific UL and/or DL Data Volume (optional) | The UL and/or DL Data Volume of the PDU Session or the Service Data Flow handled by the (I-)UPF (NOTE1). |
| > gNB ID | Identifier of the gNB serving the UE. |
| Reference to Time Interval | Indicate the time interval of the collected information (e.g. time stamps). |
| NOTE 1: When NF specific UL and/or DL Data Volume is provided, EIF shall use the NF specific UL and/or DL Data Volume instead of the Common UL and DL Data Volume for energy consumption calculation of that NF. |  |

NOTE 4:	Each entry of List of Data Volume information represents the serving gNB and UPF(s) corresponding to the same Data Volume regarding the required granularities. A new entry is added when the gNB or UPF(s) is changed.

# * * * * End of changes * * * *