3GPP SA WG2 Meeting #170	S2-2507338

# 25-29 Aug, 2025, Goteborg, SE

Source:	China Telecom

Title:	Interim principle for KI#2

Document for:	Approval

Agenda item:	20.4.1

Spec:	3GPP TR 23.700-67

Version:	0.2.0

Work Item:	FS_EnergySys_Ph2

Abstract: This pCR proposes interim agreements on KI#1 in TR23.700-67.

Introduction

Up to SA2#170 meeting, there are 16 solutions are documented in TR23.700-67 regarding KI#2. These solutions propose service adjustments for the UE based on energy consumption and energy efficiency from different aspects:

- To reuse or enhance existing 5G AM, SM, UE policy and BDT procedures for energy saving, where PCF retrieves UE's energy energy consumption (EnC) information from EIF and 5GC NF's from OAM (Solution #2, #3, #6, #13 , #16, #24 and #27).

  - Furthermore, to allow AF takes part in the energy saving decision/negotiation, i.e., the AF may indicate to the network the preferred or expected network energy saving behaviors for its applications or its UEs (Solution #5, #7, #15, #25, #26).

- New energy attribute or granularity for energy policy decision, e.g, slice level EnC (Solution #2, #6, #18), Service Data Flow level EnC (Solution #6), renewable energy (Solution #14).

- Differentiated Charging Based on Energy Efficiency Policy Decision (Solution #17) and UE category (Solution #24).

- To enhance the SMF to be aware of the energy status of NG-RAN and allow SMF to update the QoS configuration based on the PCC rules from PCF which has interacted with EIF (Solution #4).

It is proposed to agree to reuse the 5G policy principle for energy saving behaviors as the interim agreement, that is PCF is the decision maker for energy saving behavior as in the first point above. The suggesting SMF to update Alternative QoS profile as in the fourth point is not preferable. Taking AF's input regarding its expectation for network behavior is also reasonable considering energy saving might degrade network's performance and AF might have some compensation to maintain UE's experience level. Thus the negotiation and notification between PCF and AF is needed.

The new energy attribute and granularity for energy policy decision and charging might have dependency on KI#1.

Proposed Changes

* * * First Change * * * *

7	Interim agreements

7.1	Agreed Principles

7.1.2	Agreed Principles for KI#2

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.Y gets resolved and a principle is agreed.

The following principles are agreed for KI#2:

- To reuse existing 5G AM, SM, UE policy and BDT procedures for energy saving, where PCF is enhanced to retrieves UE's energy energy consumption (EnC) information from EIF and 5GC NF's from OAM.

- To allow AF takes part in the energy saving decision/negotiation, i.e., the AF may indicate to the network the preferred or expected network energy saving behaviors for its applications or its UEs and request the notification on policy changes due to energy saving action.

* * * Second Change * * * *

7.2.2	Topics for further consideration for KI#2

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#2. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in a clause 7.1.2) and can either be then marked as not pursued or postponed to a future Release.

The following topics/principles are for further consideration:

-	Whether the support of TEI20_NITZ can be reused to display readable information related to network's action in AM policy for energy saving to user could be further discussed.