3GPP SA WG2 Meeting #171	 S2-2508785

# Wuhan, China, 13-17 October 2025							                 					(was S2-2507539)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1662 | rev | 2 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to MTSI UE originated DC interworking |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Following issues regarding clause AC.7.9.4 have been identified and discussed is SA2 #169 and SA2 #170: - In Rel-19, the IMS AS is not able to report non-IMS DC related events, which means step 3 can’t be performed. - Steps 3a-3d and steps 5a-5d are overlaped. - It is DCSF instead of DCSF which determins the requirement of DC interworking. - The Nimsas_ImsEE_Notify event used in step3a is not align with the list of events for monitoring capability as specified in clause AD.2.5.2. The CR against clause AC.7.9.4 was postponed in SA2 #170 meeting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The procedures specified in clause AC.7.9.4. are corrected with following changes. - The “Application Data Channel requires interworking is established” event is detected when the application DC is initiated. - The application DC is initiated either by the DCMTSI UE or the DC AS after the IMS session with only audio/video is established. - The DCSF instructs the IMS AS to trigger the DC interworking toward the DC AS and DCMTSI UE |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Application Data Channel Interworking via DC AS for originating MTSI UE is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.7.9.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES <<<<

### AC.7.9.4	Signalling Procedure of Application Data Channel Interworking via DC AS for originating MTSI UE

Figure AC.7.9.4-1 depicts a signalling flow diagram for establishing an application data channel between DC application server and a DCMTSI UE, providing interworking of application data channel to MTSI UE via DC application server. The involved network entities (e.g., IMS AS, DCSF, NEF) are part of the terminating network.

The application data channel is setup between the DC AS and the DCMTSI UE (terminating side). The the terminating DCMTSI UE sends the data channel application media to the DC AS, which performs the necessary interworking actions on the application media for the originating MTSI UE.

Figure AC.7.9.4-1: Application Data Channel set up signalling procedure for IMS data channel interworking with originating MTSI UE via DC application server

The steps in the call flow are as follows:

1.	DC AS subscribes via the NEF to be notified when any application DC for a DC application that requires interworking is established.

1a.	The DC AS subscribes to the NEF by using Nnef_ImsEE_Subscribe Request (event ID: DC Interworking Required, DC Interworking Information). The DC Interworking Information includes UE#2 side Information which is configured as an interworking provider e.g. the service number of the customer service centre.

1b.	After performing the necessary authorization, the  NEF responds to DC AS via Nnef_ImsEE_Subscribe Response.

1c.	The NEF locates the IMS AS(s) via NRF.

1d.	The NEF subscribes to all the IMS ASs as requested by the DC AS in step 1a, via Nimsas_ImsEE_Subscribe Request (Event Filter including the application ID(s).

1e.	The IMS AS responds to the NEF via Nimsas_ImsEE_Subscribe Reponse.

2.	UE#1 initiates an IMS session with audio/video media to UE#2.

3.	If the user of UE#2 intents to trigger a data channel application over the established IMS session, the application data channel between UE#2 and terminating network is requested either from UE#2 (in step 3a) or the DC AS (in step 3b).

3a.	UE#2 sends a re-INVITE to initiate the application data channel.

3b. The DC AS requests to update the existing IMS Session to add application data channel, as specified in clause AG.2.1.1.

4.	The IMS AS validates the user subscription data and determines to notify this request to the DCSF.

5.	After receiving the session event notification from the IMS AS, the DCSF determines that IMS DC interworking for the requested DC application is to be performed based on the application id, and the remote network or UE#1 not supporting IMS DC, or not willing to use IMS DC. If P2P application is requested, the DCSF changes P2P application DC to P2A application DC.

6.	The DCSF instructs the IMS AS that DC interworking is required to trigger the IMS AS notification to the DC AS.

7a.	The IMS AS notifies to the NEF that the DC Interworking Required event is detected with UE#1 side information by using Nimsas_ImsEE_Notify Request. UE#1 information may include the SIP URI or Tel URI of the UE#1.

7b.	The NEF responses to the IMS AS by using Nimsas_ImsEE_Notify Response.

7c.	The NEF notifies to the DC AS that the DC Interworking Required event is detected with UE#1 side information by using Nnef_ImsEE_Notify Request.

7d.	The DC AS responses to the NEF by using Nnef_ImsEE_Notify Response.

8.	P2A Application Data Channel is set up between the serving network and UE#2 based on the procedure described in the clause AC.7.2.2. During P2A connection establishment, the DC AS takes into account the information received in step 7c.

9.	UE#2 uses the established application data channel to provide DC application specific information (e.g. customer service menu) to the DC AS, which will be sent to UE#1 from DC AS.

10.	DC AS performs interworking actions based on the notification received in step 7c and the data received in step 9.

NOTE:	How the DC AS performs interworking actions to provide DC application specific information to the MTSI UE is out of scope of this specification. The DC AS can for example generate a hyperlink associated with the file received in step 8 and sends the hyperlink to UE#1 through a SMS, where the user of UE#1 can download the file using the hyperlink if he/she wants.

# >>>>END OF CHANGES<<<<