SA WG2 Meeting #S2-171	S2-2508799

13 - 17 October, 2025, Wuhan, China	(revision of S2-2507790)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6399 | rev | 4 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the SMF selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, InterDigital Inc., Qualcomm Incorporated, Ericsson, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UIA_ARC |  |  |  |  |  | Date: |  |  | 2025-09-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Providing QoS differentiation for Non-3GPP devices via PDU Session Modification Request has been added to the specification. However, SMF discovery and selection are still in discussion. It proposes to reuse the existing SMF selection mechanism. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clatify the DNN/S-NSSAI is used for SMF selection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SMF discovery and selection are unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.52.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 5.52.3	Session management enhancement

For the traffic of non-3GPP devices requiring differentiated QoS, the Non-3GPP Device Connection Information may be signalled by the UE as defined in TS 24.501 [47]. When a non-3GPP device is connected to the UE, the UE may include the Non-3GPP Device Connection Information in PDU Session Modification Request to the SMF. The Non-3GPP Device Connection Information may include information of more than one non-3GPP device. The SMF forwards the Non-3GPP Device Connection Information to the PCF for policy control.

NOTE 1:	To support QoS differentiation of traffic for Non-3GPP Device Identifier, how the UE determines which PDU Session to use for the non-3GPP device's traffic is not specified; the existing mechanisms (e.g. URSP) can be used to determine which PDU Session to use. It is up to UE implementation to determine when to initiate the PDU Session Modification procedure for updating the Non-3GPP Device Connection Information.

NOTE x:	Subject to operator’s policy, an SMF that supports the QoS differentiation of traffic for Non-3GPP Device Identifier feature is selected according to clause 4.3.2.2.3 of TS 23.502 [3] based on the selected (DNN, S-NSSAI).

For an Ethernet PDU Session, the Non-3GPP Device Connection Information includes the following information for each non-3GPP device:

-	Non-3GPP Device Identifier;

-	MAC address of the non-3GPP device used in PDU Session;

-	Optionally, VLAN tag ID that is associated with the non-3GPP device used in PDU Session.

If the VLAN tag ID is present in the Non-3GPP Device Connection Information and if the received VLAN Tag ID from the UE is not within the allowed VLAN tags for the UE (as described in clause 5.6.10.2), the SMF rejects the PDU Session Modification Request.

For an IPv4 or IPv4v6 PDU Session, the Non-3GPP Device Connection Information includes the following information for each non-3GPP device:

-	Non-3GPP Device Identifier;

-	IPv4 Address associated with the non-3GPP device used in PDU Session;

-	Optionally, port range(s) associated with the non-3GPP device used in PDU Session.

NOTE 2:	IPv4 Address refers to the IP Address of the PDU Session or an IP Address in the range of a Framed Route that is associated with the PDU Session.

For an IPv6 or IPv4v6 PDU Session, the Non-3GPP Device Connection Information includes the following information for each non-3GPP device:

-	Non-3GPP Device Identifier;

-	IPv6 Address/prefix(sub) associated with the non-3GPP device used in PDU Session;

-	Optionally, port range(s) associated with the non-3GPP device used in PDU Session.

NOTE 3:	If the PDU Session type is IPv4v6, it is up to UE implementation to determine to use IPv4 or IPv6 or both Address/prefix(sub) based on the associated traffic of the non-3GPP device and the IPv4 address and an IPv6 prefix that are allocated by the network to the PDU Session.

If the PCF indicates to the SMF, by rejecting the SM Policy Association Modification, that any of the corresponding Non-3GPP Device Identifier(s) in the Non-3GPP Device Connection Information are not available in the UDR for the UE as specified in clause 6.1.3.31 of TS 23.503 [45], the SMF rejects the PDU Session Modification with a cause code to notify the UE that the Non-3GPP Device Identifier(s) are not available for the UE.

NOTE 4:	Since the inclusion of Non-3GPP Device Identifier(s) not available in the UDR for the UE leads to rejection of PDU Session Modification Request, it is recommended that UE does not include parameters that are not related to Non-3GPP Device Connection Information in a PDU Session Modification Request that request differentiated QoS for a non-3GPP device.

END OF CHANGES