SA WG2 Meeting #171	S2-2508850

13 - 17 October, 2025, Wuhan, China

# Title:	[Draft] LS on FS_Sensing_ARC-related aspects requiring coordination

# Response to:	-

# Release:	Rel-20

# Work Item:	FS_Sensing_ARC

Source:	Ericsson [to be SA2]

To:	RAN3

CC:

Contact person:	Aleksejs Udalcovs

aleksejs.udalcovs@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	NA

# 1	Overall description

SA2 has started consolidating the solutions for FS_Sensing_ARC study (see clause 7 in TR 23.700-14).

To further progress the work and reach conclusions, SA2 would like to request feedback on the following aspects identified so far.

- KI#3 on NG-RAN discovery and selection: In order to select an NG-RAN node for a sensing service, SA2 is considering to use the information defining the area where NG-RAN node can perform sensing measurements. This area is denoted as “NG-RAN Serving Sensing Area”, and the core network obtains this information either via OAM (configuration) or this area is provided by NG-RAN during establishment of association between NG-RAN and the core network. Is there any other information about NG-RAN nodes that the core network should consider when selecting NG-RAN nodes for the requested sensing service?

- KI#4 on sensing result generation: Some SA2 solutions consider a possibility of NG-RAN node performing the determination of sensing result that will be exposed by the core network to the sensing service consumer such as a third-party AF. SA2 like to understand whether RAN3 is considering such functionality as part of their study, i.e., whether NG-RAN node, when selected for the sensing service by the core network, may be able to determine a final sensing result? Determination of sensing result includes processing by an NG-RAN node required, for example, to detect an object or obtain its characteristics (e.g. Position and 3D velocity) when the object is present in an area where an NG-RAN node can perform sensing measurements.

- KI#5 on sensing result exposure: In order to specify a geographic area for which a sensing service consumer request a sensing service, SA2 considers definition of the Target Sensing Area concept as described in Solution#28 (see S2-2508853). This Target Sensing Area will be used to select NG-RAN node and sent in the sensing service request message. SA2 assumption is that the provided Target Sensing Area will be used by the NG-RAN to determine an area in which to perform sensing measurements. Could RAN3 provide the feedback on the SA2 assumption?

- KI#6 on Configuration of parameters for NG-RAN nodes: While some SA2 solutions presume that NG-RAN nodes need to be provisioned with configuration parameters for sensing measurements, the other solutions do not see the need for such provisioning and take the position that NG-RAN node can derive the radio configuration parameters for sensing measurements based on the sensing service requirements (e.g., Target Sensing Area, indication about sensing result reporting (one-time, event-triggered or periodic), time and spatial resolution of the sensing result) that the core network sends to the selected NG-RAN node inside a sensing service request message. Does any information need to be provided from the core network to the NG-RAN node prior to a sensing service request to assist with NG-RAN configuration?

[Other questions, if any, can be further added during the meeting]

# 2	Actions

To TSG RAN3

ACTION: 	SA2 kindly asks RAN3 to provide feedback on the above questions.

# 3	Dates of next SA WG2 meetings

SA2#172	November 17 – 21, 2025	Dallas, US

SA2#173	February 9 – 13, 2026	TBD, India