3GPP TSG-WG2 Meeting #171	S2-2508857

October 13 – 17, 2025, Wuhan, China		(revision of S2-250xxxx)

Source:	Ericsson

Title:	Interim agreement for KI#4

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This contribution provides agreed principles and topics for further consideration for KI#4

# 1. Discussion

Solition#1,3,5,18,21-26,34,39 are proposed for KI#4. The table below summarizes how the 3 aspects of KI#4 are addressed by those solutions.

| # | How to collect sensing data (and the associated information, if any) for Sensing result generation? | Which Sensing Function performs Sensing result generation? | Whether and what the above associated information is required for Sensing result generation? | Discussion |
| --- | --- | --- | --- | --- |
| 1 | The SF sends the sensing request to SE(s) and collects the sensing data/result from SE(s). The SE sends the sensing data/result through a direct connection to the SF. | SF | Not specified. |  |
| 3 | The SF sends the sensing request to SE(s) and collects the sensing data from SE(s). The SE sends the sensing data by establishment of a path for 3GPP sensing data reporting. | SF | Not specified. |  |
| 5 | The SF sends the sensing subscribe request to SE(s) through AMF and collects the sensing data from SE(s). The SE sends the sensing data to SF through AMF. | SF | Not specified. | AMF and NAS protocol are for UE communication service and shall not be used for sensing. SEs connect to SF through AMF is not recommended. |
| 18 | SF establishes a data connection with SE for sensing data and the associated information transport. How does the SF send a sensing request to the SE is not specified. | Not specified | Not specified | Even associated information is mentioned, there have no details. |
| 21 | The Sensing Function (SF) and NG-RAN acting as sensing entity supports direct interface to transfer sensing message. The SE sends the sensing report to the SF. | SF | Not specified |  |
| 22 | 1) A direct data tunnel is established between the Sensing Entity (i.e. gNB) and the Sensing Function via the sensing control signalling interaction. 2) The Sensing Function and the Sensing Entity (i.e. gNB) exchange their tunnel information (e.g. TEID (Tunnel Endpoint ID) and/or IP address) for data tunnel establishment. 3) The sensing data and the associated information (e.g. Sensing Service ID) are transmitted from the Sensing Entity to the Sensing Function via the established data tunnel. | SF | Sensing Service ID |  |
| 23 | The SeMF sends configuration parameters of a sensing service request to the SE. The SeMF selects a SePF and sends a processing configuration request to the SePF. The SeMF requests sensing data from the SE. The SeMF may involve a chain of SeMFs to process the sensing data. The SeMF returns the sensing result to the sensing service consumers. | SeMF | Sensing Service Request(SSR) identifier |  |
| 24 | The Sensing Function (SSMF) discovers and selects the SDCF (Sensing Data Control Function) based on the Sensing Service request from the consumer based on criteria such as location/coverage area. The SSMF sends the Sensing Configuration Request to the selected Sensing Entities. The request includes the SDCF endpoint information as selected by the SSMF to establish a data connection to coordinate the sending sensing data collection and processing from the Sensing Entities. | SSMF aggregates the sensing reports from SDCF and generates the sensing result. | Not specified. | It is recommended SDCF shall connect to SE directly instead of through AMF. |
| 25 | The SF is split into SCF and SPF. The SCF select an SPF to establish a direct gNB-SPF connection for the collection and transmission of sensing data and related information. |  | Not specified. |  |
| 26 | The SF is split into SCF and SPF. - A new Network Function, i.e. Sensing Data Management Function (SDMF) is introduced for the storage and management of the Sensing result. - When the Sensing Entity is a gNB, the SCF sends the sensing service request and the configuration information to the gNB via a new interface defined between the SCF and the gNB. - If the sensing result is not available in the SDMF, the SCF needs to send the Sensing Initiated Request and the corresponding parameters to the SPF. - When the gNB finishes the sensing data measurement, the gNB reports the sensing data to the SPF directly via the established data tunnel. - The Sensing Entities can be multiple gNBs, and the SPF performs the aggregation of the sensing data collected from multiple sources. - The SPF generates the Sensing result based on the sensing data received from the gNB/gNBs, and sends the Sensing result to the SCF and the SCF can further send the Sensing result to the SDMF for storage. | SPF |  | The needs of SDMF should be use case driven and be FFS. The storage of sensing result can also be implemented inside SF. |
| 34 | - The Sensing Function sends the sensing request to sensing entity (i.e. RAN) with the session configuration, sensing data transport information. - The configuration parameters include two part: from AF, locally created by SF. - The Sensing entity establish the connection with Sensing Function. - The Sensing entity sends the sensing data to Sensing function through the direct connection. The Sensing Function generates the sensing result. Note: The SF may be split into SCF and SPF. | SF(or SPF in case of SF is split into SPF and SCF) |  |  |
| 39 | SF chooses AMF(s) that serve a geographical area corresponding to the target sensing area. The sensing capability of these gNBs connected to these AMFs are then shared with the SF, which chooses candidate Sensing Entity (SEs) and configures the selected SEs for sensing. | Not applicable | Not applicable | The solution addresses the parameter configuration of SE from SF through AMF instead of addressing KI#. |

# 2. Text Proposal

It is proposed to agree the following principles to 3GPP TR 23.700-14.

>>>> Start of Changes <<<<

### 7.1.4	Agreed Principles for KI#4 on Sensing Data and the Associated Information Collection and Transport

In order for a 3GPP network to collect sensing data (and the associated information) and generate a sensing result, the following main principles apply:

-	Based on the sensing service request received from the Sensing Core Function (SnCF), the NG-RAN reports the sensing data/result to SnCF through a direct P2P interface between the SnCF and the NG-RAN node based on the established association.

Editor’s Note: Whether the NG-RAN sends sensing data or sensing result to SnCF needs to be coordinated with RAN WGs.

Editor’s Note: How direct P2P Interface between NG-RAN and SnCF can be established needs further coordination with RAN WG3.

>>>> End of Changes <<<<