3GPP TSG-WG2 Meeting #171	S2-2508858

October 13 – 17, 2025, Wuhan, China		(revision of S2-250xxxx)

Source:	Ericsson

Title:	Interim agreement for KI#5

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This contribution provides the agreed principles and topics for further consideration for KI#5.

# 1. Discussion

This document proposes interim agreement for KI#5.

# 2. Text Proposal

It is proposed to agree the following principles to 3GPP TR 23.700-14.

>>>> Start of Changes <<<<

### 7.1.5	Agreed Principles for KI#5 on Sensing Result Exposure

In order for a 3GPP network to expose a sensing result to the sensing service consumer based on the authorized sensing service request, the following main principle apply:

NOTE X1:	It is assumed that a sensing service requester and a sensing service consumer is the same entity.

-	A sensing result is exposed by the network to the authorized sensing service consumer in relationship with the invoked sensing service request.

-	Inside the request, the sensing service consumer shall include AF Identifier, information about the Target Sensing Area, type of sensing to perform (e.g. ‘Object Detection’, ‘Object Tracking’) and Sensing Result Detail Level.

-	‘Sensing Result Detail Level’ attribute is used by the sensing service consumer to request a specific form of the sensing result. Its value can be set, for example, to ‘Notify result only’, in which case only the Yes/No indication with no associated information is provided to the sensing service consumer, or to ‘Notify result with associated information’, in which case the sensing service consumer is provided with the sensing result and the associated information, such as time of capture, area map, etc.

-	Additionally, the sensing service consumer may request a specific event reporting mode (e.g. one-time, event-triggered or periodic reporting of the sensing result) as well as request, for instance, a specific time and/or spatial resolution of the determined sensing result or a specific time of day when the sensing result can be exposed.

-	After the sensing service request authorization and the selection of an SnCF – Sensing Core Function (see KI#2 and KI#3, respectively), the SnCF determines an NG-RAN node and invokes the corresponding sensing service request to the selected NG-RAN node. Inside the request, the SnCF includes Target Sensing Area information, type of sensing to perform, relevant parameters from the sensing service consumer, and other parameters derived by the SnCF for the authorized sensing service.

-	Based on the received sensing service request from the SnCF, it is expected that the NG-RAN collects sensing measurements and apply a certain level of processing prior to providing the sensing data to the SnCF, including the determination of a sensing result. The exact details are under RAN WGs remit.

Editor’s Note: 	Whether NG-RAN can determine and provide a final sensing result to the SnCF needs to be coordinated with RAN WG.

NOTE X2: 	The form of sensing data provided by NG-RAN depends on level processing the NG-RAN applies to the sensing measurements and, thus, can include the determined result or pre-processed sensing data for subsequent processing at the SnCF for instance.

-	The SnCF may further use the NG-RAN provided sensing data to determine a sensing result.

-	After the sensing result has been determined, the SnCF provide the sensing result to the sensing service consumer via the NEF using the Nnef_SensingEvent_Notify and Nsncf_SensingEvent_Notify service operations.

NOTE X3:	The exact names of service operations can be further revised.

>>>> Next Change <<<<

### 7.2.5	Topics for further consideration for KI#5: Sensing Result Exposure

Sensing result determination by NG-RAN nodes: Whether NG-RAN nodes can perform a level of processing required to determine a sensing result (e.g. to detect an object) is RAN WGs decision and requires further coordination.

>>>> End of Changes <<<<