SA WG2 Meeting #171	S2-2508865

13 – 17 October, 2025, Wuhan, China

Source:	Ericsson, Nokia, AT&T [,Xiaomi, OPPO, InterDigital, Google, Charter Communications]

Title:	[WT#1.2, QoS] WT & KI for QoS Framework

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution provides content on WT and KI for WT#1.2 QoS Framework.

**** First Change ****

# Annex A.X WT#1.2.X: Evolution of QoS Framework for 6G

Editor’s Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

- Investigate if any new functionality in 6G QoS framework considering, e.g., the emerging new traffic characteristics and application needs is required.

NOTE 1: To enable clear & measurable connectivity service descriptions for SLAs, the definition of 6GS QoS concepts will clearly separate into the 6G System provider perspective and into the 6G System usage perspective respectively  (clear responsibility boundaries).

- Whether and how to enhance QoS control mechanisms to

  - better support QoS targets than by the best effort approach of existing non-GBR and less resource intensive than by the existing GBR, Delay Critical-GBR approach to improve capacity utilization in the network.

  - to improve QoS for applications with dynamic requirements.

- Whether and how to monitor QoS and how to expose the monitoring result.

- Study whether and how to support the UE, application and network QoS collaboration to improve awareness in the 6GS of connectivity performance needs and awareness in the application of what can be provided by the network.

  - UE provided information to the network to assist the network on deriving the QoS treatment.

  - What information shall be exchanged between the application and the network to better support QoS [see WT#1.2 User Plane].

NOTE 2: Applications typically consist of an Application Client in the UE and application server in the DN.

- Study aspects of interworking with 5GS.

NOTE 3: Interworking aspects will be coordinated with WT#2.

NOTE 4: Aspects of traffic detection are assumed to be handled by the WT#1.2 User Plane. Coordination is needed in relation to possible impacts of traffic detection on the QoS framework.

NOTE 5: This WT will coordinate with SA WG4 and RAN WGs.

NOTE 6: The content of the 5GS QoS Framework that will be used as a starting point for discussion on 6G QoS will be determined by moderated discussion process.

NOTE 6: This WT will coordinate with SA WG4 and RAN WGs.

**** Next Change (all new text) ****

## 5.X.1	Key Issue #X1: QoS Framework for 6GS

To support QoS in 6GS following aspects will be studied.

- To support traffic characteristics and application needs study:

  - Whether and what information needs to be specified to describe the required packet treatment targets.

  - Whether and what functionality is needed to be specified, e.g., to enable the required packet treatment targets.

- Study how to better support packet treatment targets (QoS targets) than by ‘best effort’ approach in a manner that is more resource efficient compared to legacy ‘GBR’ approach.

- To support applications with dynamic connectivity performance needs, study whether dynamicity is required and if needed what:

  - type of dynamicity is required to be supported.

  - QoS control mechanisms are needed.

- To improve awareness in the 6GS of connectivity performance needs and awareness in the application of what can be provided by the network, study:

  - whether, how and what UE information is provided to the network to assist the network on deriving the QoS treatment.

  - Whether and what information shall be exchanged between the application and the network to better support QoS [see WT#1.2 User Plane].

NOTE 1: Applications typically consist of an Application Client in the UE and application server in the DN.

## 5.X.2	Key Issue #X2: QoS Monitoring for 6GS

The visibility of system behaviour with regards to achieving the packet treatment targets may require monitoring and exposure of the relevant results.

Following aspects will be studied:

- how to support monitoring of QoS, leveraging existing functionalities if appropriate and/or introducing new.

## 5.X.3	Key Issue #X3: Interworking aspects

Study aspects of interworking with 5GS.

NOTE 1: Interworking aspects will be coordinated with WT#2

**** End of Changes ****