SA WG2 Meeting SA2 #171	S2-2508879

13 - 17 October, 2025, Wuhan, China

Source:	China Mobile

Title:	Interim Agreement for KI#1: AIOTF Discovery and Selection

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release: FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract of the contribution: This contribution proposes the interim agreements for FS_AmbientIoT_Ph2_ARC / Rel-20.

- Discussion

In the normal UE reader registration procedure, the AMF is in charge of the UE reader’s registration procedure; the AIOTF is not involved in the UE reader’s registration procedure. After the UE reader has successfully registered with UDM, UDM will have the dynamic serving AMF information, but have no serving AIOTF information.

So, in the T2, if the AF provides the UE Reader’s ID in the AIoT service request, how does the NEF select the serving AIOTF based on the UE reader’s ID?

Because of the UE reader’s mobility, a static local configuration mapping between the UE reader’s ID and the Serving AIOTF in the NEF is not a practical solution.

Suggest the following solutions:

Option 1:

The NEF queries UDM to obtain the serving AMF based on the UE reader’s ID (e.g., GPSI)

The NEF obtains the serving AIOTF based on the serving AMF through local mapping configuration between the AIOTF and the AMF, or the NEF obtains the serving AIOTF by querying NRF based on the serving AMF.

Option 2:

Configuring serving AIOTF information in the AMF.

Update the UE reader’s registration procedure; AMF will update both the serving AMF and serving AIOTF information in the UDM.

NEF queries UDM to obtain the serving AIOTF based on the UE reader’s ID (e.g., GPSI)

In comparison to Option 2, Option 1 does not require enhancements to the registration process. It is recommended that NEF adopt Option 1 for the selection of AIOTF if the AF provides the UE reader ID in the AIoT service request without external target area information.

- Proposal

It is proposed to adopt the following changes to TR 23.700-30.

* * * * First Change * * * *

7	Interim agreements

7.1	Agreed Principles

7.1.1	Agreed Principles for KI#1

The agreed principles of AIOTF discovery and selection are as follows:

-	The AF may provide UE reader ID or external target area information in the AIoT service request.

-	If the AF provides external target area information, the NEF chooses the AIOTF(s) as described in clause 5.3.1 AIOTF Discovery and Selection in 23.369 [x].

-	If the AF provides the UE reader ID, the NEF retrieves the serving AMF instance from the UDM using UE reader ID information and further determines the AIOTF instance either by querying the NRF or by using local configuration based on the retrieved serving AMF instance.

* * * * End of the Change * * * *