SA WG2 Meeting #171	S2-2508884

13 – 17 October 2025, Wuhan, China

Source:	Ericsson

Title:	KI#3: New Solution for DC3/DC4 enhancements with new media management

Document for:	Approval

Agenda Item:	20.7.1

Work Item / Release:	FS_NG_RTC_Ph3_ARC / Rel-20

Abstract of the contribution: This contribution proposes a new solution for FS_NG_RTC_Ph3. The solution addresses WT#2.2, based on Nokia proposal from last meeting "Solution for DC3/DC4 enhancements", updated.

# 1. Discussion

This pCR proposes a solution to the Key Issue #3: Supporting dynamic configuration of MDC2 endpoints (“investigate whether and how to define interfaces between DCSF and NEF/AF to complete existing P2A/P2A2P Data Channel procedures by specifying DC3/DC4 interfaces to enable dynamic configuration of MDC2 endpoints”).

# 2. Text Proposal

It is proposed to capture the following changes into TR 23.700-56.

# * * * * all new change * * * *

# 6	Solutions

## 6.X	Solution #X: DC3/DC4 enhancements with new media management

### 6.X.1	Description

This solution addresses KI#3.

This solution specifies the DCAS services exposed over the DC3/DC4 interface to support UE-initiated P2A and P2A2P ADC establishment. Specifically, the following new service function is defined: Ndcas_Media_Management. the associated service parameters and corresponding procedures are also defined.

The DC AS provisions the service triggering (triggering information) criteria for establishing P2A/P2A2P ADC at the DCSF using the Nimsas_ImsParameterProvision_create service operation (see TS 23.228 V19.0.0 Appendix AA2.4.6). Such provisioning can be explicitly initiated by the DC AS or implicitly configured per DC AS in the IMS AS and/or in the service document in HSS (Repository data). Triggering can be user specific or user unrelated (i.e., PSI) and associated with filter conditions such as P2A/P2A2P ADC associated with specific applications. Upon receipt of a P2A or P2A2P ADC establishment request meeting the triggering filter conditions, the IMS AS invokes Nimsas_SessionEventControl_Notify to notify the DCSF of a media change request event and the availability of an P2A/P2A2P ADC. This notification includes the SDP information for the P2A/P2A2P ADC representation on the Mb interface. Based on the notification received, the DCSF will request anchoring of media in the MF for the originating MF-side of the MDC2 interface. The DCSF will then provide the originating MF-side of the MDC2 to the DCAS and request the DCAS-side for the MDC2 via the Ndcas_Media_Management_Create service operation. The DCSF will provide the received media information from the DCAS to IMS AS over the Nimsas_mediaControl service with the requested MDC2 configuration based on the information received from the DC AS.

This enhancement to the DC3/DC4 interface is crucial for ensuring a time sensitive and 3GPP SBA principle compliant consistent handling of MDC2 media resources between the DCSF and the DC AS. The DC AS and the MF may dynamically deploy/insert MDC2 terminations and thus use the dynamically determined IP address. The DC port and/or DC SCTP port may be dynamically associated to the session by the MF and/or the DC AS. Furthermore, the network (DCSF) may need to provide the DC AS with the UE certificate fingerprint received from the UE over IMS signaling using the Dcas_Media_Management_Update service operation.

The current P2A/P2A2P data channel procedures lack a 3GPP SBA service self-containment complaint mechanism for MDC2 media resource negotiation, leading to potential inconsistencies of MDC2 media resource between the two MDC2 endpoints. This proposed enhancement, by enabling the MDC2 media negotiation and dynamic configuration, significantly improves the robustness and flexibility of these procedures, preventing failures due to outdated or missing MDC2 media resource information. The DCSF availing of the DC AS offered / provided service, Dcas_Media_Management ensures that it is the resource control entity when interworking with DC AS.

Furthermore, this solution ensures that DC AS can be selected by DCSF (for establishing P2A/P2A2P ADC) on a per session basis without creating long lived dependencies between the entities.

### 6.X.2	Service definition

#### 6.X.2.1	General

The following table shows the DC AS Services and DC AS Service Operations.

Table 6.X.2-1: NF services provided by the DC AS

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Ndcas_Media_Management | Create | Request/Response | DCSF |
|  | Update | Request/Response | DCSF |
|  | Delete | Request/Response | DCSF |

#### 6.X.2.2	Ndcas_Media_Management

##### 6.X.2.2.1	General

Service description: This service enables the consumer (DCSF) to create, update or terminate a media resource controlled by the producer (DCAS).

##### 6.X.2.2.2	Ndcas_Media_Management_Create

Service operation name: Ndcas_Media_Management_Create

Description: This service operation enables the consumer to create a media resource (e.g., ADC (P2A, P2A2P) or other media resources like audio/Video. The information is used by the DCAS to create a resource and establish a media resource in an IMS session.

Inputs, Required: Session ID, MF-side MDC2 endpoint information (e.g. IP address, port number), certificate fingerprint of DC AS for end-to-end security between UE and DC AS in case MDC2 is in "UDP/DTLS/SCTP", "TCP/TLS" or "SCTP/DTLS" mode, trigger Information.

Inputs, Optional:

Outputs, Required: Result. The result contains the following data elements: session id, DCAS-side MDC2 media resource information, destination URL used by DCSF to send subsequent operations.

Outputs, Optional: None.

##### 6.X.2.2.3	Ndcas_Media_Management _Update

Service operation name: Ndcas_Media_Management _Update

Description: This service operation enables the consumer to update a media resource information used by the DCAS.

Inputs, Required: Session ID, MF-side MDC2 endpoint information (e.g. IP address, port number), destination URL

Inputs, Optional: certificate, fingerprint of the UE, trigger Information

Outputs, Required: Result.

Outputs, Optional: None.

##### 6.X.2.2.4	Ndcas_Media_Management _Delete

Service operation name: Ndcas_Media_Management _Delete

Description: This service operation enables the consumer to provide information used by the DCAS for media termination in an IMS session

Inputs, Required: Session ID, mediaInformation, destination URL

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

#### 6.X.2.3	Nimsas_ImsParameterProvision Service API

##### 6.X.2.3.1 	Type: ImsPpDataEntry

Table 6.5.6.2.2-1: Definition of type ImsPpDataEntry, TS 29.175

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| rcdServerInformation | RcdServerInfo | O | 0..1 | The RCD server information provisioned to the IMS AS. |
| triggerinformation | TriggerInfo | O | 0..1 | Triggering information to invoke for example dcas media management |

Editor’s note:	The TriggerInfo data type needs to be defined as common data and is for FFS.

#### 6.X.2.4	Nimsas_SessionEventControl Service API

##### 6.X.2.4.1	Type: SessionEventNotification

The parameters that are new are: afInstance, triggerinformation

Table 6.1.6.2.2-1: Definition of type SessionEventNotification TS 29.175

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| notificationEvent | NotificationEvent | M | 1 | The NotificationEvent corresponds to a report on one subscribed event. |
| sessionId | SessionId | M | 1 | The session ID is the identity of the IMS session for which the event relates to. This IE contains the information in the Call-ID header which is the typical header of SIP message. |
| sessionInfo | SessionInfo | C | 0..1 | The sessionInfo is the IMS session related information, e.g. the calling and called identity. It shall be contained when the attribute eventType of the notificationEvent is one of the "SESSION_ESTABLISHMENT_REQUEST", "MEDIA_CHANGE_REQUEST", "EXTERNAL_SESSION_CREATE", "SESSION_ESTABLISHMENT_SUCCESS" (NOTE). |
| mediaInfoList | map(MediaInfo) | C | 1..N | Media info list includes a list of media related information, e.g. media identity, media type and media specifications. The mediaId attribute within the MediaInfo data type is the key of the map. It shall be contained when the attribute eventType of the notificationEvent is one of the "SESSION_ESTABLISHMENT_REQUEST", "SESSION_ESTABLISHMENT_PROGRESS", "SESSION_ESTABLISHMENT_ALERTING", "SESSION_ESTABLISHMENT_SUCCESS", "MEDIA_CHANGE_REQUEST", "MEDIA_CHANGE_SUCCESS", "MEDIA_CHANGE_FAILURE", or "MEDIA_CHANGE_CANCEL". |
| psDataOff | PsDataOffStatus | C | 0..1 | It indicates the active/inactive status of PS Data Off for the served UE. It shall be present if the attribute eventType of the notificationEvent is set to one of the following: - "SESSION_ESTABLISHMENT_REQUEST", when IMS session establishment is invoked; - "PS_DATA_OFF_STATUS_CHANGE", when the status change of PS Data Off happens without any other changes to the IMS session; It may be present in other cases if the change of PS Data Off status happens together with other events (e.g. media changes). |
| mediaInfoSet | array(MediaInfoExternal) | C | 0..1 | Media information set includes a set of media information. It shall be contained when the attribute eventType of the notificationEvent is one of the "EXTERNAL_SESSION_UPDATE" "EXTERNAL_SESSION_CREATE" |
| afInstance | string | O | 0..1 | The string identifying the served DCAS service instance |
| triggerinformation | TriggerInfo | O | 0..1 | Triggering information to invoke dcas media management |
| NOTE: The sessionInfo shall be present when the attribute eventType of the notificationEvent is set to "SESSION_ESTABLISHMENT_SUCCESS" and the originating IMS AS received the redirected-to party identity via the History-Info header field in the 200 OK responses to the initial INVITE request. |  |  |  |  |

### 6.X.3	Procedure

#### 6.X.3.1	Person-to-Application (P2A) Application Data Channel Setup

Figure 6.X.3.1-1 depicts a signalling flow diagram for establishing an Application Data Channel in a person to application use case.

Figure 6.X.3.1-1: P2A ADC establishment with a trusted DC AS.

1.	The DCAS provisions service triggers using the Nimsas_ImsParameterProvision.

2.	The IMSAS responds the request to the DC AS confirming the provisioned data stored for the DCAS.

3.	IMS session and BDC has been established.

4-11. UE#1 requests the establishment of P2A ADC establishment. IMSAS evaluate the service triggers provided in (1). IMSAS provide the service triggers and the DCAS instance to the DCSF. DCSF determines the DC control policy and request media endpoint for the MDC2 from the IMSAS. IMS AS to request MF to reserving the originating media resources.

12.	DCSF request the DC AS to establish a media endpoint for the MDC2 via Ndcas_Media_Management_Create message. The message includes calling and called parting id, application id/DCAS instance, session id, MF-side MDC2 media resource info. source URL used by DC AS to send subsequent ADC Control message, and the certificate fingerprint of UE.

13.	DC AS allocates DC AS-side MDC2 media resource for this session. DC AS respond to the DCSF with the DC AS endpoint for the MDC2 via Ndcas_Media_Management_Create response. The message contains: result, session id, DCAS-side MDC2 media resource information, destination URL used by DCSF to send subsequent Ndcas_Media_Management messages to update or terminate the connection. Update might contain e.g. the certificate fingerprint of UE.

14-16. DCSF updates the media resources in the MF via IMS AS, based on the received the DC AS-side MDC2 media resources information. Then, the IMS AS sends response to DCSF.

17-29. IMS AS performs SDP negotiation with terminating network. With a successful SDP negotiation, the application data channel established.

#### 6.X.3.2	Person-to-Application and Application-to-Person (P2A2P) Procedure

Figure 6.X.3.2-1 depicts a signalling flow diagram for establishing an Application Data Channel in a person to application use case.

Figure 6.X.3.2-1: P2A2P ADC establishment with a trusted DC AS.

1.	The DCAS provisions service triggers using the Nimsas_ImsParameterProvision.

2.	The IMSAS responds the request to the DC AS confirming the provisioned data stored for the DCAS.

3.	IMS session and BDC has been established.

4-11. UE#1 requests the establishment of P2A ADC establishment. IMSAS evaluate the service triggers provided in (1). IMSAS provide the service triggers and the DCAS instance to the DCSF. DCSF determines the DC control policy and request media endpoint for the MDC2 from the IMSAS. IMS AS to request MF to reserving the originating media resources.

12.	DCSF request the DC AS to establish a media endpoint for the MDC2 via Ndcas_Media_Management_Create message. The message includes calling and called parting id, application id / DC AS ID, session id, MF-side MDC2 media resource info. source URL used by DC AS to send subsequent ADC Control message, and the certificate fingerprint of UE.

13.	DC AS allocates DC AS-side MDC2 media resource for this session. DC AS respond to the DCSF with the DC AS endpoint for the MDC2 via Ndcas_Media_Management_Create response. The message contains: result, session id, DCAS-side MDC2 media resource information, destination URL used by DCSF to send subsequent Ndcas_Media_Management messages to update or terminate the connection. Update might contain e.g. the certificate fingerprint of UE.

14-16. DCSF updates the media resources in the MF via IMS AS, based on the received the DC AS-side MDC2 media resources information. Then, the IMS AS sends response to DCSF.

17-24. IMS AS performs SDP negotiation with terminating network. And the IMS AS notifies the DCSF about the successful result of the SDP negotiation.

25.	DCSF updates the media resources in the MF via IMS AS, based on the received the DC AS-side MDC2 media resources information and the media resource information of the terminating network. Then, the IMS AS sends response to DCSF.

26-33. IMS AS continues performing SDP negotiation with the originating network. With a successful SDP negotiation, the application data channel established.

### 6.X.4	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

DC AS:

-	provisioning service triggers for the UE-initiated P2A/P2A2P ADC establishment,

-	receiving MF-side information for MDC2 configuration,

-	receiving the certificate fingerprint of UE for end-to-end security between UE and DC AS,

-	sending DC AS-side information for MDC2 configuration,

-	sending the certificate fingerprint of DC AS for end-to-end security between UE and DC AS.

DCSF:

-	receiving subscription of the event UE-initiated P2A/P2A2P ADC establishment,

-	notifying about the event of UE-initiated P2A/P2A2P ADC establishment,

-	sending MF-side information for MDC2 configuration,

-	sending the certificate fingerprint of UE for end-to-end security between UE and DC AS,-	sending request to create DC AS-side for the MDC2.

-	receiving DC AS-side information for MDC2 configuration,

-	receiving the certificate fingerprint of DC AS for end-to-end security between UE and DC AS,

-	requesting IMS AS for MDC2 configuration.

IMS AS:

-  receiving provisioning of trigger information from the DCAS,

-	receiving MDC2 configuration request from DCSF,

-	instructing MF for MDC2 configuration.

MF:

-	receiving MDC2 configuration instructions from IMS AS,

-	performing MDC2 configuration.

# * * * * End of changes * * *