3GPP TSG-SA WG2 Meeting #171	S2-2508892

Wuhan, China, October 13 – 17, 2025

Source:	Ericsson

Title:	KI#2 interim agreements

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes interim agreements for KI#2.

# 1. Discussion

For KI#2 “Support of service adjustments for the UE”, 16 solutions in the TR are summarized and discussed in the following table:

| Sol# | Solution Summary | Energy Trigger/Evaluation | Service Adjustment | Charging Impacts | Discussion |
| --- | --- | --- | --- | --- | --- |
| 2 | Policy Control Considering Slice or 5GC Energy Consumption. Controls policies with respect to energy usage of slices or 5G core network. | Slice and 5GC core network energy usage threshold. | AM policy, SM policy adjustment or BDT policy renegotiation, such as lower UE-AMBR/Session-AMBR. | No | It is proposed the Slice or 5GC core network energy usage threshold can be used as triggers for energy control in normative works before the fine-granular energy related information, like per UE EnC, becomes robust. |
| 3 | AF can indicate its level of adaptability in terms of QoS requirements, similar to “alternative QoS profiles/service-requirements” and/or transfer time, a list of alternative time slots in PDTQ negotiation to PCF through NEF. The PCF identifies the energy-optimized candidate time slots whose QoS also falls into the range of AF QoS Adaptability and includes each time slot in each of the list of PDTQ policy for negotiation with the AF. | Prediction of energy-related information such as energy consumption or renewable energy ratio in the node level (i.e. gNB and (I)UPF) from the OAM. Operator policy to decide what is “energy optimized” | PCF and AF negotiate energy optimized PDTQ policy which QoS fulfilment is within the range of AF QoS Adaptability. | No | The QoS Adaptability has same effect as providing alternative service requirements which allow the PCF to select a time window which meets the one of the requirements and is also energy optimized. It is proposed that either AF provides Energy Saving(alternative) Service Requirements or PCF generates according to operator policy (SLA agreement) for PDTQ the same ways as Solution#7 can be considered in normal normative works. |
| 4 | - to enhance the SMF to be aware of the energy status of NG-RAN (e.g. Notification from RAN to SMF to indicate whether the NG-RAN is using renewable energy or SMF gets this from OAM) and then decide whether to update Alternative QoS profile or not to the RAN. - to allow SMF to update the QoS configuration based on the PCC rules from PCF which has interacted with EIF. | - Energy attributes of RANs to SMF. Note: this is only mentioned in the 6.4.5 Impacts on existing services, entities and interfaces. In the description, it also describes RAN sends notification to SMF. - PCF modifies service requirements through SMF by interaction with EIF. The details are not covered in the solution. | No changes to existing QoS model and parameters: - alternative QoS profile to RAN, - updated QoS rules from PCF to SMF. | No | Solution#7 and Solution #4 both propose RAN to apply Energy Saving QoS profile. However, the Solution#7 works closer to the existing alternative QoS profile function which seems better. |
| 5 | PCF makes the policy decisions based on network energy saving behaviours which can be either provided by AF or configured in operator policy, and the energy related information. | AF can subscribe to the Energy Related Information Exposure and decide to send network energy saving behaviours, which includes threshold and granularity, QoS adjustment. PCF based on the energy related information from EIF, network energy saving behaviours and Energy saving indicator subscription data to perform service adjustment. | AM policy update, SM policy update or UE policy update. | No | Generally, AF provided provide energy saving policy can be considered. What exact information can be provided can be for further consideration. |
| 6 | The PCF takes the energy related information in the notifications from the EIF into account for policy control. | Operator policy on Energy Consumption information at UE, S-NSSAI, PDU Session and Service Data Flow (e.g. per UE per application) granularity like threshold is crossed and Energy Saving Indicator Subscription. | AM policy update, SM policy update. | No | PCF applies service adjustment for AM and SM policy can be viewed as a general principle to be used in normative works. The slice level or 5GC energy saving trigger should be used before fine-granular energy related information, like per UE, becomes robust. |
| 7 | PCF provides Energy Saving QoS profiles based on AF provided energy saving service requirement or operation policy to RAN. RAN can apply Energy Saving QoS profile for energy saving and notify selected Energy Saving QoS profile to PCF and to UPF through SMF for charging purpose. | NG-RAN decides what trigger is used. | RAN applies Energy Saving QoS profiles, similar to Alternative QoS profile. | Yes, but it does not describe what is the impact. | This does not work in the same way as the similar function, the alternative QoS flow work. And RAN only enforces GBR QoS flow today. This shall not be promoted. |
| 13 | The PCF adjusts the AM policy for individual UE in a network slice by monitors network slice EnC. | PCF monitors network slice EnC, by aggregating per UE per slice EnC from the EIF of all UEs in the network slice, reaches a threshold. | AM policy update (e.g. network slice replacement) | No. | New mechanism such as PCF gets the slice EnC from OAM or EIF re-exposes slice EnC from OAM is more efficient instead of aggregating the per UE per application EnC from EIF. |
| 14 | The PCF initiates the SM Policy Association Modification to increase the QoS of target UE, PDU session or QoS flow based on EIF renewable energy info and Renewable Energy indicator from subscription data. Finally, the UEs which are served with more renewable energy can enjoy better QoS guarantees in 5GS. | Renewable energy or the ratio of renewable energy from EIF exceed a threshold. | AM policy, SM policy updates such as - Increase flow bit rates or aggregate bit rates of both UL/DL. - ARP with higher priority for UE. - Maximum Packet Loss Rate or Packet Error Rate of target UE. | No | How to provide renewable energy info should be for further consideration. |
| 15 | UE energy saving services that users can subscribe to via the service provider's Application Function (AF) or the operator, based on SLA agreements. Two service levels are proposed: - Level A, a premium option maintaining high performance. - Level B, a cost-effective option that may lower service quality to conserve device energy. The Policy Control Function (PCF) manages energy saving policies for each service, and the AF can activate them according to application-level monitoring. | The AF may monitor the communication with the UE over the application and based on certain criteria like UE battery goes below a threshold. | The AF activates the UE energy saving service by providing UE energy saving service parameter to UDR. PCF receives notification from UDR and activates the UE energy saving policy, Level A or Level B. | No | Generally, that AF provides energy saving related polies can be considered. What exact information should be provided can be for further consideration. |
| 16 | The PCF takes the energy related information into account for URSP generation. The energy related information in this solution can be from the OAM. | The PCF monitors the energy related information for a slice (EnC/EnE, renewable energy information) from OAM. | The PCF updates URSP rule like time window, priority in RSD. | No | UE policy updates by PCF for energy saving can be viewed as principle in normative works. |
| 17 | PCF adds an IE of "reason for a policy change" in charging info of the associated PCC rule when service adjustment is applied for an SDF due to energy saving. SMF sends the reason for a policy change together with existing charging info to CHF. | Not covered in this solution. | Not covered in this solution. Can be AM policy, or SM policy updates. | Yes | This is proposed as a baseline in normative works for charging enhancement to record service adjustment due to energy saving. |
| 18 | There is an Energy Saving indication in the Session Management Subscription data per DNN/S-NSSAI. The SMF retrieves the Session Management Subscription data using Nudm_SDM_Get. The SMF provides Energy Saving indication to PCF in the SM policy association establishment. Later on, when the PCF make the policy decision, it can take this Energy Saving indication into consideration. | The PCF applies service adjustment based on Energy consumption information from EIF and Energy Saving indication in the Session Management Subscription data per DNN/S-NSSAI. | ? | No | The solution does not describe which Energy consumption information from EIF is monitored. |
| 24 | PCF performs service adjustments for selected UEs and PDU sessions based on energy consumption events from EIF. The PCF subscribes to EIF notifications, assessing parameters such as energy consumption percentile, energy consumption per bit, and UE energy behaviour categories, to identify UEs that consistently exceed configured thresholds. Based on operator policies, the PCF then applies policy-driven service adjustments, such as modifying QoS or restricting certain applications by updating policies to the AMF and SMF, which can include the received categorization information in subsequent requests to the CHF. | EIF provides info identifying the UEs that are in a higher percentile band or higher energy consumption per bit, persistently higher for N samples of energy related information received from the, where N is the number of samples defined based on operator policies. | AM Policy or UE Policy updates. | Yes. UE energy categorization information | AM policy or UE policy updates by PCF for energy saving can be viewed as principle in normative works. It is not clear whether the percentile event is robust enough. |
| 25 | PCF triggers Background Data Transfer (BDT) policies re-negotiation for energy optimization based on energy-related triggers when BDT is ongoing. The PCF invalidates the ongoing BDT policy after a defined grace period, provides updated candidate BDT policies via the NEF, and communicates the reason for change in the BDT notification. | The energy consumption, co2 emission, or renewable energy for a network slice (from OAM), Application or the 5GC reached curtained threshold. | PCF provides a BDT policy with a future time window which is energy efficient for core network. | No Reuse existing mechanism to apply normal charging if BDT is not discontinued. | This is proposed to be considered in normative works. |
| 26 | The AF can subscribe via NEF or directly to PCF event exposure to receive early notifications when a UE's energy consumption crosses an early threshold, allowing time for application or UE traffic adjustments before service degradation (e.g., QoS reduction, RAT restrictions) takes effect. AFs may request temporary exclusions from such service degradation and PCF considers these inputs in its policy decisions, while also considers the Energy Saving Indicator provisioned in the UDM. | Thresholds for energy consumption of the required granularity from EIF according to operator policy and AF request | PCF provides earlier notification to AF energy saving policy is about to trigger. PCF performs energy saving policy and provides notification to AF. AF can request temporary exclusions of energy saving policy control for certain services. | AM policy, SM Policy updates. | The solution does not describe which Energy consumption information from EIF is monitored. PCF applies service adjustment for AM and SM policy can be viewed as a general principle to be used in normative works. The slice level or 5GC energy saving trigger should be used before fine-granular energy related information, like per UE, becomes robust. |
| 27 | The PCF can subscribe to the EIF to receive energy consumption data at various granularities (per UE, Network Slice, PDU Session, QoS Flow) and, based on factors such as subscription data Energy Saving indications, operator policies, and local configuration, determine which UEs or sessions require energy usage control. The PCF then informs the SMF of the selected targets, and the SMF adjusts session control parameters accordingly to optimize network energy savings, both during initial session establishment and through dynamic policy control. | PCF monitor UE/Network Slice per UE/PDU Session per UE/QoS Flow from EIF. PCF gets Energy Saving indication for per-UE/Network Slice/DNN is stored in the Subscription Data of a UE to provide a policy to control the energy used by a UE to SMF. PCF based on operator policy and local configuration and above information to make policy decision | The PCF may inform the SMF of the selected UE/PDU Session of the UE/QoS Flow of the UE that should be under energy usage control. The SMF may adjust the session control parameters related to base on pre-configuration information, operator policy. | No | This solution describes the control parameters such inactivity timer, buffering size, it is not clear whether SM policy like PCC rule is changed. Whether we need additional indication or the existing Energy Saving Indicator at UE level is enough? It is also not clear whether this is just indicator or including energy saving policy like thresholds, how to adjust QoS? |

# 2. Text Proposal

It is proposed to agree the following to 3GPP TR 23.700-67

First change (all new)

### 7.1.2	Agreed Principles for KI#2

The interim agreements on principles for KI#2 are as follows:

-	The PCF can make the following policy control decisions for network energy saving based on AF requests and/or operator policies, while also considering the energy saving indicator found in subscription data.

- AM Policy.

- SM policy.

- UE policy, like generating and updating URSP rules.

-	The AF can supply a list of QoS requirements at the SDF level for both GBR and Non-GBR flows. The PCF is permitted to apply these QoS requirements to support energy-saving measures, and it may inform the AF of the specific QoS requirement it has selected.

-     The PDTQ policy negotiation between the PCF and the AF may be enhanced to consider an energy optimization time window, while ensuring that the QoS requirements are met. For example, the PCF may, based on AF-provided input containing a list of acceptable QoS requirements or according to operator policy, adjust the time window and/or the QoS for data transfer accordingly.

-  The PCF may initiate BDT policy re-negotiation during an ongoing BDT session and provide the AF with an updated list of available BDT policies. Based on the received list, the AF may select a new BDT policy and, if desired, terminate the current BDT to obtain a more favourable charging offer for the data transfer. The PCF shall apply normal charging if the BDT remains active after the expiry of a defined graceful period.

-    The energy consumption related information at a large granularity, such as at the network slice level or 5GC level, obtained from the OAM, may be utilized as a trigger for service adjustment to achieve a significant impact on network energy consumption. Fine-granular energy consumption information, like UE, per PDU Session per UE, etc, are not robust enough to be used for service adjustments.

-  The PCF should notify the SMF when making adjustments that affect UE services. The SMF, in turn, should provide the reason for the service adjustment to the CHF for statistical purpose.

Second change (all new)

### 7.2.2	Topics for further consideration for KI#2

The following topics/principles are for further consideration:

-	The PCF may subscribe to notifications from the EIF regarding fine-granular energy-related information, once the solution specified in KI#1 has provided such information sufficiently robust:

-	energy Consumption information at UE, S-NSSAI per UE, PDU Session and SDF granularity.

-    future energy-related information defined in KI#1, including Renewable energy consumption or ratio at various levels of granularity.

-	The PCF may subscribe to the EIF for slice-level energy consumption information, which is re-exposed via the SBI by the EIF based on data retrieved from the OAM. This mechanism is used instead of the PCF deriving the slice energy consumption by aggregating per-UE per-slice energy information received from the EIF:

-	Energy Consumption information at Network Slice granularity.

-	renewable energy consumption ratio at Network Slice granularity.

-    The AF may provide the PCF with energy saving policies, such as thresholds for triggering service adjustments and guidelines for adjusting QoS requirements at various levels (e.g., UE level, SDF level). Further study is required to determine the types of policies that may be supported.

# * * * * End of changes * * * *