3GPP TSG-SA2 Meeting #171	S2-2508928

13 – 17 Oct, 2025, Wuhan, China

Source:	Ericsson

Title:	(KI#2) Resolve editor´s notes in Sol #28

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 /Rel-20

Abstract of the contribution: This contribution proposes to update Solution #28 to resolve the editor´s notes.

# 1. Discussion

This PCR proposes to update Sol #28 to resolve the editor´s notes.

In addition, the Sol #28 is generalized (analysis of user plane traffic towards malicious and/or forbidden sites was an example) to analysis of user plane traffic patterns.

Specifically, the following editor´s notes are resolved:

Editor's note:	Whether and how can UPF use the model is FFS.

The above editor´s note is proposed to be resolved by providing an example on how UPF can use the ML Model to detect user plane traffic pattern(s).

Editor's note:	Which the input and output of the model is FFS.

The above editor´s note is proposed to be resolved by indicating the input data is the same as the input data retrieved for training and the output data is not to be specified since it is not exposed externally to UPF.

Editor's note:	Whether there is the relationship with the AnalyticsID on PFD determination, if so which the relationship is FFS.

The above editor´s note is proposed to be resolved as the output of the AnalyticsID on PFD determination includes the new/updated PFDs, while the output of the proposed ML Model is directly the detected traffic pattern(s), not the rules (PFDs) which are used to detect a certain application.

Editor's note:	How to avoid extensive signalling load between NWDAF and UPF during training is FFS.

The above editor´s note is proposed to be resolved by proposing to use existing mechanisms (e.g. UE sampling or CT4 proposed enhancements).

Editor's note:	The indication to request a model for detection of traffic towards malicious and/or forbidden sites is FFS.

The above editor´s note is proposed to be resolved as some indication (e.g. application identifier) is needed, so MTLF knows which ML Model to train.

Editor's note:	What the trigger to step 1 is for the UPF to request a trained ML Model is FFS.

The above editor´s note is proposed to be resolved by proposing the trigger to step 1 for UPF to request a trained ML Model can e.g. be based on UPF local configuration (e.g. to periodically request an updated ML Model).

Editor's note:	Whether the NWDAF retrieves content filtering DB information via NEF is FFS.

The above editor´s note is proposed to be resolved as the solution is generalized to analysis of user plane traffic patterns.

# 2. Text Proposal

It is proposed to capture the following changes into TR 23.700-04.

*** Start of changes ***

## 6.28	Solution #28: Analysis of user plane traffic patterns

## 6.28.1	High Level principles

The high level principles of this solution are as follows:

-	A consumer (i.e. UPF) retrieves a trained ML Model which allows detection of UP traffic patterns from NWDAF (MTLF).

-	The training of this ML Model is performed mostly based on UPF data (e.g. UP traffic parameters).

-	The UPF uses this ML Model for detection of user plane traffic patterns. The ML Model is associated with a N4 rule (e.g. a PDR with PDI set to an application identifier, corresponding to the specific traffic pattern to be detected).

-

### 6.28.2	Description

This solution aims the following aspects for KI#2:

-	UPF retrieving the ML Model is a solution to minimize the load for doing inference in the control plane.

This solution proposes a 5GC enhancement to support analysis of user plane (normal and/or abnormal) traffic

patterns.

Once the ML Model is retrieved, UPF does inference. For inference, the input data is the same as the input data retrieved for training. The output is not to be specified since it is not exposed externally by UPF.

### 6.28.3	Procedures

Figure 6.28.3-1 depicts the procedure for the proposed solution, for the specific case UPF (acting as NWDAF Service Consumer) requests a ML Model for UPF-based detection of traffic patterns, where the ML Model is trained by an NWDAF containing MTLF.

In order to avoid extensive signalling load between NWDAF and UPF during training, the reuse of existing mechanisms (e.g. sampling UE PDU sessions) is proposed.

Editor's note:	Whether the UPF can retrieve a ML Model from OAM is FFS.

Figure 6.28.3-1: Proposed solution for retrieval of a trained ML Model from NWDAF with inference at UPF

1	NWDAF Service Consumer (UPF) subscribes to NWDAF containing MTLF for a trained ML Model related to detection of user plane traffic patterns, by invoking the Nnwdaf_MLModelProvision_Subscribe service operation, including the following input data:

-	Indication (e.g. an application identifier) to request a model for detection of the traffic pattern of interest.

-	Optionally, Vendor ID, ML Model Filter Information (e.g. Area of Interest), Target of ML Model Reporting, ML Model Target Period, Time when model is needed and ML Model Monitoring Information.

-	If vendor specific information is required, then the ML Model Interoperability Information is included.

-	If the UPF supports multiple AI/ML Models, indication of support for multiple ML Models, optionally with Number of ML Models and Accuracy level(s) of Interest.

When a ML Model subscription is received, the NWDAF containing MTLF may:

-	determine whether existing trained ML Model(s) can be used for the subscription then steps 2 to 6 are skipped; or

-	determine whether triggering further training for the existing trained ML Models is needed for the subscription;

If the service invocation is for a subscription modification or subscription cancelation, the NWDAF service consumer includes an identifier (Subscription Correlation ID) to be modified in the invocation of Nnwdaf_MLModelProvision_Subscribe.

The trigger to step 1 for UPF to request a trained ML Model can e.g. be based on UPF local configuration (e.g. to periodically request an updated ML Model).

2.	If the NWDAF containing MTLF determines that further training is needed, this NWDAF may initiate data collection from

UPF to retrieve user plane traffic information, by invoking the Nupf_EventExposure_Subscribe service operation, including as eventType="USER_DATA_USAGE_MEASURES" and the target SUPIs.

NOTE:	It is assumed both UPF event subscription and reporting applies per SUPI, but optimizations (to minimize signalling) as discussed by CT4 can also be applied.

3-4.	The UPF collects data for the above event and notifies NWDAF by invoking the Nupf_EventExposure_Notify service operation, including the eventType="USER_DATA_USAGE_MEASURES" and the event information, e.g. 5-tuple(s), SNI(s), URL(s) for the target SUPIs.

5.	Optionally, e.g. depending on the requested ML Model (for detection of the traffic pattern(s) of interest), NWDAF may initiate data collection from other NF(s), e.g. from AF (via NEF if AF is external/untrusted) to retrieve information relative to (expected) traffic pattern.

6.	The NWDAF containing MTLF, based on collected data and e.g. by correlating the information about the (expected) traffic pattern and the usage per application retrieved from the UPF, trains the ML Model which allows UPF to detect user plane traffic pattern(s).

7.	NWDAF containing MTLF notifies the NWDAF Service Consumer (e.g. UPF) by invoking the Nnwdaf_MLModelProvision_Notify service operation, including the following information:

-	ML Model identifier and ML Model Information;

-	Optionally, ML Model Filter Information and/or Target of ML Model Reporting, if the ML Model provisioning request includes multiple ML Model Filter Information and/or Target of ML Model Reporting;

-	Optionally, Indication of whether the ML Model identifier is updated (e.g. retrained ML model);

-	Optionally, Validity period, Spatial validity, ML Model accuracy Information.

8.	The NWDAF Service Consumer (e.g. UPF), based on the above information, retrieves the ML Model and uses it to detect user plane traffic pattern(s). It is assumed UPF applies the corresponding policies (e.g. QoS handling) based on existing policy framework (e.g. via a PCC rule for an application ID relative to the user plane traffic pattern(s)).

### 6.28.4	Impacts on services, entities and interfaces

NWDAF containing MTLF:

-	New ML Model which allows UPF to detect UP traffic pattern(s), which is trained mostly based on UPF data (e.g. UP traffic parameters).

-

UPF:

-	Retrieval (either from NWDAF containing MTLF or via OAM (depending on coordination with SA5) of the trained ML Model to detect traffic pattern(s).

-	Use the trained ML Model (inference) to detect traffic pattern(s).

-	Enforcement of policies (e.g. QoS handling) based on existing policy framework (e.g. via a PCC rule for an application ID associated to the traffic pattern(s)).

*** End of changes ***