SA WG2 Meeting #171	S2-2508936

13 – 17 Oct, 2025, Wuhan, CN

Source:	Apple

Title:	[WT#1.2, SBA Framework] SBA enhancements in 6G

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution provides updated technical reasoning, gap and impact analysis behind WT#1.2 for SBA Framework and proposes a revision of work task and key issue descriptions based on the progress made in SA2#170.

# 1   Justifications

The 6G SA2 SID in SP-250806 [x2] approved at TSG SA #108 contains the following sub–work task corresponding to the SBA framework:

1.2.	Study whether and how to support and/or enhance the following aspects in 6G: the SBA framework, network slicing, network sharing, user plane architecture, QoS framework, policy framework, network exposure framework, architecture for specific scenarios e.g. fixed wireless access, localized service access.

## Architectural Motivation

It is broadly assumed that the SBA framework defined in 5G can be reused for the 6G system. SBA offers several advantages such as extensibility to add new functionality in a simple manner, loose coupling between services, and reusability. However, 3GPP needs to evaluate if the functionality and system features inherent to that framework is still sufficient to fulfil the requirements for NF communication in 6G.

## Gap Analysis

Compared to traditional point-to-point signalling with its carefully crafted procedures between rather fixed termination points, SBA is known to cause an increase in signalling traffic within the Core Network. Furthermore, the proliferation of NFs has been observed in the 5G era. It is anticipated that the number of NFs will continue to increase in the 6G landscape, coupled with the prevailing trend of decomposing functional components into smaller, self-contained services. More granular services offer smaller, more specific functionalities, necessitating multiple interactions to complete intricate tasks. This approach enhances flexibility and adaptability but requires more signalling message exchange. Consequently, it is probable that the signaling load will increase further. Another aspect is that a high number of NF interdependencies translates into a need to reduce latency. Stateless NFs, lacking persistent storage, frequently perform fetch and update operations, leading to additional latency. A database instance may also store data from one or multiple NFs within a chain of messages exchanges, resulting in latency for each step. Both of these aspects, signalling load and reduced delays, represent performance requirements that SBA will have to fulfil in the future.

Observation 1: SBA involves a lot of signalling traffic and requires reasonable TN latency. The number of NFs and related interactions between NFs is expected to grow further.

Rel-16 introduced the SCP to help enable dynamic scaling and management of communication and services in the 5G network which could be considered somewhat similar to the STP in 2G/3G or the DSC/DRA in Diameter. Even though the SCP includes functions for indirect communication with/without delegated NF discovery, the process of querying the NRF is still a cumbersome operation (with lots of parameter information being provided) where the NRF remains a critical resource in the 3GPP system. Since these aspects are central to the operation of SBA, potential optimizations and options to enhance the resiliency of the system can be included to the study.

Observation 2: Efficient NF discovery and/or grouping of NFs may enhance the resiliency of the NF system.

In relation to the interworking architecture, 4G-5G introduced the N26 interface, which is a P2P interface between MME and AMF. As 5G adopted SBA and assuming 6G is going to be based on SBA also, 3GPP can study adopting a service-based interface between 5G and 6G networks. From SA2 perspective, architectural implications, e.g., to describe NF service and related functional aspects, can be studied under WT#2 (in general) or as part of WT#1.2 under SBA framework (for more specific interface aspects).

In the current 3GPP system N4 is a P2P interface between UPF and SMF. 5G studied UPF event exposure and defined Nupf services to expose UPF related information. In continuation of this trend, 6G may study incremental enhancements for better scalability and flexibility, including whether and how to evolve N4 into a service-based interface, e.g., to enable direct communication of some information without routing through AMF or SMF. From architectural perspective and according to the conclusions of SA2#170, whether and how to extend SBA for the user plane control interface to the UPF can be studied under WT#1.2 under User Plane Architecture.

Observation 3: In conjunction with the overall architecture, the 6G study may evaluate if additional service-based interfaces need to be defined. Based on the progress in SA2#170, this can be studied in respective other Work Tasks.

## Technical Impact

Overall, the technical impact this WT may have on the 3GPP system may be incremental. There is a potential to optimize SBA transport while reusing general principles from 5G. In addition, several areas need to be analysed, which may influence the NF system in general and the interworking between different NFs in particular. Also, API parameter enhancements related with updates to SBA transport protocols may be expected. Furthermore, any new SBIs will need to be evaluated considering how it maps into the overall architecture and what are the Pros and Cons.

## Conclusions

In conclusion, this contribution discusses the scope of the SA2 6G SID for WT#1.2 in Rel-20. We have the following observations:

Observation 1: SBA involves a lot of signalling traffic and requires reasonable TN latency. The number of NFs and related interactions between NFs is expected to grow further.

Observation 2: Efficient NF discovery and/or grouping of NFs may enhance the resiliency of the NF system.

Observation 3: In conjunction with the overall architecture, the 6G study may evaluate if additional service-based interfaces need to be defined. Based on the progress in SA2#170, this can be studied in respective other Work Tasks.

Proposal: Based on the observations and analysis above and following the discussions in SA2#170, this paper proposes a revision of the Work Task scope as well as an update of the Key Issue based on the postponed document S2-2508112 at SA2#170.

# 2   Proposed Changes to 23.801-01

**** First Change ****

## Annex A.X. WT#1.2 Scope for SBA Framework

The following is work scope proposal for SBA related topics.

- Study whether and how to optimize NF discovery and selection for efficient message forwarding and routing compared with 5G. This also includes aspects such as how to limit/reduce NF signalling, NF interdependencies, and information exposure between NFs.

- Study whether and how to improve NF resiliency compared with 5G.

NOTE x: Impacts on SBA and applicability can be studied in other WTs such as WT#1.2 for user plane architecture and WT#2 for interworking between 6GS and 5GS.

**** Second Change ****

## 5.X Key Issue #X: SBA Framework

This key issue focusses on enhancement of SBA related topics, including the following aspects.

- Study whether and how to optimize NF discovery and selection for efficient message forwarding and routing compared with 5G. This also includes aspects such as how to limit/reduce NF signalling, NF interdependencies, and information exposure between NFs.

- Study whether and how to improve NF resiliency compared with 5G such as WT#1.2 for user plane architecture and WT#2 for interworking between 6GS and 5GS.

NOTE x: Impacts on SBA and applicability can be studied in other WTs.

**** End of Changes ****