3GPP SA WG2 Meeting #171	S2-2509007

Wuhan, China, October 13 – 17, 2025	(was S2-2507437)

Source:	Samsung

Title:	KI#3: Support for emergency calls over NB-IoT NTN

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC/ Rel-20

Abstract of the contribution: Solution to address the support for emergency calls over NB-IoT NTN.

# 1.	Discussion

Proposes changes to support IMS emergency calls over NB-IoT NTN.

# 2.	Text proposal

It is proposed to agree the following changes for TS 23.700-19:

*************************** Start of Changes ***************************

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 |
| X |  |  | X |  |

***************************    Next Changes   ***************************

## 6.X	Solution #X: Support for emergency services over NB-IoT NTN

### 6.X.1	High level principles

This solution has the following high level principles:

-	This solution assumes that emergency call SIP signalling can be initiated using CP CIoT EPS optimization as described for multiple solutions of KI#1.

-	MME include an indication for Emergency Services Support within the Attach/TAU Accept message to the UE when the UE is accessing over NB-IoT NTN;

-	eNB broadcast cell supports Emergency Services over NB-IOT NTN for UEs in limited service state. For UEs in limited services state the UEs choose NB-IOT NTN RAT as a last priority because it may have delay when compared to other RATs.

-	Handling suspension of data transfer while ESM procedure is executed.

-	Handling when NAS PDUs are queued in the UE for transmission and emergency service is triggered.

-	Network should not activate the serving PLMN rate control or APN rate control, for emergency PDN connections.

-	The UE is allowed to send NAS PDUs related to emergency PDN connection even if control plane data back off timer is running.

### 6.X.2	Description

To support IMS emergency services over NB-IoT NTN following changes are expected:

1)	MME needs to include an indication for Emergency Services Support within the Attach or TAU Accept message when for UE is accessing over NB-IoT NTN.;

2)	eNB broadcast cell supports Emergency Services over NB-IOT NTN for UEs in limited service state. For UEs in limited services state the UEs choose NB-IOT NTN RAT as a last priority because it may have delay in service when compared to other RATs.

3)	Handling suspension of data transfer while ESM procedure is executed:

TS 24.501 clause 6.1.2 states below:

“During procedures related to EPS bearer contexts, the MME and the UE shall not initiate the transport of user data via the control plane procedure until the ongoing procedure is completed.

Except for the remote UE report procedure and ESM information request procedure, during transaction related procedures, the MME and the UE shall not initiate the transport of user data via the control plane procedure until the ongoing procedure is completed.”

Currently, the UE and MME suspends to send/receive user data while executing the ESM procedure for e.g. PDN connection establishment for normal services. This suspension is OK for normal user data. But if the user data is a SIP traffic related to emergency services/emergency PDN connection, then this cannot be suspended. i.e. while the ESM procedures are ongoing, the UE and MME is allowed to send/receive ESM DATA TRANSPORT message carrying user data if the NAS PDU is related to emergency PDN connection(by checking the EPS bearer ID) and those NAS PDUs are processed with priority.

4)	It has been observed in the field that it takes up to minutes to transmit a NAS PDU carrying user data using NB-IOT RAT from the UE. If more than one NAS PDUs (carrying user data) are queued for transmission within the UE, and if the emergency call is triggered, it may significantly delay the transmission of NAS PDUs of emergency services. To prioritize NAS signalling related to emergency call for e.g. of SIP signalling the NAS packets queued in the UE including the one at lower layers are deprioritized by the UE before sending the NAS signalling related to emergency call. For this UE will need to re-adjust the NAS count values/security header used in previous generated NAS PDU. How UE achieves this is up to UE implementation.

Editor's note:	For bullets 3 and 4, co-ordination with CT1 is expected.

5)	Network should not activate the serving PLMN rate control or APN rate control, for emergency PDN connections.

6)	The UE is allowed to send NAS PDUs related to emergency PDN connection even if control plane data back off timer is running.

### 6.X.3	Procedures

Existing procedures are reused.

### 6.X.4	Impacts to Services, Entities and Interfaces

UE

-	Should not suspend user data transfer using CP CIoT EPS optimization for emergency PDN connections.

-	Prioritize the NAS PDUs of emergency services over other NAS PDU carrying other user data.

-	Follow current domain selection procedure, Additionally, for UEs in limited services state the UEs choose NB-IOT NTN RAT as a last priority because it may have delay when compared to other RATs.

-	The UE is allowed to send NAS PDUs related to emergency PDN connection even if control plane data back off timer is running.

MME

-	Support to indicate Emergency Services Support in NAS accept message for NB-IOT RAT similar to TN.

-	Should not activate serving PLMN rate control or APN rate control, for emergency PDN connections.

-	Should not suspend user data transfer using CP CIoT EPS optimization for emergency PDN connections

eNB of NB-IOT NTN

-	Support emergency support bit to support UEs in limited service state. Similar to the one available for TN.

*************************    End of Changes   ************************