3GPP TSG-WG SA2 Meeting #171	S2-2509013

Wuhan, CN, 13th Oct – 17th Oct, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	[WT#1.2, Localized Service] Justifications and Proposals for WT and Key Issue

Document for:	Approval

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC / Rel-20

Abstract: This document provides a gap analysis of 5G architecture and argues for the need to improve architecture support for PLMN offered localized service access.

1. 	Introduction

6G provides the opportunity for operators to achieve CAPEX/OPEX reduction by introducing improvements to the overall 3GPP system performance, e.g., by means of simplifying the overall system, integrating of new technologies, etc. 5G core enabled a cloud-native deployment, and promoted business opportunities by unlocking the possibility of providing services to verticals alongside the traditional consumer services. To shape the 6G architecture to better support the localized service access, this document first provides a gap analysis and identifies the limitations of 5G and subsequently provides architectural motivations as the justifications for studying the architecture support for PLMN offered localized service in 6G.

2. 	PLMN offered localized service access

2.1 	Localized service access

The service requirements for providing access to localized services are captured in clause 6.41 in TS 22.261. The hosting network providing the access to localized services can be a PLMN or NPN. 5G localized service focus on the scenario where localized services are provided by NPN (SNPN or PNI-NPN) as specified in Annex N of TS 23.501, the scenario where localized services are offered by PLMN are not fully investigated. This paper would like to focus on the scenario where the localized services are offered by PLMN.

Localized services can be offered via PLMN to support various services in public area, for example:

-	Provisioning local data connectivity service and voice service in the disaster area in response to an urgent event (e.g., disaster, emergency and DDoS events) to improve network resilience, as described in clause 5.6.2 of TR 22.870.

-	Verification of the new network services (e.g., XR services) rolling out on the local area network, running test on the selected users in response to new business contract to enable agile and zero-downtime rollout of new services, as described in clause 5.9.5 of TR 22.870.

-	Offering customized local area network service (e.g., customized capabilities, and more localized management) for vertical scenarios like factories or campuses, adapting to differentiated and changing demands on network services of various local stakeholders, to simplify the network and improve the network flexibility, as described in clause 5.9.6 and 11.9 of TR 22.870.

Offering localised services requires the operator to configure part of the PLMN as a local area network. The local areas network should be easy to deploy, offer the adequate performance and efficient costs, be sufficiently flexible to meet different demands etc. Taking these points into consideration, the PLMN offered localized service access can be characterized by the following attributes:

-	it is possible to instantiate millions of PLMN offered localized networks

-	the requirements for the PLMN offered localized services become increasingly diverse across difference scenarios and are dynamically changeable

-	flexible enough to enable agile service delivery

-	allow freedom to enable local control: local subscription, local signalling control, and local traffic forwarding for users, under the lightweight/no control of the PLMN

-	changes of PLMN offered localized network are confined locally

-	flexible inter-interactions or inter-connectivity among networks of the same PLMN operator

As specified in Annex N of TS 23.501, Localized Service is a service, which is provided at specific/limited area and/or can be bounded in time. The DNN/S-NSSAI can be used as service parameters to determine each Localized Service. And based on attributes characterized above, Figure 2.1-1 gives an architectural example of PLMN offering multiple localize services with each core serving a particular localized service.

Figure 2.1-1: Architecture for PLMN offered localized service access

2.2 	Gap Analysis

SA1 has proposed new requirements to support certain level of local control for PLMN offered localized service access, TR 22.870, clause 5.6.2, [PR 5.6.2.6-1] Subject to operator policies and service level agreements, the 6G system shall enable operators to provision network services as part of the operator’s PLMN network on-demand, e.g., in response to an urgent event (e.g., disaster, emergency and DDoS events), with certain level of local control and specific functionalities in a given area during a specific time period. 5G defined many features that could be used to provide localized service towards verticals such as e.g., NPN, EDGE, Slicing etc., however, those features only provide the centralized control for CP signalling, user subscription, policy management. Figure 4.2-1 illustrates how these features can be used to create localized service access but they also suffer from a number of shortcomings. In particular:

-	PNI-NPN requires PLMN to offer UDM, PCF, CAG and slicing services;

1) PNI-NPN uses the CAG feature for access control (i.e., prevent unallowed UEs to access the NPN).

1.1) The CAG feature introduces impacts to UE and RAN, thus 5G localized services have dependencies on UE and RAN. The localized services take a hard time to take off because of the complex implementation of CAG feature on UE/RAN and have not yet been widely commercialized till now.

1.2) The network providing localized services is bound to the localized services. Ideally, this network can only be selected during network selection if the UE is subscribed to and aiming to access the localized services. However, the CAG ID is used for network selection and this CAG is independent from any S-NSSAI as specified in clause 5.30.3.1 of TS 23.501. Instead, URSP mechanism is used for access to localized service. This introduces on extra UE dependency that delays the commercialization of localized service.	2) PNI-NPN supports interaction with PLMN network via roaming architecture as specified in Annex N.2.1 of TS 23.501. Even when PNI-NPN is deployed by the same operator as the PLMN network, the existing roaming architecture is used as specified in Annex N.2.1 of TS 23.501.

2.1) When networks providing localized services and the PLMN network are deployed by the same operator, the roaming architecture should not apply for the interactions among the Core network of those networks.

2.2) Policy management: for the home routed roaming case, V-PCF is not involved and thus policy control is managed by H-PCF in the HPLMN. The local policy control is not available in this case.

2.3) Subscription management: for both local breakout and home routed roaming case, UE subscription is managed by UDM in the H-PLMN. This means that local control of the access and mobility management, session management is not available in this case. What is more, the subscription to localized service (time/location validity) need frequent modification of UE subscription. Frequent modification of UE subscription in HPLMN is not efficient and large signalling would be caused.

-	EDGE requires PLMN to offer AMF, PCF, UDM and SMF services;

-	SLICING requires PLMN to offer network resources such that AMF, SMF, UDM, PCF, UPF are shared among tenants;

-	SNPN can be deployed by PLMN operator, but SNPN introduces NID feature that have the same issues as CAG feature. In addition, the SNPN architecture uses underlay-overlay arrangements to support simultaneous connection to localized services and PLMN services. This arrangement is both inefficient and cumbersome for fulfilling the interworking requirement proposed in clause 5.6.2 of 3GPP TR 22.870: [PR 5.6.2.6-2] Subject to operator policies and service level agreements, the 6G system shall enable a network operator to authorize a UE, that is subscribed to local network services, to access services from the PLMN of the same operator.

Figure 2.2-1: Architecture for PNI-NPN, Edge, Slicing, and SNPN

5G also attempts to define the certain level of local control on dedicated aspects: e.g., session management in ETSUN or information exposure in UPEAS. However, those features only allow very limited local control:

-	In case of ETSUN, when the UE is outside of the SMF service area, an I-SMF and I-UPF are inserted to locally serve the UE location, and SMF and UPF anchor services are still remained for the PDU session.

-	UPEAS only enables the local exposure for certain information (e.g., Private UE IP, Usage Data Usage etc.) via local NEF and UPF.

Observation 1: PLMN offered localized service access with the desired level of local control (e.g., Policy management, subscription management, access and mobility management, session management) cannot be fully supported in the context of 5G architecture. Additionally, the access control mechanism used for networks providing localized service doesn’t consider the UE’s localized service information and introduces heavy UE dependencies that hinder the commercialization.

As described in clause 11.9 and 5.6.2 of TS 22.870, the UE that is subscribed to the localized services can also use PLMN services or other localized services deployed by the same operator, [PR 11.9.6-2] Subject to an agreement between a PLMN operator and authorized 3rd party, operator policies and regulatory requirements, the 6G network shall support a mechanism to authorize and authenticate UEs owned by a trusted authorized 3rd party to access services from the PLMN operator. When the NRF is deployed at slice-specific level and the localized services are bound to a specific slice, there is no possibility to enable the interactions between the NFs tied to localized services (e.g., for verticals) and other NFs in the PLMN. When the NRF is deployed at PLMN-level or shared-slice level as the centralized NRF, in order to discover NFs tied to localized services by other NFs (e.g., AMF, SMF) in the network (or vice versa), following the current 5G mechanism, those NFs should register their NF profiles directly to the centralized NRF, or indirectly via an intermediate NRF serving the corresponding localized services, as specified in clause 5.2.2.2.3 of 3GPP TS 29.510.

c) the registering NRF may include nrfInfo which contains the information of e.g. udrInfo, udmInfo, ausfInfo, amfInfo, smfInfo, upfInfo, pcfInfo, bsfInfo, nefInfo, chfInfo, pcscfInfo, lmfInfo, gmlcInfo, aanfInfo, nfInfo and nsacfInfo in the nfProfile locally configured in the NRF or the NRF received during registration of other NFs, this means the registering NRF is able to provide service for discovery of NFs subject to that information;

This means the centralized NRF holds the profile for all NFs (not only NFs for PLMN services, but also NFs tied to localized services), causing extra load on NRF. And more importantly, this brings security risks because any topology changes (including incorrect or malicious topology information) from substantial localized services are updated to the centralized NRF, and also this potentially affects other PLMN/localized services in the PLMN network.

Observation 2: If localized service uses the current 5G NF management mechanism, it might bring heavy load on the centralized NRF and any changes of localized services will cause wide impacts across the entire network.

In case of cross-domain interaction in 5G, e.g., roaming between visiting and home network as shown in Figure 2.2-2, NF discovery request and service request are both rerouted over the home network as shown in the below figure, causing extra signalling and procedure delay. This problem also exists when a UE, that is subscribed to local network services, accesses PLMN services, as described in clause 11.9 and 5.6.2 of TS 22.870. In this case, the local network services and PLMN services can be treated as visiting PLMN and home PLMN respectively.

Figure 2.2-2: NF discovery and service request signalling in roaming cases

Observation 3: Current 5G NF discovery and selection mechanism is inefficient for cross-domain message forwarding and routing in case of localized service access.

When UE accesses localized services, local NFs, i.e., AMF and SMF in 5G, need to discover the centralized UDM for UE subscription data retrieval. As shown in Figure 2.2-3, local NFs tied to localized services can subscribe to changes of the UDM registered in the central NRF via the local NRF. In case of network upgrade, the centralized NRF needs to notify all subscribed NFs including local NFs, of NF profile changes of the upgraded NF, while such NF profiles can be the UDM that includes lots of SUPI ranges. This results in signalling storm of notification messages with large payload in the whole network.

Figure 2.2-3: Subscribe-Notify with intermediate NRF

Observation 4: If localized service uses the current 5G NF management mechanism, it might cause signalling storm with large payload packet when the network upgrades, which happens frequently for localized service.

Study how to design 6G architecture to support the PLMN offered localized service access with certain level of local control is necessary. The motivation is not only to satisfy the SA1 requirements, but also, this promotes business opportunities for PLMN operators, and extremely enlarges the monetization space for operator’s PLMN network. In addition, this can help gain benefits on the operators’ CAPEX/OPEX reduction when providing localized services, e.g., by signaling overhead reduction in case of cross-domain interactions and avoiding signalling storm in case of network upgrade or network scaling.

Proposal: It is proposed to study 1) how to enable the 6G architecture to support certain level of local control for PLMN offered localized service access, and 2) how to enable efficient interactions between networks providing localized service and PLMN networks, and between different networks providing localized service.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.801-01.

# * * * * First change * * * *

Annex A.X. WT#1.2 Scope for localized service access

The scope of localized service access is to study the overall architecture design and enablers for localized service access in 6G, including:

NOTE 1: 	Support for localized service as defined in 5G is the starting point for the discussion on support of localized service access in 6G.

- 	architecture impacts to enable local control of UE for localized service access within the network providing localized service, including:

- 	access and mobility management, session management, policy management, subscription management for UE attempting for localized service access

- 	authorization and authentication of UE for localized service access

- 	interface and interaction procedure between PLMN network and networks providing localized service, or between networks providing localized service:

NOTE 2: 	The networks providing localized service can be part of the same PLMN network, e.g., they are deployed by the same PLMN operator.

- 	Whether and how to support UE mobility between PLMN network and network providing localized services, or between networks providing localized service

- 	Whether and how to support service continuity for localized service, including access to localized service after leaving the original network providing localized service

# * * * * Second change * * * *

5	Key Issues

Editor's Note: This clause identifies key issues to be addressed.

5.Y	Key Issue #Y: Architecture impacts to support localized service offered by PLMN

To offer localized services, the network providing the localized service (e.g., local core network or access network) can be configured by MNOs as part of the PLMN network. The network providing the localized service should be easy to deploy and use, provide adequate performance, be cost effective, sufficiently flexible to meet demands etc. The key issue of localized service access is to study the overall architecture design and enablers for localized service access in 6G, including:

-	study how to enable the 6G architecture to support certain level of local control for PLMN offered localized service access, including:

- 	How to enable the architecture to support local control for the UE accessing the PLMN network providing localized service, e.g., access and mobility management, session management, user subscription, policy management, topology management etc.

- 	How to authenticate and authorize a UE, that is subscribed to localized services offered by PLMN network, to access the localized services when the UE is in specific area, with minimum impacts to PLMN network.

- 	How to support fast delivery of the new or updated services/capabilities for a specific localized service within the PLMN network providing localized service, and with no/minimal disruption to existing services.

- 	How to enable a UE to access the PLMN network providing localized services.

-	study how to enable efficient interactions procedure between PLMN network and networks providing localized service, or between networks providing localized service when the networks providing localized service are part of the same PLMN network, including:

-	How to minimize the impacts towards PLMN when performing NF discovery and selection, message forwarding and routing between PLMN network and network providing localized services, or between networks providing localized service.

- 	Whether and how to support UE mobility between PLMN network and network providing localized services, or between networks providing localized service.

- 	Whether and how to support service continuity for localized service, including access to localized service after leaving the original network providing localized service.

Editor's Note: New KIs, derived from other WTs, such as WT#3, 4, 5, 6, are to be added.

# * * * * End of changes * * * *