3GPP TSG-WG SA2 Meeting #171	S2-2509032

Wuhan, CN, 13th Oct – 17th Oct, 2025	(revision of S2-250xxxx)

Source:	Huawei, HiSilicon

Title:	Discussion on UE serving satellite identifier report for UE-Satellite-UE communication

Document for:	Discussion

Agenda Item:	19.1.2

Work Item / Release:	5GSAT_Ph3-ARC / Rel-19

Abstract: Overview of the options for satellite identifier reporting and proposal to resolve this issue.

# 1. Introduction

Based on current call setup procedure of UE-satellite-UE communication in IMS described in TS 23.228, after receiving an INVITE message from the originating side P-CSCF, the terminating side P-CSCF requests the Access Network Information from 5GC to obtain UE’s serving satellite identifier, however the UE may be in IDLE state, i.e., the IMS PDU Session may be deactivated. In this case, the serving satellite identifier cannot be obtained.

During last meeting online and offline discussions, two different directions were proposed to solve this issue and need further analysis.

# 2. Discussion

## 2.1. Network-Triggered Service Request Procedure enhancement

In last meeting, we proposed to enhance the SMF to trigger a Service Request procedure if it receives a request from the PCF for an Access Network Information Report with the Satellite Identifier Report parameter in the request:

Based on the Satellite Identifier Report parameter in the request from PCF for Access Network Information reporting, if the IMS PDU Session is deactivated, the SMF activates the IMS PDU Session by invoking the Network Triggered Service Request procedure described in clause 4.2.3.3 of TS 23.502. During the Service Request procedure, the AMF reports the serving satellite identifier of UE to the SMF.

The Network Triggered Service Request procedure described in clause 4.2.3.3 of TS 23.502 needs an update to reflect the impacts on SMF.

Observation 1: In this way, the terminating side P-CSCF will wait for a satellite ID report from the PCF and send a SIP INVITE message to UE via the UPF after determination whether to perform UE-satellite-UE communication and selection of IMS-AGW on satellite or ground.

Observation 2: Compared with a general IMS call, if the terminating side UE is in IDLE state, the SIP message is not used to trigger Network Triggered Service Request procedure, instead a control plane request for satellite identifier report is used to trigger Network Triggered Service Request procedure.

## 2.2. Send the SIP INVITE message to terminating side UE first

Another way was proposed during offline discussions at the last meeting is to send a SIP INVITE Message to terminating side UE first. This means the terminating side P-CSCF needs to select a ground IMS-AGW by default and determine whether to reselect the IMS-AGW on satellite after determination of whether to perform UE-satellite-UE communication. The procedure can be deduced as follows:

After receiving the SIP INVITE message, the terminating side P-CSCF selects a terrestrial IMS-AGW and sends the SIP INVITE message carrying the assigned IMS-AGW address to the UE. If the UE is in an IDLE state, the UPF triggers Network-Triggered Service Request Procedure described in clause 4.2.3.3 of TS 23.502. During this procedure, the UE's serving satellite identifier is reported to the SMF by the AMF. The SMF reports the satellite identifier to the PCF and the PCF reports it to terminating side P-CSCF. Based on this information, the P-CSCF may re-select an onboard IMS-AGW and sends a SIP re-INVITE message to the UE.

When the P-CSCF receives the 18X Response from the UE, the P-CSCF may wait the satellite identifier report from the PCF to check whether to change the IMS-AGW. If the IMS-AGW is reselected to enable UE-satellite-UE communication, the P-CSCF needs to send a SIP RE-INVITE message to the UE to update the IMS-AGW, this will cause the terminating side P-CSCF to function as a B2B UA, however this was excluded during the study phase.

Observation 3: To move the determination of whether to perform UE-satellite-UE communication after sending SIP INVITE message to UE, the terminating side P-CSCF needs to function as a B2B UA.

Observation 4: Although the terminating side P-CSCF does not need to hold SIP INVITE message, it still needs to wait for the satellite identifier report before handing the 18X Response from the UE and this may cause extra IMS-AGW relocation related steps.

# 3. Conclusion and proposal(s)

Based on observations above for the two directions, currently we see two different options to solve the issues.

Option 1: Network-Triggered Service Request Procedure enhancement.

CRs for TS 23.501 and 23.501 502 CRs for Option 1 are provided in S2-2509033 and S2-2509034.

Option 2: Send the SIP INVITE message to terminating side UE first.

CR for TS 23.228 CR for Option 2 is provided in S2-2509035.

Proposal: Option 1 is preferred to solve the issue, as the only impact on the SMF is to support to activate the IMS PDU session if the IMS PDU session is not activated, and the PCF requests to report satellite identifier via Access Network Information report request.