3GPP TSG-WG SA2 Meeting #170	S2-2509061

Wuhan, CN, 13th Oct – 17th Oct, 2025	(revision of S2-2507251)

Source:	Huawei, HiSilicon

Title:	KI1 New Sol: Topology 2 RRC-based option baseline architecture

Document for:	Approval

Agenda Item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Abstract: Rel-19 provided interim conclusions for topology 2 support for RRC option for T2.

1. Introduction/Discussion

This solution provides the conclusion from the Rel-19 TR 23.700-13 for topology 2 based on the RRC option into the Rel-20 TR for the indirect connectivity option only.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-30.

# * * * * First change (all new) * * * *

6.X	Solution #X: Topology 2 RRC-based option baseline architecture

6.X.0	High-level solution Principles

The Rel-19 Ambient IoT study (see TR 23.700-13 [x]) provided conclusions for 2 Topology 2 architectures, while the KI for topology 2 focuses only on the RRC-based option. This is the Rel-19 RRC-option baseline solution based on the Rel-19 conclusions.

The Rel-19 conclusion provided subscription aspects, radio resource management aspects and how requests/responses are transferred between the AIOTF and UE Reader.

6.X.1	Description

Subscription Aspects:

-	The UE subscription in the UDM will be extended with UE Reader subscription information, which consists of the following:

-	information indicating whether the UE is allowed to operate as a UE Reader.

-	UE Reader subscription information is available to AMF and AIOTF.

-	If AMF receives, as part of the subscription information, the indication that the UE is authorized to operate as a UE Reader, then the AMF informs NG-RAN that the UE is authorized to operate as a UE Reader.

Radio resource management for UE Reader operation:

-	If the gNB has received the indication that a UE is authorized to operate as a UE Reader, then the gNB may assign radio resources to the UE for UE Reader operation.

-	The UE Reader is assumed to request radio resources for Reader operation (e.g., taking into account assistance information received from the AIOTF) from the gNB.

NOTE 1:	Assistance information is specified in TS 23.369 [x].

NOTE 2:	The details of UE Reader radio resource management will be defined by RAN WG2.

In the indirect connectivity architecture messages between the UE Reader and the AIOTF are delivered using RRC between UE and RRC and NGAP between gNB and AMF, and using an SBI interface between AMF and AIOTF. The related protocol stack is shown in Figure 6.x.1-1.

Figure 6.x.1-1: Protocol Stack for the indirect connectivity RRC option

NOTE x:	The UE requires a serving AMF, and the AMF used by the AIOTF does not need to be the same AMF.

6.X.2	Procedures

6.x.2.1	Subscription Aspects

6.x.2.1.1	General

In order to support AIOT radio resource control in NG-RAN, AIoT UE Reader Authorisation information needs to be made available in NG-RAN for the UE.

6.x.2.1.2	UE Registration procedure

The Registration procedure for UE is performed as defined in TS 23.502 [5] clause 4.2.2.2 with the following additions:

-	The UE includes the AIoT UE Reader Capability as part of the "5GMM capability" in the Registration Request message. The AMF stores the AIoT UE Reader Capability operation.

-	The AIoT UE Reader Capability indicates whether the UE supports performing Ambient IoT service operations.

-	The AMF obtains the UE subscription data as part of the user subscription data from UDM during UE Registration procedure using Nudm_SDM service as defined in clause 4.2.2.2.2 of TS 23.502 [5].

-	The AMF determines whether the UE is authorised to provide AIoT UE Reader operations based on UE's AIoT UE Reader Capability and the AIoT Service Authorisation included in the subscription data received from UDM

-	If the UE is authorised to provide AIoT UE Reader operations, then the AMF shall include in a NGAP message sent to NG-RAN:

-	"AIoT UE Reader Authorisation" information

-	If the UE is authorised to provide AIoT UE Reader operations, then the AMF should not initiate the release of the signalling connection after the completion of the Registration procedure. The release of the signalling connection relies on the decision of NG-RAN, as specified in TS 23.502 [5].

6.x.2.1.3	Service Request procedure

The Service Request procedures for UE in CM-IDLE state are performed as defined in TS 23.502 [5] clause 4.2.3.2 and clause 4.2.3.3 with the following additions:

-	If the UE is authorised to provide AIoT UE Reader operations, then the AMF shall include "AIoT UE Reader Authorisation" information in the NGAP message, indicating the UE is authorised to provide AIoT UE Reader operations.

6.x.2.1.4	N2 Handover procedure

The N2 based handover procedures for UE are performed as defined in TS 23.502 [5] clause 4.9.1.3 with the following additions:

-	If the UE is authorised to provide AIoT UE Reader operations, then the target AMF shall send the " AIoT UE Reader Authorisation" information, to the target NG-RAN in the NGAP Handover Request message.

6.x.2.1.5	Xn Handover procedure

The Xn based handover procedures for UE are performed as defined in TS 23.502 [5] clause 4.9.1.2 with the following additions:

-	If the " AIoT UE Reader Authorisation" information is included in the UE context, then the source NG-RAN shall include a "AIoT UE Reader operations authorised" information, in the XnAP Handover Request message to the target NG-RAN.

-	If the " AIoT UE Reader Authorisation" information is included in the UE context, then the AMF shall send the "AIoT UE Reader Authorisation" information to the target NG-RAN in the Path Switch Request Acknowledge message.

6.x.2.2	Procedures between AIOTF and UE using indirect connectivity architecture

An AIOTF and UE use an indirect interface via an AMF and NG-RAN as described in clause 6.x.1 and using the procedure defined in TS 23.369 [3] for Procedures between AIOTF and NG-RAN for Indirect Connectivity.

The additional steps used between AIOTF and UE are shown in Figure 6.x.2.2-1 for message delivery to the UE.

Figure 6.x.2.2-1: Procedure for Message Delivery from AIOTF to UE via NG-RAN Indirect Connectivity

The procedure is based on the Message delivery to NG-RAN procedure defined in TS 23.369 [3].

1.	The AIOTF sends Namf_AIoT_MessageDelivery Request message (NGAP AIoT information, NG-RAN ID, AIOTF ID, Correlation ID, AIoT NGAP Message Type, Notification endpoint, [RAN AIoT Device NGAP ID]) to the AMF.

The AIoT NGAP Message Type identifies the NGAP message to send to NG-RAN and NGAP AIoT information is provided to NG-RAN in the NGAP message.

If the AMF receives a Notification endpoint, the AMF creates a context for the transaction and stores the NG-RAN ID, AIOTF ID, Correlation ID and Notification endpoint. The AIOTF is implicitly subscribed to Namf_AIoT_Notify events.

NOTE x:	How the AIOTF, AMF and/or NG-RAN determine which UE to use and how to ensure that the UE is in CM_CONNECTED will be defined by other solutions.

2.	The AMF responds to the AIOTF with a result indicating whether the AMF will handle the request. If a failure is indicated by the result indication, then the procedure stops and the remaining steps are skipped.

3.	The AMF sends an NGAP message (AIOTF ID, Correlation ID, NGAP AIoT information, [RAN AIoT Device NGAP ID]) to the target NG-RAN.

3x.	[new] NG-RAN verifies that that the UE has "AIoT UE Reader operations authorised".

The NG-RAN sends messages to the UE and receives messages from the UE using RRC, including information from NGAP AIoT information which provides the operation and its associated parameters for the AIoT operation for the UE to perform.

NOTE y:	The details of the RRC messages, sequences, radio resource allocation for AIoT will be defined by RAN WG2.

4.	The procedure for handling NGAP messages from UE Reader is used for route NGAP messages from NG-RAN to the AIOTF, see clause 6.2.4.3.

The procedure is based on the Message delivery to AIOTF procedure defined in TS 23.369 [3], with the difference the UE provides the information for the NG-RAN to construct the NGAP AIoT information to send to the AIOTF.

Figure 6.x.2.2-2: Procedure for UE Message handling using indirect connectivity via an AMF

1.	Before any message can be routed from the UE Reader to the AIOTF the procedure in Figure 6.x.2.2-1 needs to be executed.

1a	[new] The NG-RAN receives messages from the UE using RRC, including information from NGAP AIoT information which provides the results of the operation the UE has performed.

2.	NG-RAN sends an NGAP message (AIOTF ID, Correlation ID, NGAP AIoT information, [RAN AIoT Device NGAP ID]) to an AMF. NGAP AIoT information is determine by NG-RAN depending upon the operation the NGAP message relates to, and constructs the NGAP AIoT information to send to the AIOTF.

3.	AMF determines the Notification endpoint using the AIOTF ID and Correlation ID received in step 1 from NG-RAN, and then sends the Namf_AIoT_Notify message (AIoT NGAP Message Type, NGAP AIoT information) to the AIOTF.

Step 2 and step 3 are repeated for each NGAP message received from NG-RAN.

If the message received from NG-RAN is an Inventory Failure indicating an error or an AIOT Session Release Complete, then the AMF releases the context which is associated with the AIOTF ID and Correlation ID, after sending the Namf_AIoT_Notify message to the AIOTF.

6.X.3	Impacts on Services, Entities and Interfaces

UE:

-	Needs to support AIOT UE Reader operations.

-	Exchanges capability to operate as a UE reader with the AMF during Registration procedures

NG-RAN:

-	Receive whether the UE is authorized for AIoT UE Reader operations from the AMF, or via Nx and N2 handover procedures.

-	Receive AIoT Information and instruct the UE over RRC to perform AIoT operations and obtain results from the UE.

AMF:

-	Support capability exchange for the UE supporting AIoT UE Reader operations

-	Determine whether the UE is authorized to be a UE Reader based on subscription data.

-	Send whether the UE is authorized for AIoT UE Reader operations to the NG-RAN.

AIOTF:

- Updated Reader selection, taking into account UE Readers.

NOTE: The updated procedures for reader selection will be defined by other solutions.

UDM:

-	Updated UE subscription data for whether the UE is authorized for AIoT UE Reader operations.

# * * * * End of changes * * * *