3GPP TSG-WG SA2 Meeting #171	S2-2509099

Wuhan, China, Oct 13 - 17, 2025                          (revision of S2-250xxxx)

Source:	Samsung

Title:	Proposal on Interim agreements for KI#2

Document for:	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC/Rel-20

Abstract of the contribution: This paper proposes Interim agreements for KI#2 for the FS_Sensing_ARC.

# 1	Discussion

There are 39 solutions present in the TR 23.700-14 V1.0.0, out of which 14 solutions address KI#2.

Solution #2, #3, #6, #8, #9, #10, #11, #12, #19, #27, #33, #36, #37 and #38 is addressing KI#2 based on TR 23.700-14 V1.0.0.

Key functionalities are summarized as below:

- Authorization of Application Function as sensing service consumer (AF)

NEF does the authorization of AF as sensing service consumer

#3, #9, #11, #12, #27, #37

- Authorization of sensing service request received from Application Function as sensing service consumer (AF)

Sensing Function or a new function other than sensing function does the authorization of sensing service request based on the authorization data/information

#3, #6, #8, #9, #10, #11, #12, #19, #27, #33, #37

- Authorization data/information content

Allowed Area for sensing service operation, Allowed time to trigger sensing service operation, Allowed sensing service type. This authorization information is applicable per AF.

#3, #6, #9, #10, #11, #12, #19, #27, #36, #37

- Authorization data/information update

The content of Authorization information which is stored at SF and new separate function other than SF can be updated via OAM, local configuration or by AF via NEF

#9, #38

- Authorization of gNB as sensing entity

Based on the capability of sensing entity like Tx, Rx or both

#8

- Revocation of sensing service request

Explicit request by AF, change in Authorization information, Network detects to revoke

#8, #9, #11, #12, #37

- Revocation of gNB as sensing entity

The architecture is designed to handle data collection from multiple Sensing Entities (e.g., multiple gNBs) and to distribute sensing data to one or multiple processing functions (e.g. SePFs), enabling complex data fusion and processing chains. See Solution #23 / #24 / #26.

- Authorization of UE as sensing service consumer and sensing service request from UE

#2, #19, #33

- Revocation of UE as sensing service consumer and sensing service request

#2

Observation 1: The items highlighted in the green color has been supported by multiple companies and aligned with the architecture proposed in KI#1

Proposal 1: Document all the green highlighted as agreed principles in clause 7.1 and keep the yellow highlighted items in clause 7.2 for further discussion

# 2	Proposal

It is proposed to include the following changes in TR 23.700-14 V0.2.0.

* * * * Start of Changes * * * *

# 7	Interim agreements

## 7.1	Agreed Principles

### 7.1.Y	Agreed Principles for KI#Y

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.Y gets resolved and a principle is agreed.

Based on the explanation in Discussion clause the following principles are proposed for interim agreement

-	Sensing Consumer Authorization:

-	If untrusted AF is the sensing service consumer, the NEF performs authorization for accepting the sensing service request. If trusted AF is the sensing service consumer, the request can be directly sent to sensing function by passing authorization check.

-	Sensing Service Request Authorization:

-	A newly introduced function named as Sensing Function or a new function different from sensing function, named Sensing Profile Function performs the authorization of sensing service request

-	Sensing service request authorization is based on the Authorization data/information present which is applicable per sensing service consumer. This Authorization data/information is stored in this new network function

- 	The Authorization data/information can have information like allowed sensing service area, allowed time, allowed sensing service type etc.

- 	This Authorization data/information can be created/updated via OAM, via NEF by AF or through local configuration.

-	Sensing Service Revocation (i.e. ongoing sensing service):

Based on some changes in the sensing service requirement, the revocation can be triggered for the ongoing sensing service.

-	Sensing service consumer may explicitly send request to revoke the ongoing sensing service via NEF or directly the sensing service

- 	Sensing service function itself can identify some reason to revoke the ongoing sensing service operation (e.g. the sensing service is completed)

-	Sensing service function receives some change in Authorization data/information which indicates the ongoing service need to be revoked (e.g. the Allowed area for the AF is changed and the current area for which sensing is going on is not authorized for the AF to get sensing service)

## 7.2	Topics for further consideration

### 7.2.Z	Topics for further consideration for KI#Z

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#Z. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in a clause 7.1.Z) and can either be then marked as not pursued or postponed to a future Release.

Editor's note:	The UE as a sensing service consumer is authorized to trigger sensing service and avail it from network.

Editor's note:	The gNB as a sensing entity should be authorized to do sensing or can be simply invoked similar any NF discovery based on the capability to provide sensing service.

* * * * End of Changes * * * *