3GPP TSG-WG SA2 Meeting #171	S2-2509100

Wuhan, China, Oct 13 - 17, 2025                          (revision of S2-250xxxx)

Source:	Samsung

Title:	Proposal on Interim agreements for KI#6

Document for:	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC/Rel-20

Abstract of the contribution: This paper proposes Interim agreements for KI#6 for the FS_Sensing_ARC.

# 1	Discussion

There are 39 solutions present in the TR 23.700-14 V1.0.0, out of which 14 solutions address KI#6.

Solution #3, #15, #20, #27, #34 and #35 is addressing KI#6 based on TR 23.700-14 V1.0.0.

Key functionalities are summarized as below:

- The configuration parameters are received from Sensing service consumer or sensing operation server

Sensing Function receives the configuration parameter from AF, NEF, some server, some parameter is additionally generated by SF itself

#3, #15, #20, #27, #34, #35

- The configuration parameters are sent directly to Sensing Entity

Sensing Function sends the configuration parameters to sensing entity directly.

#3, #15, #20, #27, #34, #35

- Configuration parameters are sent via AMF to Sensing Entity

Sensing Function sent the configuration parameters via AMF through sensing service container

#35

- The content of configuration parameters

Sensing service type, target sensing area, sensing duration, sensing KPI/QoS, Sensing object type, RCS (Radar Cross Section) range list, object velocity range

#27, #34, #35

Observation 1: Most of the solutions are in principle proposing to SF to configure the parameters to sensing entity with the exception that sol 35 suggest to have another option of sending via AMF as well

Proposal 1: Document all the green highlighted as agreed principles in clause 7.1 and keep the yellow highlighted item in clause 7.2 for further discussion

# 2	Proposal

It is proposed to include the following changes in TR 23.700-14.

* * * * Start of Changes * * * *

# 7	Interim agreements

## 7.1	Agreed Principles

### 7.1.Y	Agreed Principles for KI#Y

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.Y gets resolved and a principle is agreed.

Based on the explanation in Discussion clause the following principles are proposed for interim agreement

-	Configuration of Parameters for sensing entities:

-	Configuration parameters for sensing service can be sent by AF or any other provisioning server and received by sensing function.

-	Sensing function will derive parameters received from AF or provisioning server. Also, service function can generate some parameters applicable for sensing service.

-	Sensing function will send directly the parameters to sensing entity to configure for the sensing service.

-	Sensing parameters include Sensing service type, target sensing area, sensing duration, sensing KPI/QoS, Sensing object type, RCS (Radar Cross Section) range list, object velocity range.

## 7.2	Topics for further consideration

### 7.2.Z	Topics for further consideration for KI#Z

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#Z. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in a clause 7.1.Z) and can either be then marked as not pursued or postponed to a future Release.

Editor's note:	The parameters send to sensing entity (i.e. gNB) needs co-ordination with RAN3.

Editor's note:	Sensing function sending parameters via AMF to sensing entity depends upon the out come of KI#1 architecture principles.

* * * * End of Changes * * * *