SA WG2 Meeting #171	S2-2509102

13 – 17 October 2025, Wuhan, China

Source:	Samsung

Title:	KI#3: Solution for Dynamic MDC2 endpoint configuration for IMS Data Channel

Document for:	Approval

Agenda Item:	20.7.1

Work Item / Release:	FS_NG_RTC_Ph3_ARC / Rel-20

Abstract of the contribution: This contribution proposes a new solution for KI#3.

# 1. Discussion

This paper proposes solution for KI#3 in TR 23.700-56 for the SID NG_RTC_Ph3_ARC, how the dynamic configuration of MDC2 endpoints can be provided to for IMS Data Channel in the case of UE initiated P2A and P2A2P ADC.

# 2. Text Proposal

It is proposed to capture the following changes into TR 23.700-56.

# * * * * First change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | KI#1 | KI#2 | KI#3 |
| #1 | X |  |  |
| #2 | X |  |  |
| #3 | X |  |  |
| #4 | X |  |  |
| #X |  |  | X |

## 6.X	Solution #X: Dynamic MDC2 endpoint configuration IMS Data Channel

### 6.X.0	High-level solution Principles

This solution addresses Key Issue #3: Supporting dynamic configuration of MDC2 endpoints.

The main principle of this solution for support of dynamic MDC2 endpoint configuration for IMS Data Channel during UE initiated P2A and P2A2P session as follows

-	The DC AS shall discover and select the DCSF handling the UE for P2A or P2A2P session

-	To discover DCSF address, DC AS subscribe with HSS and HSS notifies once DCSF register its address in HSS.

-	Similar to third party registration done by S-CSCF to IMS AS, IMS AS does third party registration with DCSF and then DCSF register its address in HSS.

-	Upon successful discovery and selection, DCSF subscribes for the establishment of P2A or P2A2P events with DCSF and get notified when DCSF detects the events.

-	After receiving notification, If DC AS accepts P2A application data channel establishment request , DC AS sends an MDC2 reserved media resource in the newly proposed Ndcsf_MediaControl service operation message .

- 	DCSF after receiving MDC2 endpoint information from DC AS, invoke IMS AS to provide the MDC2 endpoint address of DC AS and request for MDC2 endpoint from MF side.

- 	DCSF provides the MF side MDC2 endpoint address to DC AS in response to Ndcsf_MediaControl service operation.

### 6.X.1	Description

This solution proposes how the DC AS discovers the DCSF and subscribes to get notification for the UE initiated P2A and P2A2P session and then shares the MDC2 endpoint addresses. This will overcome the existing gaps of the P2A and P2A2P session which does not provide any flexibility in dynamically exchanging MDC2 endpoint addresses between DC AS and MF.

### Discovery of DCSF by DC AS:

The trusted DC AS can directly discover the DCSF with the help of HSS and in case untrusted DC AS, the discovery happens through NEF. For this purpose, HSS can expose one service operation using which DC AS/NEF will subscribe and get notification about the DCSF address for the UE.

When UE registers with IMS network, S-CSCF does third party registration with IMS AS and then IMS AS registers its address with HSS. Similarly, this solution proposed IMS AS does third party registration with DCSF and then DCSF registers its address with HSS. Once HSS comes to know the DCSF address, it notifies the NEF/ DC AS.

### Subscription with DCSF by DC AS:

Upon successful discovery, DC AS shall subscribe with DCSF for the UE initiated P2A or P2A2P session establishment events.

### Providing MDC2 endpoint address by DC AS:

Upon notified by DCSF about establishment of P2A or P2A2P call, DC AS provides its MDC2 endpoint address with DCSF.

Table 6.X.1-1 List of events supported by DCSF

| Event | Description and Detection criteria | Which NF detects the event |
| --- | --- | --- |
| P2A pr P2A2P Application Data Channel established | Detected when an Application Data Channel is established. Network detects that an Application data channel offer takes DC Application Server as target endpoint and requires to anchor in the MF | DCSF |

### 6.X.2	Service definition

#### 6.X.2.1	General

The following table shows the required service operation.

#### 6.X.2.1.1	NEF Service

The NEF services for IMS Event exposure is already defined in TS 23.502. Only the enhancement is shown below.

Table 5.2.6.1-1: NF Services provided by the NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_ImsEventExposure | Subscribe | Subscribe/Notify | AF, DC AS |
|  | Unsubscribe | Subscribe/Notify | AF, DC AS |
|  | Notify | Subscribe/Notify | AF, DC AS |

##### 5.2.6.39.2	Nnef_ImsEventExposure (ImsEE) service

5.2.6.39.2.1	Nnef_ImsEE_Subscribe service operation

Service operation name: Nnef_ImsEE_Subscribe

Description: The NF consumer (for e.g. AF or DC AS) subscribes for a specific IMS event.

Inputs, Required: Event-ID(s) representing the event of interest, Event Reporting Information as defined in Table 4.15.1-1.

Inputs, Optional: List one or more IMS Public Subscriber identity, Event Filters, Notification Target address, Notification Correlation Information, Expiry time, Subscription Correlation ID (in the case of modification of the event subscription), AF Identifier.

IMS Public subscriber identity entry is only present for subscriber specific IMS events.

NOTE 1:	The semantics of the Event ID and Event Filters are transparent to the NEF.

Outputs, Required: When the subscription is accepted: Subscription Correlation ID, Expiry time (required if the subscription can expire based on the operator's policy).

Outputs, Optional: If applicable to the event type, the current status related to the subscribed event type is included, if Immediate Reporting Flag is included in the request within Event Reporting Information and if available (see clause 4.15.1).

NOTE 2:	If the current status related to the subscribed event type is available and included in the response, it is not considered as an event report (i.e. it is not considered a one-time notification on event detection). That is, it is not deducted from the maximum number of reports if included in the request within the Event Reporting Information.

#### 6.X.2.1.2	HSS Service

### AA.2.1.1	General

The following table shows the services exposed by an SBI capable HSS.

Table AA.2.1.1-1: IMS Services provided by an SBI capable HSS

| Service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| ImsSubscriber Data | Get | Request/Response | S-CSCF, I-CSCF, AS, DCSF |
| Management (_ImsSDM) | Subscribe | Subscribe/Notify | S-CSCF, AS |
|  | Unsubscribe | Subscribe/Notify | S-CSCF, AS |
|  | Notification | Subscribe/Notify | S-CSCF, AS |
|  | Update | Request/Response | AS, DCSF |
| Ims UE Context | Registration | Request/Response | S-CSCF, IMS AS, DCSF |
| Management (_ImsUECM) | DeregistrationNotification | Subscribe/Notify | S-CSCF |
|  | Deregistration | Request/Response | S-CSCF, IMS AS, DCSF |
|  | Authorize | Request/Response | I-CSCF |
|  | Update | Request/Response | S-CSCF |
|  | RestorationInfoGet | Request/Response | S-CSCF |
|  | RestorationInfoUpdate | Request/Response | S-CSCF |
|  | AsInfoGet | Request/Response | NEF |
|  | DcsfInfo | Subscribe/Notify | NEF, DC AS |
|  | DcsfInfoUnsubscribe | Subscribe/Notify | NEF, DC AS |
|  | DcsfInfoNotify | Subscribe/Notify | NEF, DC AS |
| ImsUE Authentication (_ImsUEAU) | Get | Request/Response | S-CSCF |
| ImsEventExposure | Subscribe | Subscribe/Notify | NEF, Trusted AF |
| (_ImsEE) | Unsubscribe |  | NEF, Trusted AF |

#### AA.2.1.2.1	Nhss_ImsUECM_Registration service operation

Service operation name: Nhss_ImsUECM_Registration

Description: If S-CSCF is the consumer, this service operation registers the serving S-CSCF assigned to an IMS User. If authentication is not to be performed, this operation also sets the registration state. The S-CSCF is implicitly subscribed to be notified when it is deregistered in HSS. This notification is done by means of Nhss_ImsUECM_DeregistrationNotification operation.

If IMS AS is the consumer, this service operation registers the IMS AS instance assigned to an IMS user.

If the DCSF is the consumer, this service operation registers the DCSF instance assigned to an IMS user.

Inputs, Required: Public Identity, S-CSCF name and Registration Type (e.g. Initial Registration, Unregistered) if the S-CSCSF is the consumer, IMS AS instance Id if the IMS AS is the consumer, DCSF instance Id is the DCSF is the consumer.

Inputs, Optional: Private Identity.

Outputs, Required: Result indication.

Outputs, Optional: List of registered Private Identities sharing the same Public Identity which is being registered, S-CSCF Restoration indication.

##### 6.X.2.1.2.1	Nhss_ImsUECM_DcsfInfoGet service operation

Service operation name: Nhss_ImsUECM_DcsfInfoGet

Description: Service consumer like NEF or DC AS subscribes with HSS to receive the DCSF address assigned for the UE. The request creates an implicit subscription to get notified about the following events, when applicable, (1) the UAS NF/NEF

Inputs, Required: Public Identity.

Inputs, Optional: Private Identity.

Outputs, Required: Result.

Outputs, Optional: None.

##### 6.X.2.1.2.2	Nhss_ImsUECM_DcsfUnsubscribe

Service operation name: Nhss_ImsUECM_DcsfUnsubscribe

Description: This service operation enables consumers to unsubscribe from the event.

Inputs, Required: Subscription correlation ID.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

##### 6.X.2.1.2.3	Nhss_ImsUECM_DcsfNotify

Service operation name: Nhss_ImsUECM_DcsfNotify

Description: This service operation enables the HSS to notify the consumer with the updated DCSF address where there is a change of DCSF happens for the UE.

Inputs, Required: DCSF address.

Inputs, Optional: IMPU, IMPI.

Outputs, Required: Result.

Outputs, Optional: None.

#### 6.X.2.1.3	DCSF Service

The following table shows the DCSF Services and DCSF Service Operations.

Table AA.2.4.1-1: NF services provided by the DCSF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Ndcsf_ADCControl_MDC2 | Create | Request/Response | NEF, DC AS |
|  | Update | Request/Response | NEF, DC AS |
|  | Delete | Request/Response | NEF, DC AS |

#### 6.X.2.1.3.1	Ndcsf_ADCControl_MDC2 service operation

Service operation name: Ndcsf_ADCControl_MDC2

##### 6.X.2.1.3.2	General

Service description: This service enables the consumer to create, update or terminate an ADC.

##### 6.X.2.1.3.3	Ndcsf_ADCControl_MDC2Create

Service operation name: Ndcsf_SessionEvent_Create

Description: This service operation enables the consumer to create information used by the DCSF for ADC establishment in an IMS session.

Inputs, Required: Session ID, DC AS-side MDC2 endpoint information (e.g. IP address, port number), certificate fingerprint of DC AS for end-to-end security between UE and DC AS in case MDC2 is in "UDP/DTLS/SCTP", "TCP/TLS" or "SCTP/DTLS" mode.

Inputs, Optional: securitySetup if DC AS initiates the DTLS negotiation.

Outputs, Required: Result.

Outputs, Optional: None.

##### 6.X.2.1.3.4	Ndcsf_ADCControl_MDC2Update

Service operation name: Ndcsf_ADCControl_Update

Description: This service operation enables the consumer to update information used by the DCSF for ADC updating in an IMS session.

Inputs, Required: Session ID.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

##### 6.X.2.1.3.5	Ndcsf_ADCControl_MDC2Delete

Service operation name: Ndcsf_ADCControl_Delete

Description: This service operation enables the consumer to provide information used by the DCSF for ADC termination in an IMS session

Inputs, Required: Session ID.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

#### 6.X.2.2	Ndcsf_SessionEvent

##### 6.X.2.2.1	General

Service description: This service enables the consumer to subscribe with the supported events by DCSF and get notified when the event is detected.

The following operations are available for this service:

-	Subscribing and unsubscribing with events supported by DCSF.

-	Notifying after event is detected to consumers, e.g. NEF, AF.

##### 6.X.2.2.2	Ndcsf_SessionEvent_Subscribe

Service operation name: Ndcsf_SessionEvent_Subscribe

Description: This service operation enables consumers to subscribe to the event of P2A or P2A2P ADC session establishment.

Inputs, Required: Notification Target address, Event ID

Only one Event Id is currently defined: establishment of a MDC2 channel is needed

Inputs, Optional: Target list of User IDs (IMPUs), list of Application binding information

When no IMPU is given, the subscription is assumed to be non-UE specific and thus to apply to Any UE (otherwise an explicit Any UE target indication may be defined).

Outputs, Required: Result, Subscription correlation ID when the subscription is accepted.

Outputs, Optional: None.

##### 6.X.2.2.3	Ndcsf_SessionEvent_Unsubscribe

Service operation name: Ndcsf_SessionEvent_Unsubscribe

Description: This service operation enables consumers to unsubscribe from the event of P2A or P2A2P ADC session establishment.

Inputs, Required: Subscription correlation ID.

Inputs, Optional: None.

Outputs, Required: Result.

Outputs, Optional: None.

##### 6.X.2.2.4	Ndcsf_SessionEvent_Notify

Service operation name: Ndcsf_SessionEvent_Notify

Description: This service operation enables the DCSF to notify the NF subscriber about the subscribed event.

Inputs, Required: Session ID, MF-side MDC2 endpoint information (e.g. IP address, port number), Resource URL(for any Ndcsf_ADCControl operation that would correspond to the Notification), DC application ID, certificate fingerprint of UE for end-to-end security between UE and DC AS in case MDC2 is in "UDP/DTLS/SCTP", "TCP/TLS" or "SCTP/DTLS" mode.

Inputs, Optional: Calling party and called party IDs, application ID, securitySetup if UE initiates the DTLS negotiation.

Outputs, Required: Result.

### 6.X.3	Procedure

#### 6.X.3.1	DCSF instance registration in HSS

Figure 6.X.3.1-1depicts a signalling flow diagram for an DCSF registering in HSS the DCSF instance assigned to a registering UE.

Figure 6.X.3.1-1: DCSF Instance Registration in HSS

1.	UE performs initial IMS Registration (see clause 5.2).

2.	S-CSCF performs third party Registration with the IMS AS instance assigned to the registering UE.

3.	IMS performs third party Registration with the DCSF instance assigned to the registering UE

4.	DCSF assigned to the UE/IMPU registers in HSS using Nhss_ImsUECM_Registration service operation or using Sh interface.

NOTE 1:	Figure 6.X.3.1-1 shows only SBA services. Corresponding Sh procedure is not included in the Figure.

#### 6.X.3.2	DCSF address discovery by DC AS

Figure 6.X.3.2-1depicts a signalling flow diagram for discovering DCSF address by DC AS.

.

Figure 6.X.3.2-1: DCSF address discovery by DC AS

1.	DCSF address is already registered as per Fig1 solution.

2.	DCAS discovers DCSF by sending Nnef_ImsUECM_DcscfInfoGet service operation to NEF.

3.	NEF discovers DCSF by sending Nhss_ImsUECM_DcscfInfoGet service operation to HSS

4.	HSS provides DCSF address in Nhss_ImsUECM_DcscfInfoGet response service operation to NEF.

5. 	NEF provides DCSF address in Nnef_ImsUECM_DcscfInfoGet response service operation to DC AS.

#### 6.X.3.2	Person-to-Application (P2A) Application Data Channel Setup

Figure 6.X.3.2-1 depicts a signalling flow diagram for establishing an Application Data Channel in a person to application use case.

Figure 6.X.3.2-1: P2A ADC establishment with a trusted DC AS.

1.	The DC AS subscribes to the session event notification at the DCSF via Ndcsf_ImsEventExposure_Subscribe service

2.	The DC AS The DCSF responds the request to the DC AS

3.	IMS session and BDC have been established

4-11.	UE#1 requests the establishment of P2A ADC establishment. DCSF determines the DC control policy and indicates the IMS AS to request MF for reserving the originating media resources.

12.	DCSF notifies DC AS about the session establishment via Ndcsf_ImsEventExposure_Notify message. The message includes calling and called parting id, application id, session id, MF-side MDC2 media resource info, Resource URL used by DC AS to send subsequent ADC Control message.

13.	DC AS responds to the Ndcsf_ImsEventExposure_Notify message.

14.	DCSF waits for ADC control instruction (Ndcsf_ADCControl_Create). If such instructions are not provided then DCSF has the ADC establishment procedure aborted (Nimsas_MediaControl_MediaInstruction).

15.	DC AS allocates DC AS-side MDC2 media resource for this session. DC AS sends Ndcsf_ADCControl_Create message with the instruction for session control and the allocated DC AS-side MDC2 media resource information. The message includes session id, DC AS-side MDC2 media resource info.

16-18. DCSF updates the media resources in the MF via IMS AS, based on the received the DC AS-side MDC2 media resources information. Then, the IMS AS sends response to DCSF.

19. The DCSF sends response of Ndcsf_ADCControl_Create to DC AS.

20-32. IMS AS performs SDP negotiation with terminating network. With a successful SDP negotiation, the application data channel established.

#### 6.X.3.3	Person-to-Application and Application-to-Person (P2A2P) Procedure

Figure 6.X.3.3-1 depicts a signalling flow diagram for establishing an Application Data Channel in a person to application use case.

Figure 6.X.3.3-1: P2A2P ADC establishment with a trusted DC AS.

1.	IMS session and BDC has been established. The DC AS subscribes to the session event notification at the DCSF via Ndcsf_ImsEventExposure_Subscribe service

2.	The DCSF responds the request to the DC AS.

3.	IMS session and BDC has been established

4-11.	UE#1 requests the establishment of P2A2P ADC establishment. DCSF determines the DC control policy and indicates the IMS AS to request MF for reserving the originating and terminating media resources.

12.	DCSF notifies DC AS about the session establishment via Ndcsf_ImsEventExposure_Notify message. The message includes calling and called parting id, application id, session id, MF-side MDC2 media resource info, Resource URL used by DC AS to send subsequent ADC Control message.

13.	DC AS responds to the Ndcsf_ImsEventExposure_Notify message.

14.	DCSF waits for ADC control instructions (Ndcsf_ADCControl_Create). If such instructions are not provided then DCSF has the ADC establishment procedure aborted (Nimsas_MediaControl_MediaInstruction).

15.	DC AS allocates DC AS-side MDC2 media resource for this session. DC AS sends Ndcsf_ADCControl_Create message to the DCSF with the instruction for session control and the allocated DC AS-side MDC2 media resource information. The message includes session id, DC AS-side MDC2 media resource info.

16-23.	IMS AS performs SDP negotiation with terminating network. And the IMS AS notifies the DCSF about the successful result of the SDP negotiation.

24.	DCSF updates the media resources in the MF via IMS AS, based on the received the DC AS-side MDC2 media resources information and the media resource information of the terminating network. Then, the IMS AS sends response to DCSF. This follows existing procedures e.g. steps 11 and 12 of TS 23.228 [2] Figure AC.7.2.2-1

25. Void26-33.	IMS AS continues performing SDP negotiation with the originating network. With a successful SDP negotiation, the application data channel established.

### 6.X.4	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

DC AS/NEF:

-	subscribing the event of UE-initiated P2A/P2A2P ADC establishment.

-	receiving notification about the event of UE-initiated P2A/P2A2P ADC establishment.

-	receiving MF-side MDC2 endpoint information in the notify message.

-	sending DC AS-side MDC2 endpoint information in a new service operation message.

-	subscribing with HSS to discover the DCSF address.

DCSF:

-	receiving third party registration from IMS AS.

-	registering DCSF address in HSS.

-	receiving subscription for UE-initiated P2A/P2A2P ADC establishment event from NEF or DC AS and sending MF-side MDC2 endpoint information in notification message.

-	receiving DC AS-side MDC2 endpoint information in a new service operation message.

HSS:

-	receiving subscription from AF/NEF for discovering DCSF address.

-	notify with DCSF address once DCSF register its address to HSS.

# * * * * End of changes * * *