SA WG2 Meeting #171	S2-2509110

13 – 17 Oct, 2025, Wuhan, CN	(was S2-2508125)

Source:	Nokia

Title:	[WT#1.2, Localized Service Access] Support of Localized Service Access in 6G

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes the WT scope and corresponding Key Issue for supporting localized service.

# 1. Justifications

Work task 1 (in particular 1.2) scope as agreed in 6G System Study item SP-250806 is as the following:

WT#1: Define the overall 6G architecture as collection of capabilities and high-level functionalities considering the following sub work tasks and other work tasks to support 6G access network:

…

1.2 	Study whether and how to support and/or enhance the following aspects in 6G: the SBA framework, network slicing, network sharing, user plane architecture, QoS framework, policy framework, network exposure framework, architecture for specific scenarios e.g. fixed wireless access, localized service access.

…

In addition to the work tasks above, this study will identify other 5G features that will be supported and/or adapted in 6G.

Ensuring support of localized services is part of the study scope and is a key feature for 6G day-1. For that, different deployment models should be supported to ensure vertical use cases and requirements. This WT should ensure that the architectural framework for various deployments models is studied.

The support for access to localized services in 5G is enabled via standalone non-public networks (SNPNs) and public network integrated non-public networks (PNI-NPNs), as defined in Annex N of TS 23.501. Some of the key functionalities needed for supporting localised services are: UE discovery and selection of network providing access to localized service(s), authentication and authorization of UE for accessing localized service(s), onboarding and remote provisioning of credentials to enable UE access to those service(s). All of these key functionalities are already specified in 5GS through SNPN/PNI-NPN, which should serve as the starting point for discussion for support of localised services in 6G.

Furthermore, interface and interaction to support UE mobility and service continuity between PLMN network and network providing localized services, or between networks providing localized service also needs to be studied. In addition, interworking scenarios should be studied leveraging the WT#2 findings on general interworking aspects.

This paper proposes to update the WT#1.2 for localised services, and the corresponding Key Issue description to be documented in the study TR 23.801-01.

**** First Change ****

# Annex A.X. WT#1.2 Scope for localized service access

The scope of localized service access is to study the overall architecture design and enablers for localized service access in 6G, including:

NOTE 1:	Support for localized service as defined in 5G is the starting point for the discussion on support of localized service access in 6G.

-	authorization and authentication of UE for localized service access

-	interface and interaction procedure between PLMN network and networks providing localized service, or between networks providing localized service:

-	Whether and how to support UE mobility and service continuity between PLMN network and network providing localized services, or between networks providing localized service

-	onboarding of UE(s) for localised services

NOTE 2:	UE mobility and service continuity will also cover whether and how to support UE mobility and service continuity between 5G and 6G for localised service(s) that are provided through both 5G and 6G.

**** Next Change ****

# 5.X. Key Issue #1.2.X: Key Issue support for localised service access

This Key Issue aims to study the overall architecture design and enablers for localized service access in 6G, including:

NOTE 1:	Support for localized service as defined in 5G is the starting point for the discussion on support of localized service access in 6G.

-	authorization and authentication of UE for localized service access

-	interface and interaction procedure between PLMN network and networks providing localized service, or between networks providing localized service:

-	Whether and how to support UE mobility and service continuity between PLMN network and network providing localized services, or between networks providing localized service

-	onboarding of UE(s) for localised services

NOTE 2:	UE mobility and service continuity will also cover whether and how to support UE mobility and service continuity between 5G and 6G for localised service(s) that are provided through both 5G and 6G.

**** End of Changes ****