SA WG2 Meeting #171	S2-2509111

Wuhan, China, 13 – 17 October 2025	update of S2-2507376

Source:	Vodafone

Title:	[WT#1.2, user plane architecture enhancement] Multi-vendor N4 interface

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: Reviewing the drafts and revisions of the baseline ‘user plane’ document in the Gothenberg meeting, it is not clear whether the multi-vendor N4 interface topic is adequately addressed.

# 1. Justification

Total Cost of Ownership is a major concern for many operators, and, within the core network, the user plane costs are often significantly greater than the control plane costs.

A significant proportion of mobile network traffic (e.g. from Fixed Wireless Access) just requires a “least cost” method of providing a data pipe between the base station site to the internet.

A large quantity of mobile network traffic is also used by customers with “unlimited” data tariffs. “Unlimited” tariffs mean that there is zero change in HPLMN revenue when data is sent (or not sent) and hence there is little need for the HPLMN to perform (deep) packet inspection to separate “free data flows” from “chargeable data flows”.

For these and other use cases, it is interesting for operators to be able to use cost-optimised UPFs, e.g. reliable UPFs with minimal functionality.

For reasons of operational simplicity, an operator may well choose to use a single vendor for their SMFs. Then, for that operator to use a diverse set of UPFs (e.g. cost optimised UPF for FWA vs high complexity UPF for customers with data flows that need to be zero-rated), it is essential that the SMF-UPF interface supports multi-vendor operation.

However, current network deployments indicate that there is little take up of multi-vendor N4 operation. It is believed that this is partly due to the options in the R13 CUPS functional split.

To minimise Total Cost of Ownership, a truly multi-vendor, option-free interface between the UPF and SMF is expected to be desirable. The identification and removal of the options (e.g. in the functional split) on N4 should be studied.

# * * * * Changes proposed to text in S2-2508121 (last, but unagreed, version in SA2#170) * * * *

# Annex A.X. WT#1.2 User Plane Architecture for 6G Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

[WT#1.2 UP Architecture] In order to support 6G user plane for a diverse set of applications and traffic patterns, the following are studied:

- Whether and how to enhance application-network collaboration for 6G user plane traffic to enable the UE/AS to provide the network with information that allows the network to adapt to application traffic.
Editor’s Note: whether this issue belongs to UP or QoS is FFS.

- Whether and how to enhance CP-UP interaction for better multi-vendor interoperability, e.g. by identifying options on the N4 interface and studying how to remove them in the 6G System.

- Whether and how to extend the service-based architecture for the user plane control interface to UPF.

- Whether and how to support flexible user plane (i.e., UPF (re)selection) for different service requirements considering path performance and capability between access network and data network.

- Whether and how to enhance the user plane handling for session continuity and mobility between access network and data network (i.e., best path selection considering ULCL and PSA-UPF path metrics, and/or for specific services).

# * * * * End of proposed changes * * * *