3GPP TSG SA WG2#171	S2-2509126

Wuhan, China, October 13 – 17, 2025	(was S2-241xxxx)

Source:	Lenovo

Title:	Proposed Key Issues for WT3.1.1 and WT3.1.2

Document for:	Approval

Agenda Item:	20.6.3

Work Item / Release:	FS_6G_ARC/Release 20

Abstract of the contribution:

# 1	Discussion

Key Issues are proposed for WT 3.1.1 and WT 3.1.2.

The Key Issue for Work Task 3.1.1 is focused on proposing a study on evolving the core network to an intent-driven AI-enabled architecture.

The Key Issue for Work Task 3.1.2 is focused on enhancing the AI/ML related functionalities defined in 5G and introduce additional model training techniques (online learning, enhance vertical federated learning, reinforcement learning) and improve model interoperability.

Further justification is included in S2-2509125

# 2	Proposal

The following are proposed to be included in 6G study TR

**** First Change ****

5.Z. Key Issue #Z: Key Issue for WT3.1.1: Study support of a Distributed Intelligent Framework in 6G

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

The 5G System introduced the NWDAF framework, which enables data collection and providing historical / predictive analytics to other network functions. The architecture supports distributed deployments (e.g., central, regional, edge) and collaborative model training (using FL), which are critical elements for network optimization.

While the NWDAF provides the necessary analytics and the NWDAF framework successfully distributes the analytics capability the decision to acts on such analytics still resides with the analytics consumer. This separation between "analysis" and "action" can introduce latency and complexity, limiting the network's ability to react in real-time to dynamic conditions.

To achieve deeper integration of intelligence, taking also into account that AI may be an inherent functionality in each NF in the core, it is essential to evolve from a network that is analytics-driven to one that is intent-driven where based on an intent "goal" each NF in the core is able to determine that appropriate actions to achieve the goal. This requires a framework where intelligence is not just a service to be consumed, but a native capability with the power to act autonomously.

To meet the 6G goals of a lean, efficient, and fully autonomous network, the architecture must support "autonomous agency". This means embedding intelligent entities (e.g., AI Agents) directly into the core network architecture that can perceive the network environment, reason, and execute actions to achieve a high-level goal (an "intent").

As part of this key issue for WT3.1.1 the following aspects should be studied:

-	Study whether and how the 6G core network supports an intent-driven AI-enabled architecture

-	Study how an intent-driven request can be defined including which functions can be consumers and which functions can be recipients of such request.

-	Study the necessary interfaces and interactions between NFs (e.g. for the purpose of coordinating NFs) to achieve a desired goal based on the intent-driven request.

**** Second Change ****

5.Z. Key Issue #Y: Key Issue for WT3.1.2: Study support of an enhanced AI/Ml related operations in 6G

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

The NWDAF framework already supports various methods to provision trained AI/ML models to an NWDAF as well as, more importantly, defined methods to train AI/ML models, such as supporting model training using Federated Learning. In Release 19, the NWDAF was also enhanced to support training of AI/ML models for other services such as train an AI/ML model for positioning to an AI-enabled LMF. Such LMF uses the trained AI/ML model to estimate location of a UE.

For 6G networks, AI is expected to be natively supported in multiple core network functions. As such the 6G network should support a framework to enable training of AI/ML models for different AI-enabled services within the 6G core. It is proposed to evolve the existing model training framework defined for the NWDAF.

As part of this key issue for WT3.1.2 the following aspects should be studied:

-	Study how the model training framework defined in 5G (i.e. NWDAF framework) can be enhanced to support training and provisioning ML models for different AI-enabled services supported within the 6G core network. In addition study support of additional model training mechanisms such as online learning, reinforcement learning and improve existing model training mechanisms (e.g. federated learning) in order to have a comprehensive set of model training tools within the 6G network.

-	Study for provisioning ML models to different AI-enabled functions in the core how to improve ML model interoperability.

**** End of Changes ****