SA WG2 Meeting #171	S2-2509128

Wuhan, China, 13 – 17 October, 2025	(revision of S2-2508132)

Source:	Lenovo

Title:	[WT#5 scope update] Proposed WT5 scope update

Document for:	Approval

Agenda Item:	20.6.5

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution:

# 1. Discussion

At SA#101 a decision was made for 3GPP SA2 to continue defining the scope for WT5 having as a starting point the scope of 3GPP SA2 for data related aspects during 5GA.

In 5GA (Release 20 and earlier) SA2 has been developing data collection procedures for the following features:

-	AI/ML

-	3GPP TS 23.288 define the data collection procedures for the NWDAF as a data consumer to retrieve data from core Network Functions, AF (and UE), OAM to train ML models and derive analytics

-	Ongoing study in 3GPP TR 23.700-04 where a Data Collection Function retrieves radio-related data from a UE for the purpose of training a UE-side model at a 3rd party server.

-	Positioning

-	3GPP TS 23.273 define the data collection procedures for the LMF to retrieve relevant data from UE and RAN for determining location of a UE.

-	3GPP TS 23.288 define the data collection procedure for the NWDAF to retrieve relevant data from LMF to train an AI/ML model used by LMF for AI/ML enabled positioning

-	Sensing

-	Ongoing study TR 23.700-14 where a Sensing Function retrieves sensing related data from Sensing Nodes that are used by the Sensing Function to determine a sensing event.

-	Energy Saving

-	EIF obtaining data from 5GC NF(s) to assist the calculation of energy related information (clause 6.2.34 of 3GPP TS 23.501).

The following figure shows the SA2 scope in 5GA.

Figure 1 - SA2 working scope in 5GA

The proposal is to, initially, limit the scope of WT5 to the 5GA working areas as highlighted above. Additional topics can be considered in the future as per relevant use case.

# 2. Proposal

It is proposed to approve the following WT scope on top of the postponed WT5 scope document (S2-2508132)

**** First Change ****

# A.X 	Work Task X: Eficient Data Collection, Coordination, Distribution and Handling framework in 6G

The following is WT#5 in the SA2 6G SID:

WT#5: Study data framework for all aspects related to efficient and scalable data handling including, for example, data collection, distribution, processing, storage, data access and data exposure, with consideration of access control/user consent and privacy where relevant. The example of data may include data for AI and Sensing. This WT can also study any potential enhancements on system and procedure needed for user consent framework.

NOTE 6:	The work split with SA3, SA5 and RAN WGs will require TSG coordination

The scope of the WT includes the following aspects:

- Study whether and how a more efficient and scalable data collection, coordination, distribution and handling framework can be supported in 6G system. Data for AI/ML, Sensing, Energy Saving and Positioning are the initial scope.

NOTE X: Data Collection, Coordination, Distribution and Handling for other features can be studied once a relevant use case is identified.

NOTE 1:	Coordination with SA5 and RAN WG when required to align with the data handling mechanisms defined in the respective WGs..

- Study from SA2 architectural and system level on user consent, privacy, security and data governance to support the 6G data framework.

NOTE 4:	Coordination with SA3 is required for user consent, privacy, security and data governance related topics.

**** End of Changes ****