SA WG2 Meeting #S2-171	S2-2509129

13 - 17 October, 2025, Wuhan, China

Source: 	China Telecom

Title: 	KI#1 Interim agreement update

Document for: 	Approval (P-CR)

Agenda Item: 	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: This paper proposes an interim agreement update on KI#1 for FS_AIML_CN_Ph2.

- Discussion

Whether and how the UE controls UE data transfer

As per RAN’s use case and requirement, it should be 5GC to initiate, terminate and manage UE data transfer. However, the 5GC control of UE data transfer cannot be achieved without the assistance from UE, e.g.

- 5GC can only initiate UE data transfer after UE data collection is finished, which means the UE needs to tell the network it’s ready for data transfer and it’s up to the network to determine whether to initiate transfer or not.

- If the UE has transferred all the collected data, the network should also receive the indication from the UE. The 5GC can either terminate the data transfer directly, or verify the received data and request some of them retransmitted if not available.

- If the UE’s status is no longer available for the data transfer ongoing, the UE should also inform the network. Based the indication from the UE, the 5GC can terminate the data transfer, and determine whether and when to resume the data transfer process.

It is proposed to clarify how the UE being involved in the control of UE data transfer.

Whether and how to select UEs for UE data collection

Most solutions of KI#1 have described the criteria of UE selection, e.g. based on the PEI list, AoI, UE’s AIML capability, etc. It is no doubt that UE selection is needed for UE data collection.

- Different data type requests different UE capability, service type, network environment, etc.

- A single selection criteria can lead to a large amount of data duplication which is not good for model training.

In addition, even being selected for UE data collection, the UE can have rights to reject the data collection request due to e.g. the low power status, critical service ongoing, high moving speed, etc. How to select the appropriate UEs for UE data collection still need further consideration.

- Proposal

It is proposed to approve following update for KI#1 interim agreement and further consideration of TR 23.700-04 v1.0.2.

* * * * Start of 1st change * * * *

### 7.1.1	Agreed Principles for KI#1

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#1. This may be populated directly or e.g. also when a topic in clause 7.2.1 gets resolved and a principle is agreed.

The interim agreements on principles for KI#1 are as follows:

-	A UP path is used between the UE and a 5GC NF, for transferring standardized collected data from the UE using PDU connectivity service provided by a PDU session.

-	Dedicated S-NSSAI/DNN is used to differentiate the traffic of collected UE data from the UE regular traffic per PDU Session level.

-	The UE establishes a new PDU Session, or modifies an existing PDU Session which has been established for UE data transfer, to transfer the standardized UE data, based on URSP rules.

-	The initiation, termination, management of UE data transfer is controlled by 5GC, which may receive UE data collection and transfer related indication (e.g. ready for data transfer, transfer finished, unavailable for data collection/data transfer, etc.) from the UE.

* * * * End of 1st change * * * *

* * * * Start of 2nd change* * * *

### 7.2.1	Topics for further consideration for KI#1

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#1. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in a clause 7.1.1) and can either be then marked as not pursued or postponed to a future Release.

The following topics/principles are for further consideration:

-	How the 5GC NF controls UE data transfer.

-	Whether an existing 5GC NF or a new 5GC NF is used for controlling UE data transfer.

-	How the configuration information for UE data transfer is sent to the UE.

-	How the 5GC NF sends its UP information (i.e. IP address or FQDN of the 5GC NF), for standardized UE data transfer from the UE to the 5GC NF.

-	How the 5GC NF checks the reported UE data to support the full visibility of standardized UE data contents.

-	How to select UEs for UE data collection.

-	Whether or not to support differentiating the traffic of collected UE data from the UE regular traffic at QoS flow level (e.g. using the FQDN or address of the 5GC NF as traffic filter).

-	Whether the UE needs to be authorized for UE data collection.

-	Whether and how to align the UE data measurement with the requirement of UE data transfer provided by the 5GC NF.

* * * * End of 2nd change * * * *