SA WG2 Meeting #170	S2-2509145

13th – 17th October, 2025, Wuhan, China

Source:	ETRI

Title:	New Sol: Emergency SMS over NAS

Document for:	Approval

Agenda Item:	20.8.1

Work Item / Release:	FS_SMS2EC_ARC / Rel-20

Abstract of the contribution: The contribution proposes a new solution for KI#1 and KI#2 of TR 23.700-65.

1. Introduction

The key issue #1 of the TR 23.700-65 is:

This key issue aims to study solutions to:

-	identify whether an SMS should be routed to a PSAP;

-	identify the type of emergency service (police, ambulance, fire brigade, etc.) required;

-	address both the case of the UE detecting and the UE not detecting an SMS that is to be routed to Emergency Response Centre;

-	consider the scenario where the serving PLMN does not support routing an SMS to an Emergency Response Centre.

The key issue #2 of the TR 23.700-65 is:

To support the SMS delivery to the Emergency Response Centre via NAS based on local regulations, following aspects are included:

-	study how to route the SMS via NAS procedures for the support of SMS delivery to local Emergency Response Centre, e.g. UE location determination, via SMS service centre of the serving PLMN;

NOTE 1:	Identification of SMS delivery to Emergency Response Centre is based on the conclusion of other KIs.

-	how to support Emergency Response Centre callback for sending short messages using regular SMS delivery service.

The key issue covers SMS delivery via NAS both in EPS and 5GS.

NOTE 2:	The solutions for EPS and 5GS are based on same principles.

The key issue covers both roaming and non-roaming scenarios.

2. Proposal

It is proposed to agree the following updates to TR 23.700-65.

*** Start of Changes ***

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 |
| X | x | x |  |
| - |  |  |  |
| - |  |  |  |
| - |  |  |  |
| - |  |  |  |

*** Next Change ***

6.X	Solution #X: <Solution Title>

6.X.0	High level principles

This solution addresses KI#1 and KI#2.

The principles of this solution are listed below:

-	UE sets the RP-DA to the emergency number if the entered number is identified as an emergency number; otherwise, the UE sets the RP-DA to the address of the SMSC.

-	PLMN sends an RP-ERROR if it does not support routing of SMS to an Emergency Response Centre.

-	SMSF in 5G and SMSC in EPS determine whether the SMS is an emergency SMS based on the RP-DA and TP-DA.

-	SMSC selects the appropriate PSAP based on the TP-DA and the UE’s location information (ULI).

6.X.1	Description

For SMS over NAS, the UE constructs the SMS as follows. The UE compares the entered number with the Emergency Number List. If the entered number matches an emergency number, the UE shall set the RP-DA to the emergency number. Otherwise, the UE shall set the RP-DA to the address of the SMSC. The TP-DA is set to the entered number. Based on the RP-ACK or RP-ERROR received from the network, the UE determines whether the emergency SMS has been successfully delivered to the PSAP.

For Emergency SMS over NAS, the SMSF of 5G and SMSC in EPS determine whether the SMS is an emergency SMS based on the RP-DA and TP-DA. If either the RP-DA or the TP-DA corresponds to an emergency number, the message is considered as an emergency SMS. The SMSC determines the appropriate PSAP based on the TP-DA and the UE’s location information (ULI), and forwards the message to the selected PSAP.

If the serving PLMN does not support routing of SMS to an Emergency Response Centre, the SMSC returns a forward result containing an appropriate error code to the SMSF.

Upon receiving a Submit Report that contains an error code, the SMSF shall return an RP-ERROR to the UE with the corresponding cause value.

6.X.2	Procedures

#### 6.x.2.1 Emergency SMS over NAS in 5G

Figure 6.x.2.1-1: Emergency SMS over NAS in 5G

1.	The UE performs domain selection for UE originating SMS as defined in clause 5.16.3.8 of TS 23.501 [2] if SMS delivery via non 3GPP access is allowed and possible. If an UE under CM-IDLE state is going to send uplink SMS message, then UE and network perform the UE Triggered Service Request procedure firstly as defined in clause 4.2.3.2 to establish a NAS signalling connection to AMF.

2a.	The UE builds the SMS message to be sent as defined in TS 23.040 [7] (i.e. the SMS message consists of CP-DATA/RP-DATA/TPDU/SMS-SUBMIT parts). The SMS message is encapsulated in an NAS message with an indication indicating that the NAS message is for SMS transporting.

2b.	The AMF forwards the SMS message and SUPI to the SMSF serving the UE over N20 message by invoking Nsmsf_SMService_UplinkSMS service operation. In order to permit the SMSF to create an accurate charging record, the AMF adds the IMEISV, the current UE Location Information (ULI) of the UE as defined in clause 5.6.2 of TS 23.501 [2] and if the UE has sent the SMS via 3GPP access, the local time zone.

2c.	The SMSF invokes Namf_Communication_N1N2MessageTransfer service operation to forward SMS ack message to AMF.

2d.	The AMF forwards the SMS ack message from the SMSF to the UE using downlink NAS transport.

3-5.	Based on the RP-DA and TP-DA, the SMSF determines if the message is an Emergency SMS. If it is an Emergency SMS, the SMSF may change RP-DA with local SMSC address and forwards the SMS, including the ULI, to the SMS-IWMSC. For the emergency SMS the SMSF shall not check the SMS management subscription data. If SMS delivery is allowed, the procedure defined in TS 23.040 [7] or TS 23.540 [84] applies. The SMSC selects PSAP based on the TP-DA and ULI and forwards the message to the selected PSAP. When the serving PLMN does not support routing of SMS to an Emergency Response Centre, the SMSC returns a forward result with an appropriate error code to the SMSF.

6a-6b.	The SMSF forwards the submit report to AMF by invoking Namf_Communication_N1N2MessageTransfer service operation which is forwarded to UE via Downlink NAS transport. If the SMSF knows the submit report is the last message to be transferred for UE, the SMSF shall include a last message indication in the Namf_Communication_N1N2MessageTransfer service operation so that the AMF knows no more SMS data is to be forwarded to UE. If the submit report contains an error code then the SMSF shall return an RP-ERROR to UE with the corresponding cause value.

NOTE:	The behaviour of AMF based on the "last message indication" is implementation specific.

If the UE has more than one SMS message to send, the AMF and SMSF forwards SMS /SMS ack/submit report the same way as described in step 2a-6b.

6c-6d.	When no more SMS is to be sent, UE returns a CP-ack as defined in TS 23.040 [r1] to SMSF. The AMF forwards the SMS ack message by invoking Nsmsf_SMService_UplinkSMS service operation to SMSF.

#### 6.x.2.2 Emergency SMS over NAS in EPS

Figure 6.x.2.2-1: Emergency SMS over NAS in EPS

1.	The UE performs domain selection for UE originating SMS if SMS delivery via non 3GPP access is allowed and possible. If an UE under CM-IDLE state is going to send uplink SMS message, then UE and network perform the UE Triggered Service Request procedure.

2a.	The UE builds the SMS message to be sent as defined in TS 23.040 [7] (i.e. the SMS message consists of CP-DATA/RP-DATA/TPDU/SMS-SUBMIT parts). The SMS message is encapsulated in an NAS message with an indication indicating that the NAS message is for SMS transporting.

2b.	The MME forwards the SMS message to the MSC/VLR in an Uplink Unitdata message. In order to permit the MSC to create an accurate charging record, the MME adds the IMEISV, the local time zone, the Mobile Station Classmark 2, and the UE's current TAI and E-CGI.

2c.	The MSC/VLR acknowledges receipt of the SMS to the UE using downlink unit data message.

2d.	The MME forwards the SMS ack message from the MSC/VLR to the UE using downlink NAS transport.

3-5.	These steps are performed as defined in TS 23.040 [14]. The SMS message is forwarded to the SC that returns a delivery report message. Based on the RP-DA and TP-DA, the SMSC determines if the message is an Emergency SMS. If it is an Emergency SMS, the SMSC selects PSAP based on the TP-DA and UE's current TAI and E-CGI and forwards the message to the selected PSAP. When the serving PLMN does not support routing an SMS to an Emergency Response Centre, the SMSC returns forward result with appropriate error code to MSC/VLR.

6a-6b.	The MSC/VLR forwards the received submit report to the MME associated with the MS/UE in a Downlink Unitdata message. If the submit report contains an error code then the MSC/VLR shall return an RP-ERROR to UE with the corresponding cause value. The MME encapsulates the received delivery report in an NAS message and sends the message to the MS/UE.

6c-6d.	The UE acknowledges receipt of the delivery report to the MSC/VLR.

6.X.3	Impacts to Services, Entities and Interfaces

UE:

-	Support to detect emergency numbers based on the input number.

-	Support to include emergency number in RP-DA for the detected emergency input number.

-	Support to check RP-ERROR to determine whether the serving PLMN supports routing of SMS to an Emergency Response Centre.

SMSF:

-	Support to return an RP-ERROR to the UE with the corresponding cause value for error codes received from SMSC.

-	Support to determine whether the message is an Emergency SMS based on the RP-DA and TP-DA.

SMSC:

-	Determine whether the message is an Emergency SMS based on the RP-DA and TP-DA.

-	Support to select PSAP based on the TP-DA and UE's location.

-	Support to forward emergency SMS to PSAP.

-	Support to return a submit report containing an error code when the PLMN does not support routing of SMS to an Emergency Response Centre.

*** End of Change ***