3GPP TSG SA WG2#171	S2-2509148

Wuhan, China, October 13 - 17, 2025

Source:	Lenovo

Title:	Evaluations and proposed Interim conclusion for KI#5

Document for:		Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing-ARC / Rel-20

Abstract of the contribution: Proposes evaluations and interim conclusion for KI#5 Sensing Result Exposure

# 1. Introduction

This pCR discusses the evaluations and proposes the interim conclusion for KI#5 Sensing Result Exposure. This paper mainly focuses on the following issues:

-	what types of Sensing results with or without the Sensing contextual information as defined in TS 22.137 [2] could be exposed from the Core Network to the Sensing Service Consumer.

-	How the Core Network exposes Sensing results to the Sensing Service Consumer (e.g. one time, periodic or event trigger).

# 2. Discussion

There are 13 solutions which have been proposed to address KI#5, i.e., Solution#1-3, Solution#19, Solution#23, Solution#27-33, Solution#37.

- Solution#1 proposed that a sensing service is initiated towards a Sensing Function (SF), (which calculates and provides the sensing result), from a Sensing Service Consumer, which can be a NF, a UE or via a Gateway Function. Based on the configuration from the Sensing Entity may provide the Sensing Data/Result to a location different from the SF, e.g. a Sensing Consumer, directly.

- Solution#2 proposes a new Sensing Support Function (SSF) to enable a UE that operates as a Sensing Service Consumer to request a sensing result from the core network via a user plane connection.

- Solution#3 proposes a sensing request or subscription to be send to a SF from a sensing service consumer, i.e., an untrusted AF authorized by NEF. A Gateway sensing NF is also defined for service authorization enabling a sensing service request that needs support from multiple sensing service area with the actual sensing service coordination performed by a (distributed) sensing NF which serves the target sensing service area.

- Solution#19 proposes a procedure for sensing request and exposure coming from a Sensing Service Consumer that may reside within a 3rd party AF involving an exposure function or a UE via the mobility function. A sensing management function coordinates the request initially including validation/authorization and then for the preparation and result provision either the sensing processing function in the core network or the sensing entity.

- Solution#23 proposes to support multiple sensing processing functions (SePF) and may require supporting of chaining of multiple SePFs to get a final Sensing Result; this allows the selection of a single or multiple SePFs.

- Solution#27 proposes sensing parameters for the sensing request towards the SF and in turn to the SE. A Sensing request may be a one-time, or a subscription with periodic reporting or event-based, issued by a trusted or untrusted sensing consumer. The network can support the sensing capabilities to detect objects, expose location and velocity, and attributes (e.g. e.g. type of object, size/shape of object, orientation of object etc.) and environment sensing (e.g. rain detection). The SF can receive multiple and heterogeneous sensing requests from a Sensing Client (e.g., AF/NF), indicating: (i) sensing service type, (ii) sensing area, (iii) Sensing Quality of Service, (iv) Sensing time parameters (iv) Sensing contextual information (e.g., target object type, target object mobility profile, sensing environment), and (v) Sensing results configuration (e.g., expected sensing output, reporting periodicity or subscribed events). A report may contain one or more objects including: (i) object ID, (ii) object type, (iii) location, (iv) detection time, (v) object mobility and (vi) confidence.

- Solution#28 proposes an exposure capability towards an untrusted AF authorized by NEF. An AF invokes a sensing service to request the network to perform: (i) object detection or (ii) object detection and tracking within a requested area. To specify the form of sensing results, the AF may include the Sensing Result Detail Level parameter in the request: (i) notify result only or (ii) notify result with associated information. The Sensing Core Function (SnCF) or the NG-RAN if capable, may determine a sensing result. Inside the request the AF includes the following mandatory and optional parameters: (i) AF ID, (ii) target sensing area, (iii) type of sensing to perform, (iv) sensing result detail and (v) optional event reporting and sensing result exposure parameters (e.g. time and/or spatial resolution of the determined sensing result).

- Solution#29 supports sensing result exposure to an AF assuming two type of sensing results: i) real-time sensing results (e.g., real-time trajectory reporting) and ii) Subscribe/Notification sensing results or event based. Real-time sensing results require high exposure frequency and large data volume. Hence, exposing is provided by a data connection. Event triggering sensing results require a low frequency and small data volume, hence both methods of signaling and data connection can be used.

- Solution#30 proposes a service interface and parameter set that supports one-time requests, continuous monitoring, updates, and notifications for sensing services. A SF aggregates data from distributed sensing entities, processes the information, and delivers results enriched with contextual information. Optional sensing assistance information (e.g. area of interest, motion thresholds, target object profile) may be provided to improve sensing accuracy and relevance. Each sensing request includes: (i) Sensing type, (ii) Target object or area, (iii) Sensing Quality, (iv) Reporting mode, (v) Optional sensing assistance information. The Sensing result contains: the Sensing result payload and contextual information including timestamp, accuracy estimation, confidence level.

- Solution#31 proposes a Gateway Sensing Management Function (GSMF) to control sensing result exposure, i.e., can determine if sensing results are already stored or needs to be prepared the Sensing Service Management Function. The GSMF determines the result transferring mode, i.e., Control Plane or via a Data connection, which needs to be established before transferring the sensing result, based on the capability of the sensing result provider and consumer, result format, sensing service type and operator configuration. The request may include: (i) consumer ID/address, (ii) sensing service type, (ii) target sensing service area, (iv) expected sensing result, including result format, time availability and QoS requirements, and (v) capability of AF for transferring sensing result over data connection

- Solution#32 proposes a Sensing Management Function that receives requests from authorized untrusted and trusted AFs. The attributes present in a sensing service request may include: (i) AF ID, (ii) sensing correlation ID, (iii) target sensing area, (iv) sensing service type(s), (v) sensing time window, (vi) object definition and (vii) type of event reporting (one time, periodic, number of event reports, duration of reporting), (viii) quality indication of the sensing result (i.e., confidence and accuracy). The sensing service response may include: (i) object presence detection and count, (ii) object definition (e.g., object ID, properties, object status (e.g. moving/still), object dimensions and location), (iii) quality indication of sensing result, (iv) velocity of sensed object, and (v) timestamp.

- Solution#33 proposes a sensing service consumer may request a one-time, periodic, or an event triggered sensing service report based on a target sensing service area or for a location of target UE. When a sensing service is authorized, a gateway sensing NF may assign a Sensing Operation ID to represent the authorized sensing operation. A sensing request may include: (i) sensing service type (object detection, object tracking, environment sensing) and (ii) sensing service requirements (e.g. accuracy, latency, resolution), (iii) a target region or a target UE and (iv) sensing report type (e.g., one time, periodic, or event triggered). A UE sends a Service Request for Sensing in a UL NAS Transport message containing the UE ID. When an AF requests a sensing service it is based on the location of target UE.

- Solution#37 proposes a solution that enables untrusted AF consumers to request a sensing service. The result exposure can be based on a request for one-time, periodic, or event subscription request with the AF submitting a follow-up request referring to previous request(s) based on sensing result. The AF may have the role of a sensing service requester and additionally it can be the consumer. The offered sensing services include: (i) Object Detection within a Target Sensing Service Area (may also corresponds to the path), (ii) Border Crossing Detection, and (iii) Path Monitoring, Path Tracking. The input for each type of request may include: (i) Target Sensing Service Area, (ii) list of the object and (ii) some filtering parameters, while the output detects objects with associated sensing information (e.g. object identifiers and result ID, crossing boarder direction, matching criteria, etc.)

# 3. Text Proposal

It is proposed to capture the following changes in TR 23.700-14.

# * * * * First change * * * *

# 7	Interim agreements

## 7.1	Agreed Principles

### 7.1.Y	Agreed Principles for KI#5

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.Y gets resolved and a principle is agreed.

- SF calculates and exposes sensing result towards an AF (trusted/untrusted via NEF).

- Consumer may request sensing service for target area, target object or for a location of a target UE.

- SF may receive a one-time sensing request, a periodic subscription, an even based notification request related to a sensing result.

- Sensing results can be provided via the control plane.

- Sensing results can be stored and retrieved if needed or can be prepared by SF.

- A Sensing request may include: (i) sensing service type, (ii) target sensing area or a target UE, (iii) Sensing Quality indication (accuracy, latency, resolution), (iv) Sensing time parameters, (iv) Sensing contextual information (e.g., target object type, target object mobility profile, sensing environment), (v) Sensing reporting information (e.g., reporting mode, remaining report number, event reporting, time, etc.), and (vi) consumer ID/address.

- When a sensing service is authorized, it is assigned a Sensing Operation ID.

- The network can support sensing capabilities to detect objects, object tracking within a requested area or expose object location, velocity, or other attributes (e.g. type of object, size/shape of object, orientation of object etc.) and environment sensing (e.g. rain detection).

- A sensing report may contain one or more objects including: (i) object presence, (ii) object type or object dimensions, (iii) object location, (iv) object status (e.g. moving/still), (v) Object ID, (vi) detection time, (vii) object mobility, (viii) confidence level and quality indication of sensing result, and (ix) timestamp.

# * * * * End of changes * * * *