SA WG2 Meeting #171	S2-2509155

October 13 – 17, 2025, Wuhan, China

Source:	Ericsson

Title:	Aspects to fulfil for conclusions on KI#2 IMS enhancement for GEO NB-IoT NTN access

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: The contribution discusses the consideration of both a prerequisite and an evaluation criterion when assessing the proposed solutions for KI#2 – IMS enhancement in GEO NB-IoT NTN access. Furthermore, it proposes new text to be added to the Architectural Assumptions section.

# Discussion

As of SA2#171, there are 12 proposed solutions that address KI#2 – IMS enhancement for GEO NB-IoT NTN access. These solutions encompass a wide range of SIP signaling combinations, which makes their assessment quite complex. To aid in this evaluation, it is recommended to adopt the following prerequisite and evaluation criterion:

Prerequisite: An agreement is needed on the exact  Rtransmissionbps value to be used in Call Setup time calculations.

Reason: The 22.261 stage 1 requirements specify a range between 1 and 3 kbps. The estimation of ETimsDelay component of the overall Call setup-time is extremely sensitive to the value of the available maximum bit rate over the service link.

The definition of Rtransmissionbps and ETimsDelay can be found in TR 23.700-19, 1.0.0, Annex A.

Evaluation Criterion: –	Ensure the solutions under evaluation avoid the “ghost ringing” risks (see definition below)

Reason: When terrestrial users experience the ghost ringing, there is a high likelihood they will hang up, leading to a waste of GEO satellite resources and to negative impacts on specific operators’ KPIs   The use of preconditions was specifically added in SIP to avoid the “ghost ringing” risk. Some of the proposed solutions eliminate the use of precondition to reduce the number of SIP messages exchanged over the satellite UNI. Moreover, the long SIP Message UNI propagation delay amplifies the “ghost ringing” risks.

Ghost Ringing - Experienced by the call recipient, who, after answering the call (“Hello…”), does not receive any answer from the calling side and thinks there is no one at the other end of the call.

# Proposal

The contribution proposes to consider the following when discussing conclusions for KI#2 - IMS enhancement for GEO NB-IoT NTN access:

- Prerequisite: An agreement is needed on the exact  Rtransmissionbps value to be used in Call Setup time calculations (see TR 23.700-19, 1.0.0, Annex A for details).

- Evaluation Criterion: Ensure the solutions under evaluation avoid the “ghost ringing” risks

* * * Start of change * * * *

# 4	Architectural Assumptions

For IMS voice call over NB-IoT (GEO), the following architecture assumptions and principles are applied to this study:

-	The EPC architecture with GEO satellite access for NB-IoT, as defined in TS 23.401 [5] is used as baseline.

NOTE 1:	CP CIoT EPS optimization, UP CIoT EPS optimization and S1-U based NB IoT NTN solutions will be studied.

NOTE 2:	Solutions should take into account that CP CIoT EPS optimization data in current pre Rel-20 NB-IoT is regarded as NAS signalling and by default has higher priority than UP data.

-	There will be no mobility enhancements developed for the NB-IoT NTN in this study.

-	Enhancements to NB-IoT (GEO), EPC, and IMS for supporting voice call via GEO shall be minimized.

-	Only architecture options with data via SGi will be considered in the study. The option with data via SCEF/T8 will not be considered.

-	The IMS reference architecture, as specified in TS 23.228 [6] for non-emergency calls is used as baseline.

- 	The call setup time requirement documented in TS 22.261 [8] (clause 7.4.2) must be satisfied across the entire specified transmission data rate range of 1–3 kbit/s.

- 	IMS optimizations intended to achieve the required call setup time must also preserve key terrestrial user telephony expectations (e.g. absence of ghost ringing) that contribute to a successful call establishment.

For emergency IMS sessions over NB-IoT (GEO), the following additional architecture assumptions and principles are applied to this study, compared to above:

-	The IMS reference architecture, as specified in TS 23.167 [7] for emergency calls is the baseline.

-	Enhancements to LCS architecture for IMS emergency sessions and other regulatory services are out of scope of this study.

The following architecture assumptions are applied to the study for UE-Satellite-UE Communication:

-	The 5GS architecture for NR satellite access as defined in TS 23.501 [2] is used as a baseline.

-	Inter-Satellite Links (ISL) and Feeder link are assumed to act only as transport layer links and are not specified in 3GPP, and the use of ISL is assumed to have no impact on the reference points between core network entities when these core network entities are onboard.

-	NR UE-Satellite-UE Communication assumes a feeder link is always available.

-	NR UE-Satellite-UE Communication for non-IMS assumes a gNB and a UPF is present onboard of the satellite.

-	NR UE-Satellite-UE Communication assumes that the serving satellite will be of NSGO type.

-	NR UE-Satellite-UE Communication service assumes communication between UEs under the coverage of the same satellite or different satellites of the same PLMN.

-	No UE impacts are expected for UE-Satellite-UE Communications.

* * * End of change * * * *