SA WG2 Meeting #171	S2-2509182

13 – 17 OCT, 2025, Wuhan, China

Source:	Oracle

Title:	[WT#1.2] Proposal on SBA framework enhancements

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: this contribution suggests SBA framework enhancements as a key issue related to WT#1.2

# 1. Justifications

WT#1.2 includes the following:

1.2.	Study whether and how to support and/or enhance the following aspects in 6G:

…

SBA framework,

…

Current 5G SBA:

Below are some key functionalities currently supported by the 5G SCP:

- Message Routing & Load Balancing:

The SCP directs service requests to the appropriate NF service producers, distributing traffic efficiently across multiple instances and improving performance and reliability.

- Enforcement of Common routing policies:

SCP, provides a centric place for implementing policies (e.g., retries, timeouts, routing, etc.).

- Delegated Service Discovery:

Instead of direct communication, the SCP can offload consumer NF from the process of discovering and selecting suitable NFs for a given service by interacting with the NRF.

- Load and Overload Control:

It monitors NF availability and load, implementing measures like circuit breaking to prevent requests from being sent to degraded or unavailable NFs.

- Network Protection from Signaling storms:

SCP protects the producer NFs from signaling storms by implementing congestion control or rate limiting features.

- Interoperability/mediation:

By acting as an intermediary, the SCP can transform or harmonize messages exchanged between Network Functions from different vendors or supporting different 3GPP releases or/and different HTTP versions, easing vendor integration.

- Security:

In the encrypted 5G core, the SCP can provide TLS termination and mirroring of traffic for security monitoring, ensuring the overall security of indirect communication. In addition, SCP can protect the network from unauthenticated consumer NFs.

- Observability:

Being a node that sits on the path of all SBA communication of 5G core network, SCP can provide the complete network visibility in terms of NF health, traffic flow, 5G SBI message processing status at different NFs including latency, success/error response rate etc.

- Resiliency:

The SCP enhances network resilience by providing a unified view of NF services and enabling fast recovery from failures by re-routing traffic around affected instances.  SCP can detect unavailable or unhealthy NFs and redirect requests, improving service resilience and continuity.

- NF offloading:

The SCP allows the NF to focus on business logic, while offloading concerns such as discovery, selection ,routing, signaling prioritization, recovery from signaling routing errors to name a few.

- Signaling Traffic prioritization:

Thanks to its 3GPP visibility, the SCP can perform advance tasks such as Signaling Traffic prioritization based on message contents.

- Improved Scalability and better connection management/configuration:

In direct communication, scaling NFs (e.g., handling sudden load) requires updating all other NFs with connection information. It is difficult to manage and update policies centrally.

In indirect communication, NFs connect only to SCP, not individually to one another, simplifying scale-out/in NF procedures. This becomes noticeable in Large 5G networks where operator can end up with a network of thousands of NFs (some current 5G networks have more than 8000 Full Blown NF instances). SCP acting as a central relay, help to significantly reduce point-to-point connections and mesh complexity.

- Topology hiding

SCP can assist in topology hiding by inspecting message headers or body (before they are routed to a different PLMN) and applying necessary modifications.

Observation 1: 5G SBA and its stand-alone SCP in its core provides lots of useful functionalities above and beyond signaling routing. It may be highly desired to carry over these functionalities for 6G SBA as a basis.

Current 5G RAN:

Below is figure 6.1-1 and figure 6.1.2-1 from TS 38.401. These figures reflect current 5G distributed architecture of the RAN. These can lead to many connections inside the RAN and between the RAN to the AMF.

Figure 6.1-1: Overall architecture

Figure 6.1.2-1. Overall architecture for separation of gNB-CU-CP and gNB-CU-UP

Observation 2: 5G RAN has distributed architecture. If it is decided by RAN3/SA2 to transit N2 towards REST/HTTP based SBA, it may be highly beneficial to leverage the functionalities of 5G SBA and SCP in its core for 6G SBA type of N2.

Conclusion:

It is recommended to carry over these critical capabilities from 5G SBA to 6G SBA. In other words, it is recommended not to dumb 6G SBA down compared to 5G SBA, but rather leverage the work that was done in 5G SBA and make 6G SBA an optimized and affective version of 5G SBA.

# Proposal

# It is proposed to approve the following WT scope and key issue in TR23.801-01.

**** First Change (all new text) ****

#### Annex A.X. WT#1.2.X Service-based architecture framework scope

To support SBA Framework in 6GS, the SBA Framework, defined in TS 23.501 (including Annex E) and illustrated in 23.502 section 4.17, is considered as starting point for discussion.

The following is WT#1.2 in the SA2 6G SID

WT#1.2.	Study whether and how to support and/or enhance the following aspects in 6G: the SBA framework, network slicing, network sharing, user plane architecture, QoS framework, policy framework, network exposure framework, architecture for specific scenarios e.g. fixed wireless access, localized service access.

The following is work scope proposal for SBA related topics.

1	Study whether and how to optimize NF discovery and selection for efficient message forwarding and routing compared with 5G.

2	Study whether and how to improve NF resiliency compared with 5G.

NOTE x: Impacts on SBA and applicability can be studied in other WTs.

3  Study whether and how to apply SBA aspects such as discovery and selection, forwarding and routing to N2.

NOTE y: Any discussion related to SCP enhancement for N2 should wait for the joint decision between RAN3/SA2 regarding the N2 SBA.

**** End of First Change ****

**** Second Change (all new text) ****

## 5.X. Key Issue X: Service-based architecture framework

This Key Issue will study whether and how to enhance the service-based architecture framework regarding the following aspects:

1	Study whether and how to optimize NF discovery and selection for efficient message forwarding and routing compared with 5G.

2	Study whether and how to improve NF resiliency compared with 5G.

NOTE x: Impacts on SBA and applicability can be studied in other WTs.

3  Study whether and how to apply SBA aspects such as discovery and selection, forwarding and routing to N2.

NOTE y: Any discussion related to SCP enhancement for N2 should wait for the joint decision between RAN3/SA2 regarding the N2 SBA.

**** End of Second Change ****