3GPP TSG-SA WG2#171	S2-2509213

Wuhan, China, 13-17 October 2025	(was S2-250xxxx)

Source:	Qualcomm Incorporated

Title:	Discussion and way forward for WT#2 (exposure)

Document for:	Discussion/Approval

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC / Rel-20

Abstract of the contribution: This paper discusses the proposed study directions and proposes a way forward.

# 1.	Discussion

The latest draft version of the work task scope for network exposure[1] contains the following aspects:

- X.1 How to support a common framework for the exposure of network capabilities. - Whether and how to support a common framework for the exposure of network capabilities that is exposed to AFs. The exposure framework of the 5GS (i.e. exposure via the NEF) is assumed to be the starting point for discussion of this work task. NOTE 1: Examples of network capabilities include information exposure such as UE related events, network events, analytics, and information provisioning such as UE related parameters, network configuration parameters, and service specific parameters. - How to support enhanced privacy protection, user consent, data anonymization, and service authorization mechanisms. NOTE 2: This aspect of work task has dependency on Work Task #5 (Data Collection). - Whether and how to restrict Network Exposure procedures in roaming, aiming to avoid information being sent between the home network and visited network when not allowed or when not necessary. NOTE 3: The scope of this work task is to specify the common exposure framework, including possible impact to NEF. This work task will not address the information being exposed defined by other work tasks. Other work tasks can cover services that have an exposure component (e.g. computing and sensing). The exposure framework that is developed in this work task should be adaptable to and suitable for those other services (e.g. computing and sensing). NOTE 4: What capabilities are exposed in-band will be coordinated with the work that is done under other work tasks. - X.2 How to support network capabilities exposure to UE. - Whether and how network capabilities should be exposed to a consumer on the UE, considering their untrusted environment and limiting exposure to the hosting UE’s context. - How to easily authorize consumers of network capabilities when the consumer is resident on the UE. - How to support and authorize the UE request for network information exposure. - X.3 How to support In-band Information Exposure. - Whether and how to expose information in-band between application endpoints and 6G network in an operator-controlled manner. - Whether and how the exposure/reporting path of an exposed capability can be optimized (e.g. in-band or not in-band). - How to discover, authorize and activate in-band exposure services offered by the network and whether and how to integrate in-band exposure in the common exposure framework. NOTE 5: In-band information exposure will not replace existing NEF APIs, e.g., for parameter provisioning. NOTE 6: This work task will cover how capabilities are exposed in-band. What capabilities are exposed in-band will be coordinated with the work that is done under the User Plane and QoS aspects of Work Task 1.2. - X.4 Considerations for Intent-Based Exposure. - If, as a result of work that takes place under other work tasks (e.g. WT#3), it is decided that intent-based exposure is supported in the 6G system, what capabilities, if any, are needed in the network exposure framework to support intent-based exposure. The scenarios for intent based exposure (e.g. the goal of an intent) will be defined by the work that is done under Work Task #3. This wok task will study how the procedures on the southbound interfaces can achieve any intents that are identified within work task #3.

With respect to X.2 and X.3: From our perspective none of these topics can be included in the exposure work task at this point. This is because they have not been fully justified yet and would (if agreed) first need to be studied under a separate work task since they are related to QoS.

With respect to X.4: How intents can be provided to the system from e.g. UE or AF would be an integral part of what needs to be discussed under WT#3. From our perspective it is not realistic to only discuss scenarios under WT#3 and cover all further details, specifically “procedures on the southbound interfaces” as part of network exposure. This is not a useful way of splitting up the work in our view.

With respect to X.1 (How to support a common framework for the exposure of network capabilities.): Enabling a common framework for various exposure needs can only be studied when other work tasks that may require exposure support have made at least some initial progress to be able to derive their exposure needs.

Conclusion: All topics proposed for this work task have dependencies on other work tasks. It is not realistic to properly scope the work and start work on a common exposure framework before other work tasks that may or may not require exposure support have made at least some progress.

Proposal: This paper proposes to agree a generic objective for the exposure work task and to put the work on this work task on hold until work tasks have made sufficient progress so that the exposure requirements of those other work tasks can be derived. The work on this work task is not assumed to start before SA2#175 (Q2/2026).

# 2.	References

[1]	Interdigital: [WT#1.2, Network Exposure Framework] Network Exposure Framework. S2-2508136.

# 3.	Text proposal

It is proposed to agree the following changes to TR 23.801-01.

# >>>>BEGINNING OF CHANGES<<<<

# Annex A.X. WT#1.2 Network Exposure Framework

- X.1 How to support a common framework for the exposure of network capabilities.

  - Whether and how to support a common framework for the exposure of network capabilities that is exposed to AFs. The exposure framework of the 5GS (i.e. exposure via the NEF) is assumed to be the starting point for discussion of this work task.

NOTE:	This work task depends on all other work tasks. Work can only start once other work tasks have made sufficient progress so that the exposure requirements of other work tasks can be derived. The work is not assumed to start before SA2#175 (Q2/2026).

NOTE 10:	This work task requires coordination with SA3 (e.g. user consent, privacy and anonymization), SA6 (e.g. CAPIF), and SA5 (e.g. OAM Services).

NOTE 11:	This work task has dependency on service requirements to be specified by SA WG1.

# >>>>END OF CHANGES<<<<