3GPP TSG-SA WG2#171	S2-2509216

Wuhan, China, 13-17 October 2025	(was S2-250xxxx)

Source:	Qualcomm Incorporated, Ericsson

Title:	Discussion and way forward for WT#6 (compute support)

Document for:	Discussion/Approval

Agenda Item:	20.6.6

Work Item / Release:	FS_6G_ARC / Rel-20

Abstract of the contribution: This paper discusses the proposed study directions and proposes a way forward.

# 1.	Discussion

## 1.1	Overview

For WT#6 of SA2’s 6G SID [1] various study directions have been proposed, including:

-	UEs offloading compute tasks to the network;

-	Coordination of communication and compute, service continuity and QoS aspects;

-	Enabling AFs to discover and request operator-deployed compute resources.

Before SA2 can agree on a scope for WT#6, it is important to analyze how to interpret these high-level directions and to verify that no 3GPP solutions exist that already addresses those. The latter is of particular importance as SA2 committed itself to the SA plenary guidance to “minimize the adoption of multiple options for the same functionality” (see 6G SID [1]).

The remainder of this paper discusses the proposed study directions listed above and concludes with a way forward proposal.

## 1.2	Definition of UE offloading compute tasks to the network

Some companies argue that there is a need to study how to enable UEs to “offload compute tasks to the network”. However, it is unclear what this refers to. We see two potential interpretations:

-	UEs or UE applications to dynamically install and execute arbitrary binaries on compute resources provided by the network: A UE or UE application discovers plain compute resources available in the network (i.e., a VM with specific characteristics such as CPU architecture, available CPUs, GPUs, available memory, etc.), receives the credentials for that VM, accesses that VM using those credentials, configures the VM and any networking/firewall rules for that VM, installs an application (i.e., an arbitrary binary) on that VM, starts that application and interacts with it using application layer signaling.

-	UEs to dynamically use network-based application components instead of running the same components on the UE: This interpretation is based on the assumption that a UE application consists of multiple components where some of those can be executed locally or by a separate, e.g. a network entity. Examples include:

-	 a gaming application that consists of a front-end and a specific rendering component and the rendering component may be run locally or by a separate server (see figure 1);

-	an AI application that is built upon a specific large language model (LLM) component and that component (the model) may be executed locally or by a separate server.

In this interpretation the challenge to be addressed is to enable the UE application to discover a specific application component.

Figure 1: A gaming application that consists of a front-end and a rendering component. The rendering component can be run locally on the UE (a) or as a network-based application component on a separate compute server (b).

Observation 1: “Offloading compute tasks from UE to network can be interpreted in different ways: UEs dynamically installing and executing arbitrary binaries on compute resources provided by the network or UEs dynamically using specific network-based application components instead of running those components on the UE.

What would need to be studied to address one of these two directions is also vastly different.

In the former case (dynamically installing and executing arbitrary binaries on computing resources provided by the network), 3GPP would essentially partly or fully replicate what existing compute framework such as OpenStack Nova [2] are already addressing. This includes gathering characteristics of compute instances (CPU architecture (X86, ARM, etc.), number of CPUs, GPU architectures, and available GPUs, load levels, operating systems, etc.), specifying a discovery framework for clients, e.g. based on those characteristics, managing access credentials to these compute instance, etc.

Observation 2: If “UE offloading compute tasks to the network” refers to UEs dynamically installing and executing arbitrary binaries on compute resources provided by the network, then 3GPP would partly or fully replicate what existing compute framework such as OpenStack Nova [2] are already addressing. This includes gathering characteristics of compute instances (CPU architecture (X86, ARM, etc.), number of CPUs, GPU architectures, load levels, operating systems, etc.), specifying a discovery framework for clients, e.g. based on those characteristics, managing access credentials to these compute instance, etc.

It is worth highlighting that at least discovery of compute resources by the UE/UE applications based on compute characteristics has already been specified by SA6: 3GPP SA6 has standardized solutions for discovery and selection of edge application servers (EAS) (that can represent compute resources) with specific EAS Service KPIs like available compute, memory, storage, etc. (see Table 1).

Table1: EAS Service KPIs (source: TS 23.558 [3] clause 8.2.5)

| Information element | Status | Description |
| --- | --- | --- |
| Maximum Request rate | O | Maximum request rate from the AC supported by the server. |
| Maximum Response time (NOTE) | O | The maximum response time advertised for the AC's service requests. |
| Availability | O | Advertised percentage of time the server is available for the AC's use. |
| Available Compute | O | The maximum compute resource available for the AC. |
| Available Graphical Compute | O | The maximum graphical compute resource available for the AC. |
| Available Memory | O | The maximum memory resource available for the AC. |
| Available Storage | O | The maximum storage resource available for the AC. |
| Connection Bandwidth | O | The connection bandwidth in Kbit/s advertised for the AC's use. |
| NOTE: The maximum response time includes the round-trip time of the request and response packet for communication between the AC and EAS, the processing time at the server and the time required by the server to consume 3GPP Core Network capabilities, if any. |  |  |

Observation 3: Compute resource discovery by the UE/UE applications based on compute characteristics has already been specified by SA6, see TS 23.558 [3] clause 8.2.5.

If “UE offloading compute tasks to the network” instead refers to UEs dynamically using network-based application components instead of running the components on the UE, then the challenge to be addressed is to enable the UE to discover those application components on the network. Also, it is important to highlight that in this case there is no need for the UE or the UE application to deal with details of the underlying compute resources.

Observation 4: If “UE offloading compute tasks to the network” refers to UEs dynamically using network-based application components instead of running the components on the UE, then the only challenge to be addressed is to enable the UE/UE applications to discover those application components on the network. In this case there is also no need for the UE/UE application to deal with details of the underlying compute resources.

This challenge may be addressed in an implementation-specific manner or based on a common discovery mechanism.

It is worth pointing out that 3GPP SA6 has already defined such a discovery solution. The EES and EAS discovery procedures defined by SA6 (see TS 23.558 [3] clause 8.5) enables clients (e.g. a UE application client) to discover application servers that provide application components that the UE application can use.

Observation 5: UE applications can already discover network-based application components using the EES and EAS discovery procedures defined by SA6 (see TS 23.558 [3].

## 1.3	Coordination of communication and computing, service continuity and QoS aspects

Some companies argued that there is a need to study coordination between communication and computing, e.g. to consider both computing and communication delays.

However, the Edge Enabler architecture specified by SA6 [3] already supports such coordination of communication and computing, specifically the coordination of Edge Application Server (EAS) selection (representing either generic compute resources or network-based application components as discussed in the previous section) and selection of the appropriate UPFs to reach those servers:

-	The UE (EEC) can already provide expected and minimum service KPIs during EES and EAS discovery, which can include the expected response time. The response time covers “the round-trip time of the request and response packet, the processing time at the server and the time required by the server to consume 3GPP Core Network capabilities, if any” (see TS 23.558 [3] clause 8.2.3), i.e. covers both communication and compute delays.

-	As part of the discovery, the UE (EEC) receives the Edge Data Network (EDN) configuration where the required compute resource is available. As part of this, the UE (EEC) also learns the DNN/S-NSSAI to connect to the Edge Data Network (EDN) where the appropriate Edge Application Server is available (see Table 8.3.3.3.3-2 in TS 23.558 [3])).

Also service continuity considering both communication and compute aspects has already been addressed. This includes relocation to another Edge Application Server (representing either generic compute resources or network-based application components as discussed in the previous section) and related UPF re-selection triggered by UE mobility or based on load observations on the network side while also considering the required KPIs such as required response times (see clause 8.8 in TS 23.558 [3]).

The same applies to requesting QoS for communication between the UE and the Edge Application Server, which can be triggered by an EAS (e.g., to ensure the response times referred to above), see clause 8.8 in TS 23.558 [3].

Observation 6: The Edge Enabler architecture specified by SA6 [3] already supports coordination of communication and compute considering KPIs like expected response time (covering communication and compute delays), service continuity and QoS aspects.

## 1.4	Enabling AF requests for compute resources

The Edge Enabler architecture specified by SA6 allows for AFs to request compute resources: As shown in Figure 2, cloud application servers can interact with the Edge Enabler Servers deployed by MNOs to discover compute resources and network-based application components deployed in the edge data network.

Figure 2: 3rd parties can discover operator-provided compute resources using the Edge Enabler architecture defined by SA6. Source: TS 23.558 clause 6.2c.

Observation 7: The Edge Enabler architecture specified by SA6 [3] already supports discovery of compute resources provided by (mobile) operators.

A related aspect has been addressed by SA5: The Edge Computing Management framework specified in TS 28.538 [4] enables 3rd parties to deploy Edge Application Server workloads to operator hosted compute sites.

Observation 8: The Edge Computing Management framework specified by SA5 in TS 28.538 [4] enables 3rd parties to deploy Edge Application Server workloads to operator hosted compute sites.

# 2.	Conclusion and Proposal

Various study directions have been proposed for WT#6 of SA2’s 6G SID [1], including:

-	UEs offloading compute tasks to the network;

-	Coordination of communication and compute, service continuity and QoS aspects;

-	Enabling AFs to discover and request operator-deployed compute resources.

Related to those study directions, this paper made the following observations:

-	Observation 1: “Offloading compute tasks from UE to network can be interpreted in different ways: dynamically installing and executing arbitrary binaries on compute resources provided by the network or using specific network-based application components instead of running those components on the UE.

-	Observation 2: If “UE offloading compute tasks to the network” refers to UEs dynamically installing and executing arbitrary binaries on compute resources provided by the network, then 3GPP would partly or fully replicate what existing compute framework such as OpenStack Nova [2] are already addressing. This includes gathering characteristics of compute instances (CPU architecture (X86, ARM, etc.), number of CPUs, GPU architectures, load levels, operating systems, etc.), specifying a discovery framework for clients, e.g. based on those characteristics, managing access credentials to these compute instance, etc.

Observation 3: Compute resource discovery by the UE/UE applications based on compute characteristics has already been specified by SA6, see TS 23.558 [3] clause 8.2.5.

-	Observation 4: If “UE offloading compute tasks to the network” refers to UEs dynamically using network-based application components instead of running the components on the UE, then the only challenge to be addressed is to enable the UE/UE applications to discover those application components on the network. In this case there is also no need for the UE/UE application to deal with details of the underlying compute resources.

Observation 5: UE applications can already discover network-based application components using the EES and EAS discovery procedures defined by SA6 (see TS 23.558 [3].

Observation 6: The Edge Enabler architecture specified by SA6 [3] already supports coordination of communication and compute considering KPIs like expected response time (covering communication and compute delays), service continuity and QoS aspects.

Observation 7: The Edge Enabler architecture specified by SA6 [3] already supports discovery of compute resources provided by (mobile) operators.

Observation 8: The Edge Computing Management framework specified by SA5 in TS 28.538 [4] enables 3rd parties to deploy Edge Application Server workloads to operator hosted compute sites.

In summary, it is not clear yet how to interpret “offloading compute tasks to the network”, which suggests that SA2 first needs to agree on an interpretation for this before being able to proceed.

More importantly, all key components of enabling UEs and AF to offload compute tasks to the network while also coordinating compute and communication and addressing aspects like service continuity and QoS have all already been addressed by SA6’s Edge Enabler architecture [3] and SA5’s Edge Computing Management framework [4].

It is not clear why SA2 should study these aspects again given that other 3GPP working group have already defined solutions for the same technical challenges. Related to this, it is essential to emphasize that SA2 committed to the following SA plenary guidance [1]:

The study will work towards goals endorsed at TSG#107(Mar 2025) to "create lean and streamlined standards for 6G, e.g. by dimensioning an appropriate set of functionalities, minimizing the adoption of multiple options for the same functionality, avoiding excessive configurations, etc. Any exception to the above shall be well justified."

Conclusion: The study directions that have been proposed for WT#6 of SA2’s 6G SID [3] are already addressed by SA6’s Edge Enabler architecture [3] and SA5’s Edge Computing Management framework [4]. It is therefore not clear why SA2 should study these aspects again. This would also violate the SA plenary guidance to “minimize the adoption of multiple options for the same functionality; any exception to the above shall be well justified.” that SA2 committed to in the 6G SID [1].

As the way forward, this paper proposes the following:

Proposal

- SA2 to decide on the high-level direction for “UE offloading compute tasks to the network”, i.e. whether this refers to

  - UEs dynamically installing and executing arbitrary binaries on compute resources provided by the network and/or,

  - UEs using network-based application components instead of running the components on the UE.

- SA2, in coordination with SA6 and SA5, to perform a gap analysis of the Edge Enabler functionality defined by SA6 and SA5, and SA2 defined 5G System Enhancements for Edge Computing [5], focusing on offloading compute tasks to the network (by UE and AF), potential coordination between communication and computing, service continuity and QoS.

NOTE:	SA plenary is expected to determine the work split between SA2, SA6 (and potentially SA5) to address any gaps discovered.

# 3.	References

[1]	TSG SA: Study on Architecture for 6G System. SP-250806.

[2]	OpenStack Nova. https://docs.openstack.org/nova/latest/

[3]	TS 23.558: Architecture for enabling Edge Applications.

[4]	TS 23.538: Edge Computing Management (ECM).

[5]	TS 23.548: 5G System Enhancements for Edge Computing.

# 4.	Text proposal

It is proposed to agree the following changes to TR 23.801-01.

# >>>>BEGINNING OF CHANGES<<<<

# Annex A.X. WT#6 Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

WT#6: Study aspects on support of computing in 6G

- Discuss and decide on the high-level direction for “UE offloading compute tasks to the network”, i.e. whether this refers to

  - UEs dynamically installing and executing arbitrary binaries on compute resources provided by the network or,

  - UEs using network-based application components instead of running the components on the UE.

- In coordination with SA6 and SA5, perform a gap analysis of the Edge Enabler functionality defined by SA6 and SA5 and SA2 defined 5G System Enhancements for Edge Computing, focusing on offloading compute tasks to the network (by UE and AF), potential coordination between communication and computing, service continuity and QoS.

NOTE:	SA plenary is expected to determine the work split between SA2, SA6 (and potentially SA5) to address any gaps discovered.

# >>>>END OF CHANGES<<<<