3GPP TSG-SA WG2 Meeting #171	S2-2509241

# October 13-17, 2025 Wuhan, China

Source:	Xiaomi

Title:	Interim agreement/Conclusion for KI#1 System Architecture to Support Sensing

Agenda item:	20.2.1

Document for:	Agreement

Work Item / Release: FS_Sensing_ARC/Rel-20

Abstract of the contribution: This paper proposes Interim agreement/conclusions for KI#1 System Architecture to Support Sensing

1. Discussion

TS 23.700-14 v1.0.0 has documented 12 solutions relevant to KI#1 System Architecture to Support Sensing.

This key issue will address the general system architecture and function enhancements required to provide end-to-end support for Sensing. The general architecture aspects that need to be studied include any new and/or existing functions in the Core Network and their interactions that should be involved in providing support for Sensing. Potential privacy requirements if identified will be considered for this key issue.

NOTE:	Solutions for other key issues may also impact general architecture aspects covered by this key issue.

Table 1: Mapping table for KI#1

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 | #5 | #6 |
| #1 | X |  | X | X | X |  |
| #3 | X | X | X | X | X | X |
| #4 | X |  |  |  |  |  |
| #5 | X |  |  | X |  |  |
| #6 | X | X | X |  |  |  |
| #7 | X |  |  |  |  |  |
| #19 | X | X |  |  | X |  |
| #27 | X | X |  |  | X | X |
| #28 | X |  |  |  | X |  |
| #31 | X |  |  |  | X |  |
| #34 | X |  |  | X |  | X |
| #38 | X | X |  |  |  |  |

Based on the evaluation of the relevant 12 solutions, there are following observations and proposals:

Observation 1a: Sol#1, Sol#4, Sol#5, Sol#6, Sol#7, Sol#9, Sol#11, Sol#28, Sol#34, Sol#36, Sol#38 propose one new logical network function (e.g., named SF, SSF, SeMF, etc.) or based on the assumption of on one new logical network function to support Sensing Service. The functionalities of Sensing Function are following:

Observation 1b: Sol#3, Sol#8, Sol#12, Sol#19, Sol#27, Sol#31, Sol#34 propose two new logical functions as Sensing control function, and Sensing processing function to support Sensing Service.

Proposal 1: to both include the two options for further discussion, one new logical Sensing Function in Principle 1a, and two new functions as SCF and SPF in Principle 1b.

- Principle 1a: one logical Sensing Function (SF) is defined to support for Sensing Service with following new functionalities:

- Discovery and (re-) selection of Sensing Entity(ies), e.g., based on Sensing Service Request.

- Collection of Sensing data from SE(s)

- Connection establishment/release between SE and SF for Sensing Service

- Generating Sensing Result based on the collected Sensing data and optional associated data.

- Authorization for Sensing Service Request based on the parameters provided by Sensing Service Consumer.

- Principle 1b: two logical functions are defined as Sensing Control Function (SCF) and Sensing Processing Function (SPF) to support Sensing Service:

- SCF is defined to provide the functionalities, including,

  - Discovery and (re-) selection of SE(s),

  - Interaction between SCF and SPF, e.g., Sensing processing configuration to SPF,

  - Authorization of Sensing Service Request.

- SPF is defined to provide following functionalities:

  - Collect the Sensing data from SE(s)

  - Connection establishment/release between SE(s) and SPF

  - Generate the Sensing Result and report to SCF

Proposal 2: if the Sensing Function split into two functions as SCF and SPF, the interface between SCF and SPF could be either implementation or standardized way.

- Principle 2a, If Principle 1b is supported, the interface between SCF and SPF is up to implementation.

- Principle 2b, If Principle 1b is supported, the interface between SCF and SPF is standardized.

Observation 3a: Sol#1, Sol#3, Sol#6, Sol#8, Sol#12, Sol#15, Sol#16, Sol#20, Sol#22, Sol#25, Sol#26, Sol#27, Sol#28, Sol#29, Sol#30, Sol#31, Sol#32, Sol#33, Sol#34, Sol#37 propose direct connection between SF (SCF) and SE(s) or based on the assumption of direct connection between SF(SCF) and SE(s).

Observation 3b: Sol#4, Sol#5, Sol#7, Sol#13, Sol#17, Sol#18, #24, Sol#35, Sol#36, Sol#38 propose indirect connection between SF(SPF) and SE(s) via AMF, or based on the assumption of indirect connection between SF(SCF) and SE(s) via AMF.

Proposal 3: to list both options for further discussion as captured in Principle 3a and Principle 3b.

- Principle 3a: Direct connection between SE and SF is supported to deliver the Sensing data, and/or Sensing related signallings.

- Principle 3b: Indirect connection between SE and SF is supported to deliver the Sensing data, and/or Sensing related signallings with AMF involved

Observation 4: sol#1, Sol#3, sol#6, sol#21, sol#31, sol#33, sol#34 propose new Gateway sensing function to support Sensing service Authorization, Sensing result exposure, discovery/selection for SF, etc. However, sol#3, sol#4, sol#5, #7, #8, #9, #10, #11, #12, #13, #14, #20, #21, #22, #24, #26, #27, #28, #29, #30, #32 and #36 propose to enhance NEF to support the discovery/selection of SF, and also can perform Sensing service authorization, sensing result exposure, not need to define one duplicated NF.

Proposal 4: to enhance NEF to support the following functionalities in principle 4:

- Authorization on Sensing Service Consumer (e.g., 3rd party AF);

- Performing Discovery and (Re-) Selection of Sensing Function (SF) or Sensing Control Function (SCF);

- Sensing Result and optional Sensing Contextual Information exposure to Sensing Service Consumer;

Observation 5: sol#4, #12, #26, #31 propose to store the sensing data and/or result for purposes such as enabling analytics for predictions, historical analysis, or future sensing service request.

Proposal 5: to further discuss on the storage NF is defined to support for storage of Sensing data, Sensing Result.

- Principle 5, one storage NF is defined to support for storage of Sensing data, Sensing Result.

2 Proposal

It is proposed to include the following changes in TR 23.700-14 V1.0.0.

* * * First Change * * * *

7	Interim Agreements

7.1	Agreed Principles

7.1.Y	Agreed Principles for KI#1 System Architecture to Support Sensing

To supports KI#1 System Architecture to Support Sensing, the following principles are concluded:

Principle 1a: one logical Sensing Function (SF) is defined to support for Sensing Service with following new functionalities:

- Discovery and (re-) selection of Sensing Entity(ies), e.g., based on Sensing Service Request.

- Collection of Sensing data from SE(s)

- Connection establishment/release between SE and SF for Sensing Service

- Generating Sensing Result based on the collected Sensing data and optional associated data.

- Authorization for Sensing Service Request based on the parameters provided by Sensing Service Consumer.

Principle 1b: two logical functions are defined as Sensing Control Function (SCF) and Sensing Processing Function (SPF) to support Sensing Service:

- SCF is defined to provide the functionalities, including,

  - Discovery and (re-) selection of SE(s),

  - Interaction between SCF and SPF, e.g., Sensing processing configuration to SPF,

  - Authorization of Sensing Service Request.

- SPF is defined to provide following functionalities:

  - Collect the Sensing data from SE(s)

  - Connection establishment/release between SE(s) and SPF

  - Generate the Sensing Result and report to SCF

Principle 2a, If Principle 1b is supported, the interface between SCF and SPF is up to implementation.

Principle 2b, If Principle 1b is supported, the interface between SCF and SPF is standardized.

Principle 3a: Direct connection between SE and SF is supported to deliver the Sensing data, and/or Sensing related signallings.

Principle 3b: Indirect connection between SE and SF is supported to deliver the Sensing data, and/or Sensing related signallings with AMF involved

Editor’s Note X1: Direct or indirect connection between SE (i.e., gNB) and SF needs further coordination with RANs.

Principle 4, enhanced NEF are supported to provide following functionalities:

- Authorization on Sensing Service Consumer (e.g., 3rd party AF);

- Performing Discovery and (Re-) Selection of Sensing Function (SF) or Sensing Control Function (SCF);

- Sensing Result and optional Sensing Contextual Information exposure to Sensing Service Consumer;

Principle 5, one storage NF is defined to support for storage of Sensing data, Sensing Result.

Principle 6, Sensing Entity (RAN node) may have the capability of processing the sensing data, and generate the Sensing Result.

Editor’s Note X2: whether RAN node have the capability of processing the sensing data or generate sensing result requires further coordination with RAN

Editor’s Note X3: the format and parameters defined for Sensing data collected from SE (i.e., gNB) will need further coordination with RAN WGs.

Editor’s Note X4: the detailed descriptions of the functionalities above (i.e., SF, SCF, SPF, NEF) are defined and will align with the final conclusions of other corresponding KIs.

* * * End of Changes * * * *