SA WG2 Meeting #171	S2-2509288

# October 13 - 17, 2025, Wuhan, China		       (revision of S2-2509172)

Source: 	NTT DOCOMO

Title: 	KI#1: New Sol: Non-IP transfer of voice media with a reduced RTP header for 1 PDN connection and CP/UP scenario

Document for: 	Approval

Agenda Item: 	20.1.1 (FS_5GSAT_Ph4_ARC)

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: This contribution proposes a new solution for KI#1.

1	Introduction

This contribution proposes a solution for KI#1 on IMS voice service over GEO satellite, in particular addressing SIP/I1 message size optimization and voice media size optimization.

This solution assumes the use of a CP bearer for SIP/I1 messages and a UP bearer for voice media in 1 PDN connection.

With this solution, SIP/I1 messages can be transferred over GEO satellite relying on the NAS security (NOTE: 5tuple removal is not proposed), and voice media can be transferred over GEO satellite without 5tuple and the RTP header and with a reduced RTP header.

2	Proposal

It is proposed to add the following text to TR 23.700-19 v1.0.0.

********** First Change **********

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |
| --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 |
| #X | X |  |  |  |

********** Next Change (All new text) **********

6.x	Solution #x: Non-IP transfer of voice media with a reduced RTP header for 1 PDN connection and CP/UP scenario

6.x.0	High-level solution Principles

This solution is for KI#1 on IMS voice service over GEO satellite, in particular addressing SIP/I1 message size optimization and voice media size optimization.

This solution assumes the use of a CP bearer for SIP/I1 messages and a UP bearer for voice media in 1 PDN connection.

Editor's note: Potential impacts due to coexistence of SRB(=CP bearer) and DRB(=UP bearer) need to be clarified.

With this solution, SIP/I1 messages can be transferred over GEO satellite relying on the NAS security (NOTE: 5tuple removal is not proposed), and voice media can be transferred over GEO satellite without 5tuple and the RTP header and with a reduced RTP header.

6.x.1	Description

6.x.1.1	General

The assumptions of this solution are as follows:

- It is assumed that signaling (e.g., SIP messages, I1-like messages) and voice media are transferred in different bearers in 1 PDN connection.

- It is assumed to transfer SIP/I1 signaling in NAS by using a CP bearer, which is a default bearer.

  - Usually, the transport mode IPsec ESP is used between UE and P-CSCF (See TS 33.203). It is assumed not to use it for GEO voice.

  - Security of SIP/I1 signaling relies on that of NAS.

Editor's note: Whether this security assumption is appropriate or not is left to SA3.

- It is assumed to transfer voice media by using a UP bearer, which is a dedicated bearer.

  - Due to the constraints of NB IoT, Non-GBR is assumed.

Editor's note: Potential impacts due to coexistence of SRB(=CP bearer) and DRB(=UP bearer) need to be clarified.

The principles of this solution are as follows:

- PDN type of the PDN connection is set to IP. Voice media between UE and P-GW is without 5tuple and the RTP header and with a reduced RTP header (i.e., one octet sequence number).

- ROHC is deactivated in UE and eNB for the UP bearer.

[Option 1] P-GW and the non-MMTEL part of UE perform the RTP header handling.

- UE is considered to have two parts. One part is the MMTEL part that is basically not impacted due to this solution (apart from sharing info derived from SDP to the non-MMTEL part in an appropriate timing) and that constructs SIP/I1 signaling and voice media with headers as usual. The other part is the non-MMTEL part that is impacted.

NOTE:	The non-MMTEL part of UE is located between the MMTEL part of UE and the PDCP layer, and how it works is up to the UE implementation.

- For voice media, P-GW and the non-MMTEL part of UE uses Flow description and RTP description to derive 5tuple and the RTP header to be added.

[Option 2] IMS AGW and the MMTEL part of UE perform the RTP header handling.

Editor's note: FFS of the mechanism that IMS AGW, instead of P-GW, and the MMTEL part of UE, instead of the non-MMTEL part of UE, perform the RTP header handling (i.e., exchange between the usual RTP header and the reduced RTP header).

The advantages of this solution are as follows:

- SIP/I1 signaling can be transferred over GEO satellite while relying on the NAS security. By relying on the NAS security, security in the level of SIP/I1 signaling can be skipped, and IPsec ESP header, trailer, and ICV(=Integrity Check Value) can be omitted.

Editor's note: Whether this security assumption is appropriate or not is left to SA3.

- Voice media can be transferred over GEO satellite without 5tuple and the RTP header, while a reduced RTP header is added instead.

6.x.1.2	Derivation of 5tuple and the RTP header for voice media in P-GW and UE

6.x.1.2.1	Option 1: P-GW and the non-MMTEL part of UE performs the RTP header handling

P-CSCF obtains 5tuples of UL/DL and RTP description (i.e, a=rtpmap, a=ptime) of UL during SDP offer/answer exchange and sends them via PCRF to P-GW at the timing of setting up a dedicated bearer. P-GW uses this information to reconstruct the RTP header and 5tuple for UL and to find a bearer for DL.

The MMTEL part of UE obtains 5tuples of UL/DL and RTP description (i.e, a=rtpmap, a=ptime) of DL during SDP offer/answer exchange and sends them to the non-MMTEL part of UE. The non-MMTEL part of UE uses this information to reconstruct the RTP header and 5tuple for DL voice media to be forwarded to the MMTEL part of UE, and to update TFT and find a bearer for UL voice media coming from the MMTEL part of UE.

6.x.1.3	Derivation of RTP header and reduced RTP header in detail

Editor's note: Confirmation from SA4 is needed for the RTP header handling.

The format of the RTP header is as follows according to IETF RFC 3550.

An example of a=rtpmap and a=ptime is as follows according to TS 26.1144, Table A.14.1.

- a=rtpmap:97 EVS/16000/1

- a=ptime:20

The following is a method to construct a RTP header from a reduced RTP header, which is one octet sequence number.

- P(padding) = 0 (This value needs to be determined beforehand in the network.)

- X(extension) = 0

- CC(CSRC count) = 0

- M(marker) = 0

- PT(payload type) = 97

- sequence number = 0 is set as padding in the third octet and the one octet sequence number is set in the fourth octet.

- timestamp =For the initial value of the sequence number, set 1000 for example. For the initial value of the sequence number+1, set 1000 + 320 (=16000 x 0.02) = 1320. For the initial value of the sequence number+2, set 1320 + 320 = 1640

- SSRC = P-GW decides

- CSRC = Not needed

A reduced RTP header is derived from a RTP header by taking out the least significant one octet from the sequence number.

6.x.1.4	Voice media packet handling in P-GW after the context for the dedicated bearer is set up

6.x.1.4.1	Option 1: P-GW performs the RTP header handling

When P-GW receives a DL voice media packet with IP/UDP/RTP, the P-GW recognizes the DL voice media packet is to be sent in the dedicated bearer. The P-GW removes IP/UDP header, replaces the RTP header with a reduced RTP header, and sends the resulting packet in the dedicated bearer.

When P-GW receives a UL voice media packet without IP/UDP/RTP and with a RTP reduced header in the dedicated bearer, the P-GW finds the context for the dedicated bearer, which contains information to derive the RTP header and information on IP/UDP header to be added. The P-GW derives the RTP header based on the reduced RTP header, replaces the reduced RTP header with the RTP header, adds IP/UDP header, and sends a resulting packet to IMS AGW.

6.x.1.5	Voice media packet handling in UE after the context for the dedicated bearer is set up

6.x.1.5.1	Option 1: The non-MMTEL part of UE performs the RTP header handling

The MMTEL part of UE creates a UL voice media packet with IP/UDP/RTP as usual and forwards it to the non-MMTEL part of UE. The non-MMTEL part of UE uses TFT and recognizes the UL voice media packet is to be sent in the dedicated bearer. The non-MMTEL part of UE removes IP/UDP header, replaces the RTP header with a reduced RTP header, and sends the resulting packet in the dedicated bearer.

When the non-MMTEL part of UE receives a DL voice media packet without IP/UDP/RTP and with a RTP reduced header in the dedicated bearer, the non-MMTEL part of UE finds the context for the dedicated bearer, which contains information to derive the RTP header and information on IP/UDP header to be added. The non-MMTEL part of UE derives the RTP header based on the reduced RTP header, replaces the reduced RTP header with the RTP header, adds IP/UDP header, and sends a resulting packet to the MMTEL part of UE.

6.x.2	Procedures

6.x.2.1	Attach procedure

Figure 6.x.2.1-1: Attach procedure

1.	UE sends to MME the Attach request containing PDN type=IPv6, Header compression configuration=No Compression, and Extended protocol configuration options that have "CN header handling request" (a new indicator).

2.	MME sends to S-GW the Create Session Request containing PDN type=IPv6 and Extended protocol configuration options that have "CN header handling request".

Editor's note: FFS on whether any new IE is needed (or APN is sufficient) to assist MME to select P-GW capable of accepting "CN header handling request".

3.	S-GW sends to P-GW the Create Session Request containing PDN type=IPv6 and Extended protocol configuration options that have "CN header handling request".

4.	P-GW sends to PCRF the CC-Request.

5.	PCRF sends to P-GW the CC-Answer.

6.	P-GW recognizes with "CN header handling request" or with another indication that P-GW needs to establish a dedicated bearer as well. As per the procedure defined in Annex F of TS 23.401, P-GW sends to S-GW the Create Session Response containing PDN Address Allocation that includes the allocated IPv6 address and Extended protocol configuration options that include "CN header handling accepted" (a new indicator) and the Create Bearer Request containing TFT(Create new TFT, parameters list is not included).

7.	S-GW sends to MME the Create Session Response containing PDN Address Allocation that includes the allocated IPv6 address and Extended protocol configuration options that include "CN header handling accepted" and the Create Bearer Request containing TFT(Create new TFT, parameters list is not included).

8.	MME sends to eNB the Initial Context Setup request containing the Attach accept and the E-RAB to Be Setup Item IEs. This Attach accept contains Activate Default EPS Bearer Context Request containing PDN address that includes the allocated IPv6 address, Header compression configuration=No Compression, and Extended protocol configuration options that include "CN header handling accepted". This E-RAB to Be Setup Item IEs contains Bearer Type = non IP and the Activate Dedicated EPS Bearer Context Request containing TFT(Create new TFT, parameters list is not included). Bearer Type = non IP is included to stop the use of ROHC in eNB for the dedicated bearer.

9.	eNB sends to UE the RRCConnectionReconfiguration containing the Attach accept and the Activate Dedicated EPS Bearer Context Request.

10.	The Attach procedure continues.

6.x.2.2	Voice call procedure

6.x.2.2.1	Option 1: P-GW and the non-MMTEL part of UE performs the RTP header handling

Figure 6.x.2.2.1-1: Voice call procedure

1.	UE/MMTEL, which is the MMTEL part of the UE, decides to make a call.

2.	UE/MMTEL sends SIP/I1(SDP offer) to UE/non-MMTEL, which is the non-MMTEL part of the UE. "SIP/I1" refers here to either SIP INVITE or an I1-like message equivalent to it.

3.	UE/non-MMTEL sends towards P-CSCF the SIP/I1(SDP offer).

4.	P-CSCF may translate I1(SDP offer) to SIP INVITE (SDP offer). P-CSCF sends the SIP INVITE (SDP offer) to S-CSCF.

5.	S-CSCF sends SIP 183 Session Progress (SDP answer) to P-CSCF.

6.	P-CSCF sends to PCRF the AA-Request containing Flow description and RTP description (a new information). This RTP description includes information related to a=rtpmap and a=ptime in SDP for UL voice media.

7.	PCRF sends the AA-Answer to P-CSCF.

8.	PCRF sends to P-GW the RA-Request containing Flow description and RTP description.

9.	P-GW sends the RA-Answer to PCRF.

10.	P-GW recognizes that a dedicated bearer is already established and does not send any message to S-GW. P-GW stores Flow description and RTP description as a context of the dedicated bearer.

11.	P-CSCF may translate SIP 183 Session Progress (SDP answer) to another SIP message or an I1-like message containing the SDP answer. "SIP/I1" refers here to one of those messages. P-CSCF sends the SIP/I1(SDP answer) towards UE.

12.	UE/non-MMTEL sends the SIP/I1(SDP answer) to UE/MMTEL.

13.	UE/MMTEL informs UE/non-MMTEL of Flow description and RTP description.

14.	UE/non-MMTEL stores Flow description and RTP description as a context of the dedicated bearer and creates TFT for the dedicated bearer.

15.	The procedure continues, and the voice call is established.

16.	UE/MMTEL decides to send a voice media packet.

17.	UE/MMTEL sends a voice media packet to UE/non-MMTEL.

18.	UE/non-MMTEL finds a bearer, which tuns out to be the dedicated bearer, removes 5tuple, and replaces the RTP header with the reduced RTP header as is described in clause 6.x.1.3.

19.	UE/non-MMTEL sends the voice media packet without 5tuple and RTP header and with the reduced RTP header by using U-plane via eNB and S-GW to P-GW. ROHC in UE and eNB is deactivated.

20.	P-GW replaces the reduced RTP header with the RTP header as is described in clause 6.x.1.3 and adds 5tuple as is described in clause 6.x.1.2.

21.	P-GW sends the voice media packet to IMS AGW.

22.	IMS AGW sends a voice media packet to P-GW.

23.	P-GW finds a bearer, which turns out to be the dedicated bearer, removes 5tuple, and replaces RTP header with the reduced RTP header as is described in clause 6.x.1.3.

24.	P-GW sends the voice media packet without 5tuple and RTP header and with the reduced RTP header by using U-plane via S-GW and eNB to UE. ROHC in UE and eNB is deactivated.

25.	UE/non-MMTEL replaces the reduced RTP header with the RTP header as is described in clause 6.x.1.3 and adds 5tuple as is described in clause 6.x.1.2.

26.	UE/non-MMTEL sends the voice media packet to UE/MMTEL.

27.	UE/MMTEL processes the voice media packet.

6.x.3	Impacts to Services, Entities and Interfaces

UE, for Voice media packet handling:

- Handling of new indicators "CN header handling request" and "CN header handling accepted".

- Handling of 5tuple and RTP header (i.e., removal and addition of 5tuple, transformation between RTP header and reduced RTP header) by deriving a new information "RTP description” and Flow description from SDP and using them.

- Derivation of TFT based on Flow description.

- Deactivation of ROHC when PDN type=IP.

MME, for Voice media packet handling:

- Request to eNB to deactivate ROHC while PDN type=IP.

P-GW, for Voice media packet handling:

- Handling of new indicators "CN header handling request" and "CN header handling accepted"

- Handling of 5tuple and RTP header (i.e., removal and addition of 5tuple, transformation between RTP header and reduced RTP header) by receiving a new information "RTP description” from PCRF and using it and Flow description.

PCRF, for Voice media packet handling:

- Forwarding a new information "RTP description” to P-GW.

P-CSCF, for Voice media packet handling:

- Deriving and sending a new information "RTP description” to PCRF.

********** End of Changes **********