3GPP TSG-WG SA2 Meeting #171	S2-2509306

# Wuhan, China, October 13 – 17, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1491 | rev | 2 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Further corrections on QoS and policy assistance analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
|  | 5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SDF template, as defined in clause 6.3.1 of TS 23.503, contains application ID or service data flow filters (i.e. Packet Filter Set). In the procedure, there is a wrong description of consumer NF being untrusted AF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify "SDF template" for input parameters in the analytics request or subscription and for output analytics. Remove the wrong description of consumer NF in the procedure. Editorial modifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear/incorrect descriptions on QoS and policy assistance analytics. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.23.1, 6.23.3, 6.23.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * *Start of Changes * * *

6.23.1	General

Clause 6.23 describes how NWDAF can provide QoS and policy assistance predictions, based on the consumer request. The consumer can either subscribe to analytics notifications (i.e. a Subscribe-Notify model) or request a single notification (i.e. using a Request-Response model).

The service consumer may be a PCF.

The QoS and policy assistance analytics provides predictions of the QoE associated to a set of QoS parameters. When the service consumer needs assistance information to determine QoS or policy, it may send an Nnwdaf_AnalyticsInfo_Request or Nnwdaf_AnalyticsSubscription_Subscribe request to the NWDAF with one or multiple sets of QoS parameters and optionally a requested QoE. The NWDAF provides the candidate set(s) of QoS parameters and the corresponding predicted QoE that can be achieved by applying the candidate QoS parameters set(s).

The candidate QoS parameters set(s) provided by the NWDAF are within the sets of QoS parameters provided by the consumer (i.e. PCF). The NWDAF shall only include the parameter(s) and the corresponding value(s) that are provided by the consumer request (i.e. PCF) in the QoS parameter set(s).

The QoS and policy assistance analytic may be provided for an individual UE or a group of UEs (e.g. the QoE is for the application ID(s) associated to one or more QoS flow(s) of the UE(s)), or for an application (e.g. the QoE is for the service flow(s) of the application ID).

The consumer of these analytics indicates the following information in the request or subscription:

-	Analytics ID = " QoS and Policy Assistance".

-	Target of Analytics Reporting as defined in clause 6.1.3.

-	A list of one or more QoS parameter set(s) and optionally associated QoS parameter set ID(s). Based on the consumer decision (i.e. PCF), each QoS parameter set(s) may include the following individual parameters and non-empty value list(s) for each individual parameter(s):

-	5QI (standardized or pre-configured); or

-	QoS characteristics attributes (as defined in clause 5.7.3 of TS 23.501 [2]): Resource Type, Priority Level, PDB, PER, Averaging Window, Maximum Data Burst Volume.

And optionally for GBR 5QI(s):

-	GBR and MBR.

NOTE 1:	In the case of standardised and pre-configured 5QI(s), the specified/configured QoS characteristics (e.g. as defined in Table 5.7.4-1 of TS 23.501 [2]) can be pre-configured into NWDAF by operators, subject to operator policy. Therefore, the NWDAF is able to associate the QoS characteristics to the given 5QI in consumer request to derive the predicted QoE.

NOTE 2:	QoS characteristics can be included in the QoS parameter set(s) only in the case of dynamically assigned non-standardized 5QI(s).

-	Application ID or Packet Filter set.

-	Optionally, a requested QoE that indicates the NWDAF to only provide the QoS parameter set(s) and their corresponding value(s) for which the predicted QoE is equal to, or higher than, the requested QoE.

-	Analytics Filter Information optionally includes:

-	DNN;

-	Area of Interest (AOI(s));

NOTE 3:	A service consumer can use the AOI(s) in order to reduce the amount of signalling that the analytics subscription or request generates.

-	S-NSSAI and NSI ID;

-	List of analytics subsets that are requested, (see clause 6.23.3), e.g. parameter(s) in QoS parameter set(s).

-	RAT Type.

-	Frequency.

-	DNAI (for analytics with an edge application server).

-	An Analytics target period indicates the time period over which the analytics are requested.

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

-	Optionally, preferred order of results for the list of candidate QoS parameter sets and the predicted QoE associated to each candidate QoS parameter sets:

-	ordering criterion: "QoE" (i.e. QoE that is associated to candidate QoS parameter set) or "usage duration" or "number of usages" of a QoS flow,

-	order: ascending or descending.

-	Optionally, reporting Thresholds, which apply only for subscriptions and indicate conditions on the levels to be reached for the reporting of respective analytics subsets (see clause 6.23.3).

-	Optionally, maximum number of objects and maximum number of SUPIs.

-	Optionally, preferred granularity of location information: TA level or cell level or "longitude and latitude level".

* * * Next Change * * *

6.23.3	Output Analytics

The NWDAF shall be able to provide predictions on QoS and Policy Assistance to consumer NFs, i.e. PCF, as defined in Table 6.23.3-1.

Table 6.23.3-1: QoS and Policy Assistance predictions

| Information | Description |
| --- | --- |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > QoS and Policy Assistance information (1…max) | List of QoS and Policy Assistance information. Max. is the number of the candidate QoS parameter set(s), if applicable. |
| >> QoS parameter set identifier (NOTE 6) | Identifies the QoS parameter set for which the entry applies to. |
| >> SDF template (i.e. Application ID or Packet Filter Set) | Identifies the service data flow for which the predicted QoE and the candidate QoS parameter set are provided. |
| >> Duration for the Application | Duration for the corresponding application ID. |
| >> Predicted QoE (NOTE 6) | The predicted QoE of the application when the corresponding QoS parameter set is applied, e.g. average, variance, maximum, minimum of QoE. |
| >> QoS parameter values (NOTE 4) | The parameters in the QoS parameter set and their corresponding values associated to the predicted QoE. |
| >>> 5QI | The reference to 5G QoS characteristics and QoS parameters. |
| >>> QoS characteristics attributes |  |
| >>>> Priority level | The Priority Level associated with 5G QoS characteristics, as defined in clause 5.7.3.3 of TS 23.501 [2]. |
| >>>> Resource type | The resource type of the corresponding QoS flow, e.g. GBR QoS flow, non-GBR QoS flow, delay-critical GBR QoS flow. |
| >>>> Packet Delay Budget | Packet Delay Budget (PDB) indicates the upper bound for the time that a packet may be delayed between the UE and the N6 termination point at the UPF, as defined in TS 23.501 [2]. |
| >>>> Packet Error Rate | Packet Error Rate (PER) defines an upper bound for a rate of non-congestion related packet losses, as defined in TS 23.501 [2]. |
| >>>> Averaging Window (NOTE 2) | Represents the duration over which the guaranteed and maximum bitrate shall be calculated. |
| >>>> Maximum Data Burst Volume (NOTE 3) | The MDBV denotes the largest amount of data that the 5G-AN is required to serve within a period of 5G-AN PDB, as defined in TS 23.501 [2]. |
| >>> Bit Rates (NOTE 2) | The bit rates are only applied to GBR QoS flow identified by the SDF template. |
| >>>> GBR | Guaranteed Bit Rate (GBR) for UL and/or DL. |
| >>>> MBR | Maximum Bit Rate (MBR) for UL and/or DL. |
| >>> Maximum Packet Loss Rate (NOTE 2) | The maximum rate (UL, DL) for lost packets that can be tolerated for the service data flow. |
| >> Applicable time window of the QoS and Policy Assistance information | The applicable time window of the QoS and Policy Assistance information within the validity period. |
| >> RAT Type | Indicating the list of RAT type(s) for which the QoS and Policy Assistance Information applies. |
| >> Frequency | Indicating the list of carrier frequency value(s) for which the QoS and Policy Assistance Information applies. |
| >> Validity period | The validity period within the time slot for the QoS and Policy Assistance information analytics as defined in clause 6.1.3. |
| >> Spatial validity | Area where the QoS and Policy Assistance information analytics applies. |
| >> (Optional) Usage Duration information (NOTE 5) | Maximum/Minimum/Average usage duration of the QoS Flows associated to candidate QoS parameter set. |
| >> (Optional) Number of usage (NOTE 5) | The number of times that the QoS Flows associated to candidate QoS parameter set to be used. |
| > Confidence | Confidence of this prediction. |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested" and "Reporting Thresholds". NOTE 2: The output analytics only applies to GBR QoS Flow identified by the SDF template. NOTE 3: The output analytics only applies to Delay-critical GBR QoS Flow identified by the SDF template. NOTE 4: Only the parameters that have more than one candidate values provided by the consumer shall be included in the NWDAF output and only the values provided by the consumer are allowed. NOTE 5: This parameter is included in analytics output if the Target of Analytics Reporting is a specific UE or Analytics Filter contains specific Application ID. It is determined by NWDAF using the SMF QFI allocation and deallocation events. For example, the duration equals to the timestamps difference between the QoS Flow change(s) (e.g. QFI allocation, change, and deallocation); the usage number counts the number of times the application traffic is allocated to the QoS Flow. NOTE 6: The predicted QoE is reported for the entire QoS parameter set (together with QoS parameter set identifier) if only one parameter value has been provided for each parameter in the set. Otherwise separate predicted QoE for each combination of different parameter values are provided. |  |

6.23.4	Procedures

Figure 6.23.4 shows the procedure for deriving and providing the QoS and Policy Assistance analytics to a consumer 5GC NF (i.e. PCF).

Figure 6.23.4-1: Procedure for QoS and Policy Assistance Analytics

1.	The Consumer NF, i.e. PCF, requests or subscribes to QoS and Policy Assistance analytics from NWDAF and provides the request input information as specified in clause 6.23.1 to NWDAF.

2a.	The NWDAF may subscribe to the service data from 5GC NFs, e.g. AMF, SMF, UPF, etc. as defined in clause 6.23.2 by invoking the corresponding NF event exposure service operations, e.g. using Namf_EventExposure_Subscribe service operation for collecting UE location(s); using Nsmf_EventExposure_Subscribe service operation (Event ID, parameters in QoS parameter set, SUPI(s) or Application ID) for collecting the data related to QoS parameter set, etc., as defined in TS 23.502 [3].

If the required UE location information is finer granularity than TA/cell level, then NWDAF collects the location data from GMLC instead of AMF by invoking the Ngmlc_Location service as defined in TS 23.273 [39] and TS 29.515 [48].

2b.	For average UL/DL packet delay analysis between UE and PSA_UPF, the SMF subscribes to QoS Monitoring information from UPF using the N4 Session level Reporting procedure defined in TS 23.502 [3]. The UPF notifies the subscribed event report directly to the NWDAF.

2c.	The NWDAF may collect data from the OAM following the procedure captured in clause 6.2.3.2.

2d.	The NWDAF may subscribe to the service data from AF as defined in Table 6.4.2-1 by invoking Nnef_EventExposure_Subscribe or Naf_EventExposure_Subscribe service (Event ID = QoS and Policy Assistance, Application ID, Event Filter information, Target of Event Reporting = UE ID(s)) as defined in TS 23.502 [3].

3.	The NWDAF derives the requested analytics on QoS and Policy Assistance based on NWDAF internal logic and based on operator policies, e.g. the NWDAF derives the analytics directly based on the input data defined in clause 6.23.2 or the NWDAF may derive the analytics by consuming the output analytics of Observed Service Experience analytics and the input parameters in Table 6.23.2-1.

NOTE:	When requested by the NF consumer (i.e. PCF), the NWDAF may also collect data from OAM as described in clause 6.6.4 and clause 6.9.4 for the purpose of deriving NW performance and QoS sustainability information associated to the QoS parameters set(s).

4.	The NWDAF provides the requested QoS and Policy Assistance to the consumer NF, using either Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service operation used in step 1.

5-7.	If the consumer NF subscribes to QoS and Policy Assistance Analytics in step 1, once the NWDAF generates new analytics on QoS and Policy Assistance, it provides the new analytics via Nnwdaf_AnalyticsSubscription_Notify to the Consumer NF.

* * *End of Changes * * *