3GPP TSG-WG2 Meeting #171	S2-2509333

# Wuhan, China, October 13 – 17, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6194 | rev | 9 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Indication of Rate-Adaptable QoS Flows to NG-RAN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, LG Electronics, CATT, Xiaomi, Lenovo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_Ph3-Core |  |  |  |  |  | Date: |  |  | 03-10-2025 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In R2-2501347, RAN2 requested that for XR rate control, the gNB receives QoS flow information from the CN, specifying which QoS flows are subject to uplink rate control. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify that 5GC may provide an indication of QoS flows that are subject to RAN based bitrate recommendation to NG-RAN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Indication from CN on which UL flows are rate adaptable is not available to RAN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.37.1, 5.37.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | x |  | Other core specifications |  |  |  | TS 23.503, CR#1531 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The clause reference 6.1.3.27.X in 5.37.X to 23.503 to be updated from CR#1531 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** First Change ****

### 5.37.1	General

This clause provides an overview of 5GS functionalities for support of XR services (AR/VR applications) and interactive media services that require high data rate and low latency communication, e.g. cloud gaming and tactile/multi-modal communication services according to service requirements documented in TS 22.261 [2]. The standardized 5QI characteristics for such interactive services are provided in Table 5.7.4-1 and TSCAI is used to describe the related traffic characteristics as defined in clause 5.27.2. Further enhancements for these interactive media services are as follows:

-	The 5GS may support QoS policy control for multi-modal traffic, see clause 5.37.2.

-	The 5GS may support network information exposure which can be based on ECN markings for L4S, see clause 5.37.3 or 5GS exposure API, see clause 5.37.4.

-	The 5GS may support PDU Set based handling including PDU Set identification and marking, see clause 5.37.5.

-	The 5GS may ensure that the UL and DL packets together meet the requested round trip delay and also update the delay for UL and DL considering QoS monitoring results, see clause 5.37.6.

-	The 5GS may perform per-flow Packet Delay Variation (PDV) monitoring and policy control according to AF provided requirements, see clause 5.37.7.

-	The 5GC may provide traffic assistance information to the NG-RAN to enable Connected mode DRX power saving, see clause 5.37.8.

-	The 5GS may consider dynamically changed traffic characteristics for better resource management, see clause 5.37.10.

-	The 5GS may support traffic identification for multiplexed media flows in the same transport layer connection, see clause 5.37.11.

-	The 5GC may perform PDU Set Importance based transport level packet marking, see clause 5.8.2.7.

-	The 5GC may provide the Multi-modal Service ID to NG-RAN, see clause 5.37.2.

-	The 5GC may provide an RAN-Controlled UL Bitrate Recommendation indication to the NG-RAN for a QoS Flow, see clause 5.37.X.

NOTE:	Home-routed roaming deployments cannot always support the low latency communication requirements of XR services or interactive media services.

**** Second Change ****

### 5.37.X	Support for RAN-controlled UL bitrate recommendation

To enable bitrate recommendation for the UL by NG-RAN (defined as UL Rate Control in TS 38.300 [27]), the AF may provide, within the AF session with required QoS procedure in clauses 4.15.6.6 and 4.15.6.6a of TS 23.502 [3], a RAN-controlled UL Bitrate Recommendation indication, when a media flow can adapt to bitrate recommendation received from RAN. The PCF, based on the AF input and operator policies, may include this indication within the PCC rule as defined in clause 6.1.3.27.X of TS 23.503[45] and the SMF forwards the indication to NG-RAN in N2 SM information when establishing and/or updating the corresponding QoS Flow.

NOTE 1: How the Application makes use of the RAN-controlled UL Bitrate Recommendation is outside the scope of this specification.

NOTE 2: RAN-controlled UL bitrate recommendation does not affect the QoS forwarding treatment of a QoS flow.

NOTE 3:  It is recommended that RAN-controlled UL bitrate recommendation is used for QoS Flow(s) with single media flow as described in clause 6.1.3.2.4 of TS 23.503 [45].

**** End of Changes ****