3GPP SA WG2#171	S2-2509443

Wuhan, China, 13-17 October, 2025	(Revision of S2-2509403, S2-2508487)

Source:	OPPO

Title:	Interim Agreement for KI#1

Spec:	3GPP TR 23.700-30

Agenda item:	20.5.1

Work Item / Release:	FS_AmbientIoT_Ph2_ARC / Rel-20

Document for:	Approval

Abstract of the contribution: This contribution is to propose the interim agreement for KI#1.

1. Introduction

The following is exerpted from TR 23.700-13

e.

In this Release, normative work will take place for Topology 1 and no normative work will take place for Topology 2.

The following aspects common for Topology 1 and Topology 2 are concluded as principles for normative work:

1.	A new core network function is introduced to support Ambient IoT (e.g. AIOTF) service for both the topology 1 and topology 2. The AIoTF performs the following functionality.

a.	The AIOTF registers itself in the NRF with its NF profile, this is to enable the discovery of AIOTF instances e.g. by an NEF. The NF profile at least includes the AIOTF ID/address, NF type, information used to allow the NEF to discover AIOT instances, e.g. based on the target area information in AF request. Other details of the NF profile, if needed, will be completed in normative phase. This part was already captured in TS 23.369.

b.	For topology 1, the AIOTF selects AIOT RAN nodes and optionally a list of the BS readers. For topology 2, the AIOTF selects the UE readers (e.g. candidate or final UE readers) and provides the selected UE Reader list to the RAN. This part will be part of the UE reader selection.

NOTE 1:	Providing the UE Reader list to the RAN only applies to the RRC-based option.

For topology 1, NGAP is terminated at an AIOT RAN node. An AIOT RAN node that supports one or more BS readers, may report its supported BS reader information e.g. BS reader IDs and their corresponding service areas. The OAM may configure the AIOTF with BS Reader information including e.g. their corresponding service area when the BS Readers are integrated in the network.

NOTE 2:	Whether RAN3 will specify BS reader information (e.g. BS reader ID, service area) reporting to AIOTF over NGAP depends on RAN3 decision and if so SA WG2 may include this feature in SA WG2 specifications.

NOTE 3:	If the request from the AIOTF to the AIOT RAN does not include a BS reader list, then AIOT RAN can use all available BS readers.

NOTE 4:	The BS reader’s service area needs to coordinate with RAN WG(s).

c.	The AIOTF receives an AIoT service request from the AF and triggers the BS/UE Readers to perform AIoT service operations towards the AIoT Devices(s). This part will be updated the Inventory procedure and command procedure.

d.	The AIOTF aggregates the service operation results (including the removal of the duplicated devices records) from BS Readers and UE Readers and sends to AF. This part will be updated the Inventory procedure and command procedure.

e.	The AIOTF may provide the following assistance information to AIoT RAN/UE Reader:

-	AIoT service type (e.g. Inventory, Command);

-	approximate number of AIoT devices based on AF request;

-	approximate D2R message size based on AF request.

NOTE 5:	If there are multiple Readers selected for the AIoT Service, the AIOTF may provide the approximate number of AIoT devices to each Reader based on implementation.

NOTE 6:	The approximate D2R message size considering the overhead of AIoT Device NAS layer will be determined later in cooperation with CT WG1 and SA WG3.

NOTE 7:	Further assistance information can be added during the normative phase and in cooperation with other WGs if necessary.

Editor's note:	For RRC based solution of topology 2, whether the aggregation can be performed by the RAN is FFS and coordination with RAN WGs is needed.

This part will be updated in the Inventory procedure and command procedure.

f.	The AIOTF may provide the AIoT aggregation assistance information to the AIoT RAN. Aggregation assistance information is used to assist in the aggregation of AIoT responses received from multiple AIoT devices. The AIOTF may determine the aggregation assistance information based on operator policy, considering information provided from the AF, which consists of:

-	Time interval: The fixed time interval for which AIoT RAN collects AIoT Device responses before AIoT RAN reports the aggregated AIoT responses to AIOTF. This may potentially happen multiple times until the AIoT RAN completes the request operation. If the AF has provided a time interval, then the AIOTF should signal a time interval to AIOT RAN that is equal or shorter than the time interval received from the AF.

NOTE 8:	Further details how to use the AIoT aggregation assistance information are subject to RAN3 decision.

NOTE 9:	The AIoT responses aggregated from the AIoT devices share the same correlation identifier.

NOTE 10:	AIOTF is expected to receive an end indication from AIoT RAN, signaling the completion of reporting for a specific correlation ID.

NOTE 11:	If the AIOTF does not provide aggregation assistance information, the aggregation process in the AIoT RAN is determined by implementation. As a result, the interpretation of  gated data by the AIOTF or AF also depends on individual implementation choices.

g.	When the AIOTF sends an operation request to a UE Reader or BS Reader (via AIOT RAN), a response and one or more reports with the results of the AIoT service operation is returned to the AIOTF with the results of the AIoT service operation, and the AIOTF needs to correlate the results to a given operation request. The AMF (if used to route the requests) additionally provides an AIOTF identifier with the request from the AIOTF which is returned with the response(s) related to the request, so the AMF can be routed back the requesting AIOTF.

This part will be updated in the Inventory procedure and command procedure.

2.	A Permanent AIoT Device ID is stored in the AIoT Device and the network or a Credential Holder's AAA server. The AIOTF checks whether the AIoT Device ID from AIoT Device has a subscription and retrieves. This part was already captured in TS 23.369.

Editor's note:	How the AIoT Device ID privacy protection and including ID authentication is done will be concluded by SA WG3. SA2 alignment with SA WG3 TR conclusions for AIoT Device security including AIoT Device ID privacy protection is FFS..

3.	The AIoT Device does not distinguish whether the connectivity topology is Topology 1 or Topology 2, nor the transport used by the AIoT Reader.This was already captured in assumption in TR23.700-13.

NOTE 12:	The AIoT device is also agnostic to the potential different architectures if more than one architecture is concluded for both the topology 1 and topology 2.

4.	AIoT Device NAS protocol is supported between the AIoT Device and the AIOTF. The AIoT Device NAS layer supports Inventory Response and Command (e.g. Read and Write) Request and Response. This part was already captured in TS 23.369.

5.	The AIOTF may store and manage the AIoT device related information (also known as device context information) locally that includes e.g. the AIOT device permanent ID, the last known reader information of the AIoT device. The last known reader information can be used to support the AIOTF to select the serving reader to forward the message towards the specific AIoT device(s). This part will be updated taking into UE reader account.

NOTE 13:	The storage of security related information at the AIOTF is up to the coordination with SA WG3 later.

#### 8.1.3.1	General

-	Subscription aspects:

-	The UE subscription in the UDM will be extended with UE Reader subscription information, which consists of the following:

-	information indicating whether the UE is allowed to operate as a UE Reader.

Editor’s note:	Additional subscription information, e.g. validity information, for the UE Reader is FFS.

-	UE Reader subscription information is available to AMF and AIoTF.

-	If AMF receives, as part of the subscription information, the indication that the UE is authorized to operate as a UE Reader, then AMF informs NG-RAN that the UE is authorized to operate as a UE Reader.

Editor’s note:	Whether and how to enable authorization to the UE is FFS.

-	Radio resource management for UE Reader operation:

-	If the gNB has received the indication that a UE is authorized to operate as a UE Reader, then the gNB may assign radio resources to the UE for UE Reader operation.

-	The UE Reader is assumed to request radio resources for Reader operation (e.g. taking into account assistance information received from the AIoTF) from the gNB.

NOTE 1:	Assistance information is further clarified in other clauses.

NOTE 2:	The details of UE Reader radio resource management will be defined by RAN WG2.

#### 8.1.3.3	RRC option

The following principles apply:

-	Messages between the UE Reader and the AIoTF are delivered using RRC between UE and RRC and NGAP between gNB and AMF, and using an SBI interface between AMF and AIoTF. The related protocol stack is shown in Figure 8.1.3.3-1.

Editor's note:	How addressing works for UL is FFS.

Editor's note:	Further details how the RRC option works are FFS.

Figure 8.1.3.3-1: Protocol Stack for the RRC option (This figure is updated based on TS23.369)

#### 8.1.3.4	Interim Conclusion for authorization

The following interim conclusion are agreed for the UE Reader authorization in Topology 2:

-	The subscription data of UE Reader includes the indication that whether the UE is allowed to act as an AIoT Reader.

-	The AMF provides the UE Reader authorization indication to the NG-RAN serving the UE Reader.

Editor's note:	How to authorize the UE Reader will be determined later.

This part is duplicated.

UE reader selection has been added for two scenarios: a) Coverage enhancement; b) handheld UE reader case.

2. Proposal

It is proposed to agree the following changes to 3GPP TR23.700-30.

* * * * First Change * * * *

## 7.1	Agreed Principles

### 7.1.1	Agreed Principles for KI#1

Figure 7.1.1-1 depicts the AIoT System Architecture for Topology 2.

Figure 7.1.1-1: AIoT System Architecture for Topology 2

Figure 7.1.1-2 depicts the AIoT system architecture, using the reference point representation.

Figure 7.1.1-2:  AIoT System Architecture for Topology 2 in reference point representation

NOTE: Whether the interface between AIOTF and UDM is needed will be checked in later phase.

Message and protocol stack

-	Messages between the UE Reader and the AIOTF are delivered using RRC between UE and NG-RAN and NGAP between NG-RAN and AMF, and using an SBI interface between AMF and AIOTF. The related protocol stack is shown in Figure 7.1.1-1.

Figure 7.1.1-1: Protocol Stack for the RRC option

UE reader authorization and revocation part:

-	Subscription aspects:

-	The UE subscription in the UDM will be extended with UE Reader subscription information, which consists of the following:

-	information indicating whether the UE is allowed to operate as a UE Reader.

Editor’s note: Whether and what additional subscription information for the UE Reader is needed is FFS.

-	UE Reader subscription information is available to AMF.

-	If AMF receives, as part of the subscription information, the indication that the UE is authorized to operate as a UE Reader, AMF informs NG-RAN that the UE is authorized to operate as a UE Reader.

Editor’s note: Whether to inform the UE about the authorization information is FFS.

Editor’s note: Whether UE reader capability is provided to AMF is FFS.

-	If the subscription information has been revoked, the indication that the UE is not authorized to operate as a UE Reader from the UDM, then the AMF informs NG-RAN that the UE is not authorized to operate as a UE Reader.

Editor’s note: Whether and how UE Reader subscription information is provided to AIOTF is FFS.

-	Radio resource management for UE Reader operation:

-	If the gNB has received the indication that a UE is authorized to operate as a UE Reader, then the gNB may assign radio resources to the UE for UE Reader operation.

AIOTF Discovery and Selection for UE reader ID:

Editor’s note: How to discover and select AIOTF when AF provides UE reader ID is FFS.

UE reader selection part:

Two scenarios will be supported for UE reader selection, AF providing UE reader ID case and AF providing Area case.

For AF providing UE reader ID case:

-	If UE Reader ID is provided by the AF via the NEF for the operation, then that is used as the selected Reader if authorized as UE reader. The AIOTF provides the selected UE Reader to the NG-RAN.

Editor’s note: How to select UE reader(s) based on the AF provided area information is FFS.

UE reader ID allocation:

Editor’s note: Whether and how to allocate the UE reader ID is FFS.

* * * * Next Change * * * *

## 7.2	Topics for further consideration

### 7.2.1	Topics for further consideration for KI#1

The Editor Notes in clause 7.1.1can be moved to this clauses during the meeting.

* * * * End of Changes * * * *