3GPP TSG-WG SA2 Meeting #171	S2-2509469

Wuhan, CN, 13th Oct – 17th Oct, 2025	(revision of S2-2509027)

# Title:	[DRAFT] Response LS on Exposure of "UE Reachable for DL Traffic" event for a UE in CM-CONNECTED state with RRC-Inactive

# Response to:	S2-2508178/C4-253434 LS on Exposure of "UE Reachable for DL Traffic" event for a UE in CM-CONNECTED state with RRC-Inactive

# Release:	Rel-19

# Work Item:	TEI19

Source:	[Huawei to be] SA2

To:	CT4

Cc:	-

Contact Person:

Name:	Steven Wenham

Tel. Number:

E-mail Address:	steven DOT wenham AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	TS23.502 CR5596.

1. Overall Description:

SA2 thanks CT4 for their LS on Exposure of "UE Reachable for DL Traffic" event for a UE in CM-CONNECTED state with RRC-Inactive in S2-2508178/C4-253434.

If the AMF has received N2 notification (i.e. N2 MT Communication Handling request message) indicating that the UE is transitioning to RRC_INACTIVE state and the determined eDRX values for RRC_INACTIVE, the AMF can determine when the UE is reachable based on the eDRX value. Therefore, when the AMF receives a subscription for the "UE Reachable for DL Traffic" event, the AMF reports the UE is reachable for DL traffic when the UE becomes reachable for paging (e.g., when the UE wakes up from eDRX).

SA2 has agree the attached CR to clarify this case.

2. Actions:

To CT4 group.

ACTION: 	SA2 kindly requests CT4 group to take the above responses into account

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #172		17th Nov – 21st Nov, 2025		Dallas, US

TSG-SA2 Meeting #173		09th Feb – 13th Feb, 2026		India, IN