3GPP TSG-WG SA2 Meeting #171	S2-2509486

# Wuhan, CN, 13th Oct – 17th Oct, 2025		(revision of S2-2508996)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0206 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on UAV Altitude Reporting Thresholds provisioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-09-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TS 23.256 specifies the feature in clause 5.16.2 “Instructing aerial UEs to perform altitude reporting”, and in particular step 3b has the following descriptions: “The UAS NF/NEF may send an Namf_EventExposure_Subscribe request to the AMF to get notified about events of UAV's deviation from the assigned trajectory (i.e. Event ID = "Assigned Trajectory") as specified in clause 5.2.2.3.1 of TS 23.502 [3]. Inside the subscription request, the UAS NF/NEF may include the derived altitude thresholds and indication about reporting periodicity for the UE's altitude reporting.” The bold texts imply the UAS NF/NEF may provide multiple altitude thresholds to AMF, and AMF further signals the multiple altitude thresholds to NG-RAN. On the other hand, RAN3 agreed in R3-255900 the following parameter table for “Aerial UE Flight Information Reporting Control”. 9.3.1.yy Aerial UE Flight Information Reporting Control This IE indicates Aerial UE flight information reporting information. IE/Group Name; Presence; Range; IE type and reference; Semantics description. Higher Altitude Threshold; M; Altitude 9.3.1.ZZ; Indicates the higher altitude threshold information for the Aerial UE reporting.. Lower Altitude Threshold; M; Altitude 9.3.1.ZZ; Indicates the lower altitude threshold information for the Aerial UE reporting.. Reporting Periodicity; O; ENUMERATED {ms120, ms240, ms480, ms640, ms1024, ms2048, ms5120, ms10240, ms20480, ms40960, min1, min6, min12, min30, …}; Indicates the periodicity of the Aerial UE reporting.. The RAN3 agreement means they agreed AMF can provide two thresholds (i.e. Higher Altitude Thresold and Lower Aititude Threshold) for the purpose of Aerial UE Flight Information Reporting Control to NG-RAN. It is also observed that RAN3 agrrements align with what defined in TS 38.331 on “event H1” and “event H2”: - Event H1 (The Aerial UE altitude becomes higher than a threshold) - Event H2 (The Aerial UE altitude becomes lower than a threshold) In addition, in LS R3-255899, RAN3 seeks for further clarification from SA2 on whether the gNB can be provided with multiple aerial UE reporting configurations from the AMF for the same aerial UE. If so, RAN3 asks whether this would require the gNB to configure the aerial UE with multiple aerial UE flight information configurations to the concerned aerial UE, and whether the gNB is expected to report multiple flight information reporting to AMF in accordance with each configuration. When SA2 introduced this feature, it was assumed that no RRC impact will be imposed. Therefore, SA2 should not consider that AMF provides multiple aerial UE reporting configurations for the same aerial UE to the NG-RAN. |
| Summary of change: |  | It is proposed the following clarification in TS 23.256: Change 1, Clause 5.16.2 step 3b: 3b. The UAS NF/NEF may send an Namf_EventExposure_Subscribe request to the AMF to get notified about events of UAV's deviation from the assigned trajectory (i.e. Event ID = "Assigned Trajectory") as specified in clause 5.2.2.3.1 of TS 23.502 [3]. Inside the subscription request, the UAS NF/NEF may include the derived altitude thresholds and indication about reporting periodicity for the UE's altitude reporting. - Text “the UAS NF/NEF may include the derived altitude thresholds” is updated to “the UAS NF/NEF may include aerial UE reporting configuration (i.e. higher altitude threshold and lower altitude threshold)” Change 2, clasue 4.4.1.1.3.3 Nnef_UAVFlightAssistance_Create service operation: - add new parameter in the service operation description “aerial UE reporting configuration (i.e. higher altitude threshold and lower altitude threshold)” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SA2 description is not aligned with RAN3 agreements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.1.1.3.3, 5.16.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.16.2	Instructing aerial UEs to perform altitude reporting

Procedure shown in Figure 5.16.2-1 specifies how the 5GS instructs the aerial UEs (UAVs) to perform altitude reporting, considering requests from a USS/UTM for assistance with UAV's flight. This procedure can be used in conjunction with the NEF-assisted pre-flight planning, in-flight monitoring and USS changeover as specified in clauses 5.12.2, 5.12.3 and 5.13, respectively.

Figure 5.16.2-1: Procedure for instructing aerial UEs for altitude reporting

1.	UAS NF/NEF assists the USS/UTM with pre-flight planning, in-flight monitoring and/or USS changeover based on the received request, which is either a Nnef_RetrieveInfoUAVFlight_Get or a Nnef_UAVFlightAssistance_Create, as specified in clauses 5.12 or 5.13.

2.	Based on the flight path information included in step 1 and other information obtained as a result of either of the procedures (i.e. NEF-assisted pre-flight planning specified in clause 5.12.2, NEF-assisted in-flight monitoring specified in clause 5.12.3, or USS changeover specified in clause 5.13.2), the UAS NF/NEF determines minimum and maximum values of altitudes (i.e. altitude range) for UAV's flight across each of the determined tracking areas (TAs) from the starting point to the destination point.

The determined minimum and maximum altitude values for each of the UAV UE is used by the UAS NF/NEF to derive altitude thresholds for altitude reporting events.

NOTE 1:	Event H1 and H2 for altitude-based reporting specified in clause 5.5.4.21 and clause 5.5.4.22 of TS 38.331 [23] can be used for altitude thresholds and event-based reporting.

Additionally, the UAS NF/NEF determines whether to request the aerial UE to report its altitude information periodically, threshold-based (e.g. altitude becomes a higher/lower than the provided threshold, multiple threshold values can be provided) or event-based (e.g. change of a tracking area/NG-RAN node).

3.	Delivering the UAS NF/NEF-derived altitude thresholds and, optionally, the indication about reporting periodicity (i.e. threshold-based, event-based, or periodic) for UE's altitude reporting to the aerial UE(s).

Option A: Application layer.

3a.	The UAS NF/NEF sends the derived altitude threshold values and provides indication about the required periodicity of the reporting (on demand, event-triggered, periodic, or per change of the altitude, or change of a specific geographical area) to the serving USS/UTM either in a Nnef_RetrieveInfoUAVFlight_Get response message or in a Nnef_UAVFlightAssistance_Notify message depending on the procedure that has triggered the UAS NF/NEF to derive the altitude thresholds for the aerial UE.

4a.	The USS/UTM sends the altitude reporting thresholds and, optionally, indication about reporting periodicity (and any additional information required to perform the reporting, e.g. a list of TA(s)/NG-RAN node identifier(s) for event-based reporting, periodicity value for periodic reporting) to the UAV over the application layer for the UAV's consumption.

NOTE 2:	Application layer reporting itself is outside of 3GPP scope.

Option B: 3GPP-defined signalling.

Editor's note:	This requires RAN to provide further enhancement of the procedure under RAN responsibility.

3b.	The UAS NF/NEF may send an Namf_EventExposure_Subscribe request to the AMF to get notified about events of UAV's deviation from the assigned trajectory (i.e. Event ID = "Assigned Trajectory") as specified in clause 5.2.2.3.1 of TS 23.502 [3]. Inside the subscription request, the UAS NF/NEF may include aerial UE reporting configuration (i.e. higher altitude threshold and lower altitude threshold) and indication about reporting periodicity for the UE's altitude reporting.

when detecting UE enters into a new cell/TA, the AMF provides the NG-RAN with an updated aerial UE reporting configuration (i.e. higher altitude threshold and lower altitude threshold), and NG-RAN over-writes the existing one with the new one.

Additionally, the UAS NF/NEF may also include an indication to perform UE's altitude reporting for all aerial UEs in a specific TA(s) or NG-RAN node(s).

| Assigned Trajectory | <Parameter Type = Notify when the UE deviates from the assigned trajectory, Value=TAI, Cell ID, or RAN Node ID, Scheduled Time in a particular TA, Cell, or gNB and/or Height, acceptable deviation (e.g. Distance, Time)> (to report only when the UE deviates from the assigned values of TAI, Cell ID, or RAN Node ID over the acceptable deviation of distance, Scheduled Time in a particular TA or Cell and/or moves outside the altitude range based on the assigned flight plan over the acceptable deviation on time). The assigned trajectory may consist of multiple segments identified by geographical areas that, for instance, are served by different UAV Service Suppliers. In this case, the information of each segment shall be provided. <Parameter Type = Notify when the altitude reporting conditions are met, Value=altitude thresholds, reporting periodicity> (NOTE 4) |
| --- | --- |

4b.	The AMF sends the corresponding N2 messages (LOCATION REPORTING CONTROL message, see clause 8.12.1.2 of TS 38.413 [24]) to all applicable NG-RAN node(s), including the derived altitude thresholds and optionally reporting periodicity.

5b.	The NG-RAN node(s) instructs the relevant UE(s) to perform altitude reporting as requested by the UAS NF/NEF (via AMF).

NOTE 3:	Aspects related to AMF informing NG-RAN regarding instructing UEs to perform altitude reporting are specified in TS 38.413 [24].

# * * * * Second change * * * *

4.4.1.1.3.3	Nnef_UAVFlightAssistance_Create service operation

Service operation name: Nnef_UAVFlightAssistance_Create

Description: Providing a NEF/UAS NF information about the planned flight path(s) and a time schedule for the UAV from a starting point to a next point in the flight path (even when the next point is located in geographical area served by different USSs) or providing a NEF/UAS NF information about the in-flight monitoring of a target UAV. The request creates an implicit subscription to get notified about the following events, when applicable, (1) the UAS NF/NEF determines that UAV will leave a geographical area served by the USS, (2) the UAV enters a TA/Cell with a border-crossing point to another USS, (3) the UAV deviates from the assigned flight plan, (4) failure of altitude reporting, (5) aerial UE reporting configuration (i.e. higher altitude threshold and lower altitude threshold) and (6) the indication about the required periodicity of the reporting

Input, Required: UAV's identifier (e.g. GPSI), indication about the purpose of the request (i.e. USS changeover, in-flight monitoring).

Input, Optional: Flight path information, which may include planned flight path information for each of the segment of the paths served by different USSs, e.g. a list of TA, UAV's speed, candidate border-crossing point(s), flight height/altitude and/or time schedule for crossing/spending at each of the TAs/cells).

Output, Required: Flight plan configuration ID (associated with the changeover).

Output, Optional: None.

# * * * * End of changes * * * *