3GPP SA WG2 Meeting #171	S2-2509533

# 13 - 17 October, 2025, Wuhan, China												(revision of S2-2508215)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 1511 | rev | 1 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on timestamp of input data |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph3 |  |  |  |  |  | Date: |  |  | 2025-09-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Some input data definitions do not include an explicit timestamp or time-related information, which is essential for correlating multiple input data sources when training ML models in the NWDAF. For the timestamp in clause 6.7.5.2, the time stamp is needed to correlate the Exceptions with other data. The above change is aligned with stage 3, i.e., clause 5.6.2.6 of TS 29517, In clause 6.17.2, the timestamp is nesscary to correlate the UE location from LCS and the UE location from AF, and the NWDAF can collect the timestamp corresponding to the UE location as definded in clause 6.2.12.1, (also can be found in stage 3, clause 6.1.5.2.3 of TS 29.515). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the missing timestamp in input data. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unable to accurately correlate data from different sources, leading to reduced effectiveness or accuracy of ML model training. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.5.2, 6.17.2. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * Start of change* * * *

#### 6.7.5.2	Input Data

The Exceptions information from AF is as specified in Table 6.7.5.2-1.

On request of the service consumer, the NWDAF shall collect and analyse UE behavioural information from the 5GC NFs (SMF, AMF, AF), or OAM as specified in clauses 6.7.2.2 and 6.7.3.2 and/or expected UE behavioural parameters from UDM as defined in clause 4.15.6.3, TS 23.502 [3], depending on Exception IDs.

NOTE:	Care needs to be taken with regards to load by avoiding to cause major extra signalling when collecting data for any UE.

Table 6.7.5.2-1: Exceptions information from AF

| Information | Description |
| --- | --- |
| IP address 5-tuple | To identify a data flow of a UE via the AF (such as the Firewall or a Threat Intelligence Sharing platform) |
| Exceptions (1..max) (NOTE 1) |  |
| >Exception ID | Indicating the Exception ID (such as Unexpected long-live/large rate flows and Suspicion of DDoS attack as defined in Table 6.7.5.3-2) of the data flow. |
| >Exception Level | Scalar value indicating the severity of the abnormal behaviour. |
| >Exception trend | Measured trend (up/down/unknown/stable) |
| Timestamp | Time associated with the exception information |
| NOTE 1: The Exceptions information and the UE behavioural information as defined in clauses 6.7.2.2 and 6.7.3.2 could help NWDAF to train an Abnormal classifier, which could be used to classify a UE behaviour data into Normal behaviour or Exception. |  |

# * * * * Next change * * * *

### 6.17.2	Input Data

In Input data for the analytics ID is listed in Table 6.17.2-1. A training should be done by performing a specific LCS positioning method and comparing the position result with the ground truth UE location obtained by another method.

NOTE:	There are at least two conceivable options for the ground truth data used for the ML Model training of this analytics ID. One option is to use the known location of Positioning Reference Units (PRUs), another option is to use the positioning result of one positioning method (preferably Global Navigation Satellite System (GNSS) information such as GPS information). Collecting PRU related information by NWDAF is not supported in this release. There could be other options for ground truth data which can be determined in the future releases.

Table 6.17.2-1: Data Collected by NWDAF for Location Accuracy Analytics

| Information | Source | Description |
| --- | --- | --- |
| UE IDs | AMF | (List of) SUPI(s) |
| Finer granularity UE location | LCS NOTE 1 | UE location information. |
| >UE location |  | Geographical location or location in local coordinate (as defined in TS 23.032 [34]) |
| >positioning methods |  | The positioning methods used by the LMF when the location measurement was made |
| >LCS QoS |  | LCS QoS accuracy as defined in clause 4.1b in TS 23.273 [39] |
| >Indoor/outdoor indication |  | Indication of indoor/outdoor when LMF can distinguish indoor/outdoor environment |
| >NLOS/LOS indication |  | Indication whether the UE measurements were based on LOS/NLOS measurements |
| >Timestamp |  | A time stamp of time associated with the UE location. |
| UE location (ground truth) | AF | Timestamped UE positions |
| >UE location | NOTE 2, | GNSS location information |
| >Timestamp | NOTE 3 | Time stamp for the UE location |
| >Accuracy |  | The accuracy of the GNSS location information |
| NOTE 1: The procedure to collect location data using LCS is described in clause 6.2.12. NOTE 2: How the AF obtains timestamped GNSS location information of a UE ID is out of 3GPP scope. Whether to trust the UE location from AF is up to the implementation. NOTE 3: When AF is deployed in the untrusted domain, NEF is employed to mediate the interactions between NWDAF and AF via the Naf_EventExposure_Subscribe service specified in clause 5.2.19.2 of TS 23.502 [3], as described in clause 6.2.8.2.3. |  |  |

# * * * * End of changes * * * *