X`3GPP SA WG2 Meeting #171	S2-2509812

# Wuhan, China, October 13 – 17, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3945 | rev | 3 | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of Disaster roaming support in EPS(General clause) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | MINT_Ph2 |  |  |  |  |  | Date: |  |  | 2025-10-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | R19 MINT_PH2 work item is 90% complete in CT1 after August meeting. TSG SA respectfully asks SA2 to perform alignment to the normative work of CT. This CR proposes to align with the CT1 agreements and also introduces additional aspects of MME indicating disaster roaming indication to HSS similar to 5GS handling. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce Disaster roaming support in EPS. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Disaster roaming support in EPS will not be available. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1,3.2,4.3.5.3,4.3.5.7, 4.Y(new), 4.Y.1(new), 4.Y.2(new), 4.Y.3(new), 4.Y.4(new), 4.Y.5(new), 4.Y.6(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************************** Start of Changes *************************

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.402: "Architecture enhancements for non-3GPP accesses".

[3]	ITU-T Recommendation I.130: "Method for the characterization of telecommunication services supported by an ISDN and network capabilities of an ISDN".

[4]	ITU-T Recommendation Q.65: "The unified functional methodology for the characterization of services and network capabilities".

[5]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[6]	3GPP TS 23.203: "Policy and charging control architecture".

[7]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[8]	3GPP TS 43.129: "Packet-switched handover for GERAN A/Gb mode; Stage 2".

[9]	3GPP TS 23.003: "Numbering, addressing and identification".

[10]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station in idle mode".

[11]	3GPP TS 43.022: "Functions related to MS in idle mode and group receive mode".

[12]	3GPP TS 25.304: "UE procedures in idle mode and procedures for cell re-selection in connected mode".

[13]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

[14]	3GPP TS 29.060: "GPRS Tunnelling Protocol (GTP) across the Gn and Gp interface".

[15]	3GPP TS 43.051: "GERAN Overall description - Stage 2".

[16]	3GPP TS 25.401: "UTRAN overall description".

[17]	IETF RFC 1034 (1987): "Domain names – concepts and facilities" (STD 13).

[18]	IETF RFC 4862: "IPv6 Stateless Address Autoconfiguration".

[19]	IETF RFC 2131: "Dynamic Host Configuration Protocol".

[20]	Void.

[21]	Void.

[22]	3GPP TS 25.413: "UTRAN Iu interface Radio Access Network Application Part (RANAP) signalling".

[23]	3GPP TS 44.064: "Mobile Station - Serving GPRS Support Node (MS-SGSN); Logical Link Control (LLC) Layer Specification".

[24]	3GPP TS 23.251: "Network Sharing; Architecture and functional description".

[25]	IETF RFC 4039: "Rapid Commit Option for the Dynamic Host Configuration Protocol version 4 (DHCPv4)".

[26]	IETF RFC 768: "User Datagram Protocol".

[27]	3GPP TS 23.221: "Architectural requirements".

[28]	3GPP TS 23.008: "Organization of subscriber data".

[29]	3GPP TS 23.078: "Customized Applications for Mobile network Enhanced Logic (CAMEL) Phase X; Stage 2".

[30]	3GPP TS 23.236: "Intra-domain connection of Radio Access Network (RAN) nodes to multiple Core Network (CN) nodes".

[31]	IETF RFC 3588: "Diameter Base Protocol".

[32]	IETF RFC 4861: "Neighbor Discovery for IP Version 6 (IPv6)".

[33]	3GPP TS 25.331: "Radio Resource Control (RRC); Protocol Specification".

[34]	3GPP TS 36.304: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) procedures in idle mode".

[35]	IETF RFC 4960: "Stream Control Transmission Protocol".

[36]	3GPP TS 36.413: "Evolved Universal Terrestrial Access Network (E-UTRAN); S1 Application Protocol (S1AP)".

[37]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[38]	3GPP TS 29.061: "Interworking between the Public Land Mobile Network (PLMN) supporting packet based services and Packet Data Networks (PDN)".

[39]	Void.

[40]	3GPP TS 33.102: "3G Security; Security architecture".

[41]	3GPP TS 33.401: "3GPP System Architecture Evolution: Security Architecture".

[42]	3GPP TS 48.018: "General Packet Radio Service (GPRS); Base Station System (BSS) - Serving GPRS Support Node (SGSN); BSS GPRS Protocol (BSSGP)".

[43]	3GPP TS 29.274: "3GPP Evolved Packet System (EPS); Evolved General Packet Radio Service (GPRS) Tunnelling Protocol for Control plane (GTPv2-C); Stage 3".

[44]	3GPP TS 32.251: "Telecommunication management; Charging management; Packet Switched (PS) domain charging".

[45]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General aspects".

[46]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[47]	3GPP TS 24.008: "Mobile Radio Interface Layer 3 specification; Core Network Protocols; Stage 3".

[48]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[49]	3GPP TS 22.042: "Network Identity and Time Zone (NITZ) service description; Stage 1".

[50]	Void.

[51]	3GPP TS 32.240: "Charging architecture and principles".

[52]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[53]	3GPP TS 24.285: "Allowed Closed Subscriber Group (CSG) List; Management Object (MO)".

[54]	3GPP TS 23.261: "IP flow mobility and seamless Wireless Local Area Network (WLAN) offload; Stage 2".

[55]	IETF RFC 3168: "The Addition of Explicit Congestion Notification (ECN) to IP".

[56]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction".

[57]	3GPP TS 23.271: "Functional stage 2 description of LCS".

[58]	3GPP TS 23.272: "Circuit Switched (CS) fallback in Evolved Packet System (EPS); Stage 2".

[59]	3GPP TS 23.107: "Quality of Service (QoS) concept and architecture".

[60]	3GPP TS 23.292: "IP Multimedia Subsystem (IMS) centralized services; Stage 2".

[61]	3GPP TS 29.303: "Domain Name System Procedures; Stage 3".

[62]	IETF RFC 3376: "Internet Group Management Protocol, Version 3".

[63]	IETF RFC 3810: "Multicast Listener Discovery Version 2 (MLDv2) for IPv6".

[64]	IETF RFC 3927: "Dynamic Configuration of IPv4 Link-Local Addresses".

[65]	IETF RFC 4291: "IP Version 6 Addressing Architecture".

[66]	3GPP TS 22.368: "Service Requirements for Machine-Type Communications (MTC); Stage 1".

[67]	3GPP TS 22.011: "Service Accessibility".

[68]	3GPP TS 22.153: "Multimedia priority service".

[69]	3GPP TS 24.368: "Non-Access Stratum (NAS) configuration Management Object (MO)".

[70]	IETF RFC 6603: "Prefix Exclude Option for DHCPv6-based Prefix Delegation".

[71]	3GPP TS 23.002: "Network Architecture".

[72]	3GPP TS 23.007: "Restoration procedures".

[73]	3GPP TS 22.173: "IP Multimedia Core Network Subsystem (IMS) Multimedia Telephony Service and supplementary services; Stage 1".

[74]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[75]	3GPP TS 23.380: "IMS Restoration Procedures".

[76]	3GPP TS 36.423: "Evolved Universal Terrestrial Radio Access Network (E-UTRAN); X2 Application Protocol (X2AP)".

[77]	IETF RFC 5795: "The RObust Header Compression (ROHC) Framework".

[78]	3GPP TS 36.323: "Evolved Universal Terrestrial Radio Access (E-UTRA); Packet Data Convergence Protocol (PDCP) specification".

[79]	3GPP TS 29.128: "Mobility Management Entity (MME) and Serving GPRS Support Node (SGSN) interfaces for interworking with packet data networks and applications".

[80]	3GPP TS 22.101: "Service aspects; Service principles".

[81]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[82]	3GPP TS 36.306: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio access capabilities".

[83]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[84]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[85]	3GPP TS 37.340: "Evolved Universal Terrestrial Radio Access (E-UTRA) and NR; Multi-connectivity; Stage 2".

[86]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[87]	3GPP TS 36.321: "Evolved Universal Terrestrial Radio Access -E-UTRA); Medium Access Control -MAC) protocol specification".

[88]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[89]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[90]	3GPP TS 38.401: "NG-RAN; Architecture description".

[91]	3GPP TS 29.674: "Interface between the UE radio Capability Management Function (UCMF) and the Mobility Management Entity (MME); Stage 3".

[92]	3GPP TS 36.410: "Evolved Universal Terrestrial Radio Acces Network (E-UTRAN); S1 general aspects and principles".

[93]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[94]	IETF RFC 8415: "Dynamic Host Configuration Protocol for IPv6 (DHCPv6)".

[95]	3GPP TS 23.204: "Support of Short Message Service (SMS) over generic 3GPP Internet Protocol (IP) access; Stage 2".

[96]	3GPP TR 23.700-29: "Study on Security and Privacy Aspects of 5G Satellite Access; Phase 3".

[X1]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

*************************** Next Changes ***************************

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Disaster Condition: as defined in TS 22.261 [X1].

Disaster Inbound Roamer: as defined in TS 22.261 [X1].

Disaster Roaming: as defined in TS 22.261 [X1].

MME Pool Area: An MME Pool Area is defined as an area within which a UE may be served without need to change the serving MME. An MME Pool Area is served by one or more MMEs ("pool of MMEs") in parallel. MME Pool Areas are a collection of complete Tracking Areas. MME Pool Areas may overlap each other.

MINT-EPS: as defined in TS 23.122 [10].

PLMN with Disaster Condition: A PLMN to which a Disaster Condition applies.

Serving GW Service Area: A Serving GW Service Area is defined as an area within which a UE may be served without need to change the Serving GW. A Serving GW Service Area is served by one or more Serving GWs in parallel. Serving GW Service Areas are a collection of complete Tracking Areas. Serving GW Service Areas may overlap each other.

PDN Connection: The association between a PDN represented by an APN and a UE, represented by one IPv4 address and/or one IPv6 prefix (for IP PDN Type) or by the UE Identity (for Non-IP and Ethernet PDN Types).

Default Bearer: The EPS bearer which is first established for a new PDN connection and remains established throughout the lifetime of the PDN connection.

Default APN: A Default APN is defined as the APN which is marked as default in the subscription data and used during the Attach procedure and the UE requested PDN connectivity procedure when no APN is provided by the UE.

eCall Only Mode: A UE configuration option that allows the UE to attach at EPS and register in IMS to perform only eCall Over IMS, and an IMS call to a non-emergency MSISDN or URI for test and/or terminal reconfiguration services. For a short period following either such call, an incoming call (e.g. callback from a PSAP or HPLMN operator) or other incoming session (e.g. for USIM reconfiguration) is possible. At other times when the UE is configured in this mode, the UE is required to refrain from any signalling to a network. Use of eCall Only Mode is configured in the USIM for the UE.

PDN Connection to the SCEF: The association between a UE, represented by the UE Identity, and a PDN represented by an APN to external packet data network via SCEF to allow transfer of Non-IP data. It includes establishment and persistence of T6 connection between MME and SCEF (see TS 29.128 [79]).

Emergency attached UE: A UE which only has bearer(s) related to emergency bearer service.

NOTE 1:	The above term is equivalent to the term "attached for emergency bearer services" as specified in TS 24.301 [46].

LIPA PDN connection: a PDN Connection for local access (e.g. for IP or Ethernet access) for a UE connected to a HeNB.

en-gNB: As defined in TS 37.340 [85].

SIPTO at local network PDN connection: a PDN connection for SIPTO at local network for a UE connected to a (H)eNB.

Correlation ID: For a LIPA PDN connection, Correlation ID is a parameter that enables direct user plane path between the HeNB and L-GW.

SIPTO Correlation ID: For a SIPTO at local network PDN connection, SIPTO Correlation ID is a parameter that enables direct user plane path between the (H)eNB and L-GW when they are collocated.

Local Home Network: A set of (H)eNBs and L-GWs in the standalone GW architecture, where the (H)eNBs have IP connectivity for SIPTO at the Local Network via all the L-GWs.

Local Home Network ID: An identifier that uniquely identifies a Local Home Network within a PLMN.

Presence Reporting Area: An area defined within 3GPP Packet Domain for the purposes of reporting of UE presence within that area due to policy control and/or charging reasons. In the case of E-UTRAN, a Presence Reporting Area may consist in a set of neighbor or non-neighbor Tracking Areas, or eNodeBs and/or cells. There are two types of Presence Reporting Areas: "UE-dedicated Presence Reporting Areas" and "Core Network pre-configured Presence Reporting Areas" that apply to an MME pool.

RAN user plane congestion: RAN user plane congestion occurs when the demand for RAN resources exceeds the available RAN capacity to deliver the user data for a prolonged period of time.

NOTE 2:	Short-duration traffic bursts is a normal condition at any traffic load level, and is not considered to be RAN user plane congestion. Likewise, a high-level of utilization of RAN resources (based on operator configuration) is considered a normal mode of operation and might not be RAN user plane congestion.

IOPS-capable eNodeB: an eNodeB that has the capability of IOPS mode operation, which provides local connectivity (e.g. for IP or Ethernet) and public safety services to IOPS-enabled UEs via a Local EPC when the eNodeB has lost backhaul to the Macro EPC or it has no backhaul to the Macro EPC.

IOPS network: an IOPS network consists of one or more eNodeBs operating in IOPS mode and connected to a Local EPC.

Local EPC: a Local EPC is an entity which provides functionality that eNodeBs in IOPS mode of operation use, instead of the Macro EPC, in order to support public safety services.

Macro EPC: the EPC which serves an eNodeB when it is not in IOPS mode of operation.

Nomadic EPS: a deployable system which has the capability to provide radio access (via deployable IOPS-capable eNodeB(s)), local connectivity (e.g. for IP or Ethernet) and public safety services to IOPS-enabled UEs in the absence of normal EPS

Multi-USIM UE: a UE with multiple USIMs, capable of maintaining a separate registration state with a PLMN for each USIM at least over 3GPP Access and supporting one or more of the features described in clause 4.3.33.

IOPS-enabled UE: is an UE that is configured to use networks operating in IOPS mode.

Cellular IoT: Cellular network supporting low complexity and low throughput devices for a network of Things. Cellular IoT supports IP, Ethernet and Non-IP traffic. Unless otherwise stated in this specification, Cellular IoT and all functionality applicable to Cellular IoT also apply to satellite access.

Narrowband-IoT: a 3GPP Radio Access Technology that forms part of Cellular IoT. It allows access to network services via E-UTRA with a channel bandwidth limited to 180 kHz (corresponding to one PRB). Unless otherwise indicated in a clause, Narrowband-IoT is a subset of E-UTRAN. Unless otherwise stated in this specification, Narrowband-IoT also includes satellite access.

LTE-M: a 3GPP RAT type Identifier used in the Core Network only, which is a sub-type E-UTRAN RAT type, and defined to identify in the Core Network the E-UTRAN when used by a UE indicating Category M in its UE radio capability. Unless otherwise stated in this specification, LTE-M also includes satellite access.

WB-E-UTRAN: in the RAN, WB-E-UTRAN is the part of E-UTRAN that excludes NB-IoT. In the Core Network, the WB-E-UTRAN also excludes LTE-M. Unless otherwise stated in this specification, WB-E-UTRAN also includes satellite access.

DCN-ID: DCN identity identifies a specific dedicated core network (DCN).

For the purposes of the present document, the following terms and definitions given in TS 23.167 [81] apply:

eCall Over IMS: See TS 23.167 [81].

RLOS attached UE: A UE is attached only for accessing Restricted Local Operator Services (see TS 23.221 [27]).

IAB-donor: For the purposes of this specification, this is a NR Secondary RAN node is further described in TS 37.340 [85] that supports Integrated access and backhaul (IAB) feature and provides connection to the core network to IAB-nodes. It supports the CU function of the CU/DU architecture for IAB defined in TS 38.401 [90].

IAB-node: A relay node that supports wireless in-band and out-of-band relaying of NR access traffic via NR Uu backhaul links. It supports the UE function and the DU function of the CU/DU architecture for IAB defined in TS 38.401 [90].

Feeder link: as defined in TS 36.300 [5].

Service link: as defined in TS 36.300 [5]

Store and Forward Satellite operation: An operation mode that provides to the UE a communication service when the serving satellite has a discontinuous connection to the ground network and such connection is not available when the satellite is interacting with the UE.

S&F Mode: The mode in which the RAN and core network entities perform Store and Forward Satellite operation.

MINT-EPS: as defined in TS 23.122 [10].

NTN payload: as defined in TS 36.300 [5].

NTN Gateway: as defined in TS 36.300 [5].

*************************** Next Changes ***************************

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

5GS	5G System

AF	Application Function

ARP	Allocation and Retention Priority

AMBR	Aggregate Maximum Bit Rate

CBC	Cell Broadcast Centre

CBE	Cell Broadcast Entity

CIoT	Cellular IoT

CSG	Closed Subscriber Group

CSG ID	Closed Subscriber Group Identity

C-SGN	CIoT Serving Gateway Node

CSS	CSG Subscriber Server

DAPS	Dual Active Protocol Stacks

DCN	Dedicated Core Network

DeNB	Donor eNode B

DL TFT	DownLink Traffic Flow Template

DRX	Discontinuous Reception

ECGI	E-UTRAN Cell Global Identifier

ECM	EPS Connection Management

ECN	Explicit Congestion Notification

EMM	EPS Mobility Management

eNodeB	evolved Node B

EPC	Evolved Packet Core

EPS	Evolved Packet System

E-RAB	E-UTRAN Radio Access Bearer

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

GBR	Guaranteed Bit Rate

GUMMEI	Globally Unique MME Identifier

GUTI	Globally Unique Temporary Identity

GW	Gateway

HeNB	Home eNode B

HeNB GW	Home eNode B Gateway

HFN	Hyper Frame Number

IAB	Integrated Access and Backhaul

IMEI/TAC	IMEI Type Allocation Code

IOPS	Isolated E-UTRAN Operation for Public Safety

IoT	Internet of Things

ISR	Idle mode Signalling Reduction

LAA	Licensed Assisted Access

LBI	Linked EPS Bearer Id

L-GW	Local GateWay

LIPA	Local IP Access

LWA	LTE/WLAN Aggregation

LWIP	LTE/WLAN Radio Level Integration with IPsec Tunnel

MBR	Maximum Bit Rate

MINT	Minimization of Service Interruption

MME	Mobility Management Entity

MMEC	MME Code

MTC	Machine-Type Communications

MT-EDT	Mobile Terminated Early Data Transmission

M-TMSI	M-Temporary Mobile Subscriber Identity

NB-IoT	Narrowband IoT

NR	New Radio

NR-U	New Radio Unlicensed

NTN	Non-Terrestrial Network

OCS	Online Charging System

OFCS	Offline Charging System

OMC-ID	Operation and Maintenance Centre Identity

P-GW	PDN Gateway

PCC	Policy and Charging Control

PCRF	Policy and Charging Rules Function

PRA	Presence Reporting Area

PDCP	Packet Data Convergence Protocol

PMIP	Proxy Mobile IP

PSAP	Public Safety Answering Point

PSM	Power Saving Mode

PTI	Procedure Transaction Id

QCI	QoS Class Identifier

RACS	UE Radio Capability Signalling optimization

RCAF	RAN Congestion Awareness Function

RFSP	RAT/Frequency Selection Priority

RLOS	Restricted Local Operator Services

RN	Relay Node

RUCI	RAN User Plane Congestion Information

S&F	Store and Forward

S-GW	Serving Gateway

S-TMSI	S-Temporary Mobile Subscriber Identity

SDF	Service Data Flow

SIPTO	Selected IP Traffic Offload

TAC	Tracking Area Code

TAD	Traffic Aggregate Description

TAI	Tracking Area Identity

TAU	Tracking Area Update

TI	Transaction Identifier

TIN	Temporary Identity used in Next update

UCMF	UE radio Capability Management Function

URRP-MME	UE Reachability Request Parameter for MME

UL TFT	UpLink Traffic Flow Template

ULR-Flags	Update Location Request Flags

*************************** Next Changes ***************************

#### 4.3.5.3	Tracking Area list management

Tracking Area list management comprises the functions to allocate and reallocate a Tracking Area Identity list to the UE. All the tracking areas in a Tracking Area List to which a UE is registered are served by the same serving MME.

The "tracking area list concept" is used with E-UTRAN. With this concept, when the UE registers with the network, the MME allocates a set (a "list") of tracking areas to the UE. By making the centre of this set of tracking areas close to the UE's current location, the chance of a UE rapidly making another tracking area update can be reduced.

When MME allocates TAI list for UE registered for Disaster Roaming service in EPS as specified in clause 4.Y, the MME shall only consider TAIs covering the area with the Disaster Condition.

If SIPTO at local network with stand-alone GW, Serving GW relocation without mobility and ISR are supported in the core network the Tracking Area list should only contain either Tracking Areas inside one local network or inside the macro network. If the tracking area list covers both local network and macro network, the ISR shall not be activated if the UE is allowed to use SIPTO at local network.

The MME determines the RAT type the UE is camping on, i.e. NB-IoT or WB-E-UTRAN for terrestrial access and WB-E-UTRAN or NB-IoT RAT types for satellite access, based on the Tracking Area indicated in the INITIAL UE MESSAGE by the eNodeB.

To ensure a UE initiates tracking area updating procedure when performing inter-RAT mobility between NB-IoT and WB-E-UTRAN, the E-UTRAN shall be configured such that a Tracking Area does not contain both WB-E-UTRAN and NB-IoT cells, and, the MME shall not allocate a Tracking Area Identity list that contains both NB-IoT and WB-E-UTRAN Tracking Areas.

To ensure the UE initiates tracking area update procedure when it moves to and from an MME that supports 15 EPS bearers per UE as defined in clause 4.12 to an MME that does not support 15 EPS bearers per MME and vice versa, the MME shall allocate a Tracking Area Identity list that provides homogenous support for 15 EPS bearers per UE.

Other features (e.g. User Plane CIoT EPS Optimisation, Supporting up to 15 EPS bearers per UE, satellite access) may require the MME to adapt how it creates the "list" of TAIs.

NOTE:	This TAI list functionality is different to the SGSN behaviour in GERAN and UTRAN systems. In GERAN/UTRAN the UE is only registered in one Routeing Area at a time.

*************************** Next Changes ***************************

#### 4.3.5.7	Mobility Restrictions

Mobility Restrictions comprises the functions for restrictions to mobility handling of a UE in E-UTRAN access. The Mobility Restriction functionality is provided by the UE, the radio access network and the core network.

Mobility Restriction functionality in state ECM-IDLE is executed in UE based on information received from the core network. Mobility Restriction functionality in state ECM-CONNECTED is executed in the radio network and the core network.

In state ECM-CONNECTED, the core network provides the radio network with a Handover Restriction List. The Handover Restriction List specifies roaming, area and access restrictions. If roaming restriction to GERAN or UTRAN access needs to be enforced, a MME that is connected to eNodeBs that may handover or invoke release with redirection to UTRAN or GERAN is configured with a list of HPLMN IDs that are permitted to access GERAN or UTRAN unless restricted by the UE individual access restriction information received from HSS.

For a UE requesting Disaster Roaming service in EPS, the UE is only allowed to receive services in the area with Disaster Condition as specified in clause 4.Y. The other areas shall be considered as forbidden area for the UE attached for Disaster Roaming service in EPS.

*************************** Next Changes ***************************

## 4.Y	Support of Disaster Roaming with Minimization of Service Interruption in EPS

### 4.Y.1	General

Subject to operator policy and national/regional regulations, EPS provides Disaster Roaming service for the UEs from PLMN(s) with Disaster Condition in 5GS. The UE shall attempt Disaster Roaming only if:

-	there is no available PLMN which is allowable except PLMN/E-UTRAN combinations available only for Disaster Roaming service (see TS 23.122 [10]);

-	the UE is not in RM-REGISTERED and not in CM-CONNECTED state over non-3GPP access connected to 5GCN;

-	the UE supports Disaster Roaming service in EPS;

-	the UE has been configured by the HPLMN with an indication of whether Disaster roaming in EPS is enabled in the UE set to "disaster roaming in EPS is enabled in the UE" as specified in clause 4.Y.2; and

*************************** Next Changes ***************************

### 4.Y.2	UE configuration and provisioning for Disaster Roaming

A UE supporting Disaster Roaming in EPS is configured with the following information:

-	Optionally, indication of whether disaster roaming in EPS is enabled in the UE;

-	Optionally, indication of 'applicability of "lists of PLMN(s) to be used in disaster condition" provided by a VPLMN';

-	Optionally, list of PLMN(s) to be used in Disaster Condition.

The Activation of Disaster Roaming is performed by the HPLMN by setting the indication of whether Disaster Roaming in EPS is enabled in the UE to "disaster roaming in EPS is enabled in the UE" using the UE Parameters Update Procedure as defined in TS 23.502 [84]. The UE shall only perform Disaster Roaming if the HPLMN has configured the UE with the indication of whether Disaster Roaming in EPS is enabled in the UE and set the indication to "disaster roaming in EPS is enabled in the UE".

The optional 'list of PLMN(s) to be used in Disaster Condition' may be pre-configured in USIM or provided by the HPLMN during and after a successful registration procedure over 3GPP access or non-3GPP access via Registration Request procedure or UE Configuration Update procedure as defined in TS 23.502 [84] or Attach procedure as defined in clause 5.3.2 or Tracking Area Update procedure as defined in clause 5.3.3. The 'list of PLMN(s) to be used in Disaster Condition' may be configured over non-3GPP access before Disaster Condition has occurred as defined in TS 23.502 [84].

While roaming (i.e. not in HPLMN), the Registered PLMN may provide the 'list of PLMN(s) to be used in Disaster Condition' during and after a successful registration procedure to the UE via Registration Request procedure or UE Configuration Update procedure as specified in TS 23.502 [84]. This list shall not alter any list provided by the HPLMN and shall only be used if the UE is configured by the HPLMN using the UE Parameters Update Procedure as defined in TS 23.502 [84] with the indication of 'applicability of "lists of PLMN(s) to be used in Disaster Condition" provided by a VPLMN' set to "True".

The details of the UE behaviour regarding the usage of this list are described in TS 23.122 [10] and TS 24.301 [46]. If the UE is not configured with 'list of PLMN(s) to be used in Disaster Condition', the UE follows the procedure described in TS 23.122 [10] to select PLMN to be used in Disaster Condition.

*************************** Next Changes ***************************

### 4.Y.3	Disaster Condition Notification and Determination

The E-UTRAN cell in the PLMN that provides Disaster Roaming service, broadcasts an indication of accessibility for Disaster Roaming service and optionally, a 'list of one or more PLMN(s) with Disaster Condition for which Disaster Roaming service is offered by the available PLMN' in the impacted area as described in TS 23.122 [10] and TS 36.304 [34].

A UE determines the Disaster Condition based on the information broadcasted from the E-UTRAN cell providing Disaster Roaming service in EPS and performs the network selection and the access control for the Disaster Roaming in EPS as described in TS 23.122 [10] and TS 24.301 [46].

NOTE 1:	How a PLMN is notified that another PLMN is a PLMN with Disaster Condition and how a PLMN is notified of the area where the associated Disaster Condition in EPS applies is managed by the government agencies or the authorities and is out of scope of 3GPP.

NOTE 2:	The broadcast for Disaster Roaming service in EPS from the E-UTRAN occurs only during the Disaster Condition.

*************************** Next Changes ***************************

### 4.Y.4	Attach Procedure or TAU procedure for Disaster Roaming service in EPS

For a UE to receive Disaster Roaming service in EPS from a PLMN providing Disaster Roaming service in EPS, the UE sends a NAS Attach Request message with Attach Type value set to "Disaster Roaming Attach" or TAU request message with Update Type value set to "Disaster Roaming Update":

-	When the MME in the PLMN providing Disaster Roaming service in EPS receives a NAS Attach Request message with Attach Type value set to "Disaster Roaming Attach" or TAU request message with Update Type set to "Disaster Roaming Update";

-	the MME controls if the UE is allowed to access Disaster Roaming service in EPS in the area with Disaster Condition as specified in clause 5.3.2, 5.3.3;

-	the MME may provide the indication of Disaster Roaming service in its request to HSS as specified in clause 5.3.2.1, 5.3.3.1, 5.3.3.2.

-	the MME may provide the Disaster Roaming service indication to Serving GW, PDN GW, HSS as specified in Attach procedure in clause 5.3.2 or Tracking Area Update procedure in clause 5.3.3.

NOTE 1:	The HSS is configured with Disaster Condition in EPS via OAM based on operator policy and the request by the government agencies. Based on this local configuration and/or the Disaster Roaming service indication, the HSS can execute authentication of the UE and can provides the subscription data for a Disaster Roaming service to the MME.

To support the Disaster Roaming service, the PLMN providing Disaster Roaming service is configured to support communication with the network entities in the HPLMN of the UE, i.e. configurations related to roaming interfaces for communication between serving PLMN and HPLMN shall be deployed in the affected entities. This communication between the PLMNs need only be enabled during the Disaster Condition.

The Disaster Roaming service in EPS is limited to the impacted geographic area with Disaster Condition. The E-UTRAN node’s and MME in the PLMN providing Disaster Roaming service in EPS are configured with the area information, i.e. a list of TAIs which can be formulated by the PLMN providing the Disaster Roaming service in EPS based on the geographic area with Disaster Condition in the other PLMN(s).

*************************** Next Changes ***************************

### 4.Y.5	Handling when a Disaster Condition is no longer applicable

When a UE detects a Disaster Condition is no longer applicable, the UE performs PLMN selection as described in TS 23.122 [10] and TS 24.301 [46] and may return to the PLMN previously with Disaster Condition.

A PLMN providing Disaster Roaming service in EPS:

-	May trigger the Disaster Inbound Roaming UEs to return to the PLMN previously with Disaster Condition when the Disaster Inbound Roamers attempt to transit to EMM-CONNECTED mode.

-	May trigger the Disaster Inbound Roaming UEs to return to the PLMN previously with Disaster Condition by triggering Detach procedure.

-	May trigger the Disaster Inbound Roaming UEs to return to the PLMN previously with Disaster Condition by rejecting Attach Request or TAU Request message.

-	May trigger the Disaster Inbound Roaming UEs to return to the PLMN previously with Disaster Condition by rejecting Service Request message.

-	Shall organise the return of the Disaster Roaming UEs in a manner that does not cause overload (e.g. of signalling) in the PLMN that previously had the Disaster Condition.

-	Stop broadcasting of providing Disaster Roaming service as specified in clause 4.Y.3.

NOTE:	Whether and how long the PLMN waits before paging the Disaster Inbound Roamers upon being notified that a Disaster Condition no longer applies is up to operator's policy.

The HPLMN i.e. the HSS may trigger the Disaster Inbound Roaming UEs to return to the PLMN previously with Disaster Condition by triggering Detach procedure.

*************************** Next Changes ***************************

### 4.Y.6	Prevention of signalling overload related to Disaster Condition and Disaster Roaming service

The load control, congestion and overload control mechanism specified in clause 5.19 and access control and barring specified in TS 36.331 [37] and TS 22.011 [67] can be used to mitigate the load caused by UE requesting the Disaster Roaming service in EPS in the PLMN providing Disaster Roaming service in EPS and returning of UE to allowable PLMN when Disaster Condition is no longer applicable.

To prevent signalling overload in PLMN providing Disaster Roaming service in EPS, the HPLMN or registered PLMN:

-	may provide the UE in a prioritized manner with the list of PLMNs described in clause  4.Y.2 for Disaster Roaming service in EPS;

-	may provide disaster roaming wait range information to control when the UE can initiate the Attach or TAU procedure for Disaster Roaming service in EPS upon arriving in the PLMN providing Disaster Roaming service in EPS as specified in TS 23.122 [10] and TS 24.301 [46]; and

-	applies Access barring for Disaster Roaming service in EPS as specified in TS 24.301 [46].

NOTE:	The mechanisms available at the MME for mitigation of overload and congestion are used for congestion mitigation during the Disaster Roaming.

To prevent signalling overload by returning UEs in PLMN previously with Disaster Condition which is no longer applicable, the HPLMN or registered PLMN:

-	may provide disaster return wait range information to control when the UE can initiate the registration upon returning to the PLMN previously with Disaster Condition as specified in TS 23.122 [10].

*************************** End of Changes ****************************