SA WG2 Meeting #172	S2-2510821

17 - 21 November, 2025, Dallas, USA	(revision of S2-2510256)

Source:	ZTE

Title:	pCR to TR 23.700-56: KI#1, Sol#7: Update to Address EN

Document for:	Approval

Agenda Item:	20.7.1

Work Item / Release:	FS_NG_RTC_Ph3_ARC / Rel-20

Abstract of the contribution: The paper updates the solution 7 to address editor’s notes.

1. Discussion

The proposal addresses the editor’s notes in solution 7.

2. Proposal

It is proposed to capture the following changes into TR 23.700-56.

* * * * Start of Changes * * * *

6.7	Solution #7: IMS DC Events Supported by DCSF

6.7.0	High-level solution Principles

This solution addresses Key Issue #2: enhancements to capability exposure framework for eMMTel services, especially how to support IMS DC events supported by DCSF.

The solution has the following principles:

-	The DCSF can expose some IMS DC events to NEF or AF.

-	The exposure interfaces will reuse DC3 between DCSF and NEF and DC4 between DCSF and AF.

-	The AF supports subscribing the events exposed by the DCSF and receiving the IMS DC events from the DCSF.

6.7.1	Description

This solution provides a procedure on how to support IMS DC events supported by DCSF.

The table AD.2.5.2-1 List of events for monitoring capability in clause AD.2.5.2 of TS 23.228 [2] is enhanced to support the IMS DC events supported by DCSF.

Table AD.2.5.2-1: (from TS 23.228 [2]) List of events for monitoring capability

| Event | Description and Detection criteria | Which NF detects the event |
| --- | --- | --- |
| Bootstrap Data Channel established | Detected when a Bootstrap data channel is established. Network detects that a Bootstrap data channel is established when the SDP for an established IMS DC includes the media for a Bootstrap data channel with the corresponding Bootstrap DC stream ID according to TS 26.114 [76]. | IMS AS |
| Bootstrap Data Channel Release | Detected when an established Bootstrap data channel is released. Network detects that a Bootstrap data channel is released when an IMS DC that includes an SDP with the media for a Bootstrap data channel with the corresponding Bootstrap DC stream ID according to TS 26.114 [76] is released, or an IMS DC is updated to remove the media for a Bootstrap data channel with the corresponding Bootstrap DC stream ID according to TS 26.114 [76]. | IMS AS |
| Application Data Channel established | Detected when an Application Data Channel is established. Network detects that an Application data channel is established when the SDP for an established IMS DC includes the media for an Application data channel with the corresponding Application DC stream ID according to TS 26.114 [76]. The Application DC stream ID can be explicit or any stream ID. | IMS AS |
| Application Data Channel Release | Detected when an Application data channel is released. Network detects that an Application data channel is released when an IMS DC that includes an SDP with the media for an Application data channel with the corresponding Application DC stream ID according to TS 26.114 [76] is released, or an IMS DC is updated to remove the media for an Application data channel with the corresponding Application DC stream ID according to TS 26.114 [76]. | IMS AS |
| Application Data Channel requires interworking is established | Detected when an Application Data Channel that requires interworking is in the process of establishment. Network detects that an P2A Application data channel is in process of establishment and the MDC2 resources are assigned for the Application data channel. | IMS AS |
| Application operation | Detected when an application is operated. Network detects that an application is operated, e.g. downloading. | DCSF |

NOTE:	Only "Application Operation" is added to the above table - other entries already exist in TS 23.228 [2].

6.7.2	Procedure

The procedure for DCSF event exposure is very similar to the subscribe/notify procedure for non-subscriber specific IMS events in clause AD.3.3 of TS 23.228 [2]. The following differences apply:

-	the subscribing destination is DCSF rather than IMS AS.

Figure 6.7.2-1: Procedure for subscribe/notify of events supported by DCSF

The steps in the procedure are as follows:

1.	The AF subscribes to NEF initiating the Nnef_imsEE_Subscribe Request for non-subscriber specific IMS DC events supported by the DCSF.

2.	If the subscription to the non-subscriber specific IMS event is authorized by the NEF, the NEF locates all DCSF instances that support the requested IMS DC events via NRF or by local configuration.

3.	For each DCSF instance that supports the requested IMS DC event, NEF subscribes to the DCSF for the requested IMS event via DC3.

4.	The DCSF returns to the NEF the Subscription Response via DC3.

5.	The NEF returns to AF the Nnef_ImsEE_Subscribe Response.

6.	At some point, the requested IMS DC event is detected by the DCSF.

7.	The DCSF sends a notification to the NEF.

8.	The NEF sends Nnef_ImsEE_Notify Request to the AF.

NOTE 1:	For the trusted AF, the AF may send the subscription request to the DCSF and receive the event notification from the DCSF via DC4 directly.

NOTE 2:	For the subscribing request from AF when it is subscriber specific, the NEF may discover the DCSF from the HSS; and it is assumed that the DCSF has registered to the HSS as described in Solution #9.

6.7.3	Impacts on Services, Entities and Interfaces

The solution has the following impacts:

DCSF:

-	supporting detecting IMS DC events which are subscribed by an AF;

-	supporting IMS DC event subscription and notification;

-	supporting registering to the HSS.

NEF:

-	supporting transferring the subscription request from AF to DCSF;

-	supporting transferring the event notification from DCSF to AF.

AF:

-	supporting subscription and receiving IMS DC events detected by DCSF.

HSS:

-	supporting registration and discovery of the DCSF.

* * * * End of Changes * * * *