3GPP TSG-SA WG2 Meeting #173	S2-2601569

# Goa, India, 9th - 13th February 2026										 (revision of S2-2601354)

| CR-Form-v12.5 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6487 | rev | 3 | Current version: | 20.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of EIF exposure of Energy Consumption Category and of the Energy Consumption per bit. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | EnergySys_ph2-ARC |  |  |  |  |  | Date: |  |  | 2026-02-13 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) Rel-21 (Release 21) |  |
| Reason for change: |  | Based on the conclusions documented in TR 23.700-67 it has been agreed to adopt some new EIF exposure capabilities This CR implements these additions |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding exposure of Energy Consumption Category and of the Energy Consumption per bit. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack of the new feature |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.51.2.1, 5.51.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

#### 5.51.2.1	General

The Energy Information Function (EIF) is defined to:

-	collect the UE related Energy Consumption information;

-	calculate the Energy Consumption information at UE, S-NSSAI, PDU Session and Service Data Flow (e.g. per UE per application) granularity; and

-	expose the Energy Consumption information defined in clause 5.51.2.4 to the authorized consumer NF(s) (AF/NEF).

Procedures for energy consumption information collection and exposure are defined in clause 4.29 of TS 23.502 [3].

In this Release of the specification, collecting and calculating of UE related Energy Consumption information is supported only for the UP resources of the 3GPP access type serving the UE in the 5GS.

==============Next change==============

#### 5.51.2.4	Energy Consumption information exposure

AF/NEF subscribes the EIF for Energy consumption information of required granularities (UE, S-NSSAI, PDU session and/or Service Data Flow).

-	For UE level energy exposure, the consumer NF provides UE ID (SUPI/GPSI).

-	For S-NSSAI of the UE level exposure, the consumer NF provides UE ID (SUPI/GPSI), S-NSSAI.

-	For PDU session level exposure, the consumer NF provides UE ID (SUPI/GPSI), DNN/S-NSSAI.

-	For Service Data Flow level exposure (e.g. per UE per application), the consumer NF provides UE ID (SUPI/GPSI), DNN/S-NSSAI and application information, e.g. Application Identifier, or Flow description(s).

The consumer AF/NEF may also subscribe the above information exposure with providing reporting period, or time window. An AF/NEF may also subscribe the Energy consumption information exposure of required granularity and additionally provide a reporting threshold for the required granularity and the reporting time period. If threshold-based reporting is configured for the reporting period, the consumer AF/NEF does not receive consumption information other than whether the threshold on energy consumption has been crossed in a reporting period. The information that the threshold has been crossed is sent to the AF/NEF as soon the threshold crossing is detected in a reporting period.

NOTE 1:	For the threshold-based reporting, the EIF sends the Energy consumption information of the required granularity to the NEF/AF once in the reporting period when the Energy consumption information of the required granularity matches or exceeds the threshold during the reporting period. If the threshold is not crossed in the reporting period, the EIF does not send any consumption information for the reporting period unless the periodic energy consumption reporting is separately requested without specifying the threshold for the reporting period.

NOTE 2:	In this Release of the specification, requesting energy-related information by the consumer is supported only for reporting periods and time windows occurring after the subscription time.

An EIF, if supported, may expose information additional to the energy consumption information at the requested granularity. If requested by the consumers of the energy consumption exposure services of the EIF, the additional information is provided together with the Energy Consumption information at the desired granularity and is the result of postprocessing of the information the EIF already has collected.

In conjunction with the energy consumption at the desired granularity, a supporting EIF may provide upon request from a consumer:

-	Energy Consumption per bit: this is computed by dividing the calculated energy consumption at the desired granularity by the measured data volume (in bits) at the desired granularity used to calculate the energy consumption at the desired granularity.

-	Energy Consumption Category: this is a scalar with value which is determined by the EIF by using PLMN-operator defined rules (e.g.  each Energy consumption category value could be defined one of N categories of percentile defined by the operator).

NOTE 3:	Both the rules used to define and determine the Energy Consumption Category values, and the Energy Consumption Category values are not in scope of 3GPP and defined by the PLMN operator for its customers.

NOTE 4:	The Energy Consumption Category is applicable to the same scope (e.g. regional, PLMN-wide) as the scope of the information that the EIF is collecting for the same granularity for other UEs that are e.g. associated to ongoing exposure subscriptions. The scope (e.g. regional, PLMN-wide) of the information an EIF collects is deployment specific.

==============End of changes==============