| 3GPP TS 33.369 V0.3.0 (2025-08) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Security aspects of Ambient IoT service for isolated private networks (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	8

4	Security requirements for AIoT service	9

4.1	General	9

4.2	Security Requirements	9

4.2.1	Requirements on the device 	9

4.2.1.1	Secure storage and processing of credentials	9

4.2.1.2 	Requirements related to authentication between device and network	9

4.2.1.3	Requirements for command protection	10

4.2.1.4	Requirements for identifier privacy	10

4.2.2	Requirements on the AIOTF	10

4.2.2.1	Requirement on Authentication	10

4.2.2.2	Requirements on Communication Protection	10

4.2.2.3	Requirements on Privacy	11

4.2.3	Requirements on the ADM	11

4.2.4	Security Requirements on the NG-RAN	11

5	Security procedures for Ambient IoT service	11

5.1	General	11

5.2	Authentication procedure	11

5.2.1	General	11

5.2.2	Authentication procedure	11

5.3	Protection of information during AIoT service communication	13

5.3.1	General	13

5.3.2	Security procedure on information protection during command procedure	14

5.3.3	Input parameters to integrity algorithm	15

5.3.4	Input parameters to ciphering algorithm	15

5.4	Protection of AIoT device identifier privacy	15

5.4.1	General	15

5.4.2	The AIoT device identifier protection for inventory with filtering information	16

5.4.3	Procedure for AIoT Device identifier protection with Temp ID update during Individual inventory	16

5.4.4	Out-of-Synch detection and Resynchronization of T-ID	17

5.5	Protection between AIoT network elements	17

Annex <A> (normative):	17

Key derivation functions	17

A.1	KDF interface and input parameter construction	17

A.1.1	General	17

A.1.2	FC value allocations	17

A.2	RESAIOT and XRESAIOT derivation function	18

A.3	KAIOTF derivation function	18

A.4	KCommand_enc and KCommand_int derivation function	18

Annex <B> (normative):	19

Temporary Identifier generation functions	19

B.1	T-ID generation	19

Annex <X> (informative): Change history	19

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the security and privacy aspects of AIoT services in the 5G System (5GS), complying to the requirements in TS 22.369 [4], applicable to the AIoT Device types, traffic types, use cases and connectivity topologies defined in TS 38.300 [3], and based on the architecture defined in TS 23.369 [2].

The AIoT system is defined as private network, i.e. isolated network deployment that does not interact with a public network, e.g. an SNPN.

Security features for AIoT services include:

1. Network Layer Authentication between AIoT device and 5G core

a. AIOTF is the endpoint in the 5G core

b. Credentials are securely stored in the ADM on the network side

NOTE 1: The credentials are assumed to be stored in a secure environment in the ADM. How this is realized is left to implementation. The requirements will reflect this.

c. Secure storage and processing of credentials in the AIoT device.

NOTE 2: For SNPN deployment the storage of the credentials of non-AKA based methods is out of scope as described in TS 33.501[5] Annex I 2.2.

d. Security aspects of the storage of the credentials at the ADM

2. Confidentiality, anti-replay and integrity protection of information during AIoT service communication

3. Privacy of AIoT device identifiers using the AIoT Temp ID.

4. Security to protect the permanent disabling RF transmission capabilities of AIoT device(s).

Editor’s Note: Further refinement is FFS.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]		3GPP TS 23.369: “Architecture support for Ambient power-enabled Internet of Things”.

[3]	3GPP TS 38.300: “NR; NR and NG-RAN Overall description; Stage-2”.

[4]	3GPP TS 22.369 “Service requirements for Ambient power-enabled IoT”.

[5]	3GPP TS 33.501 “Security architecture and procedures for 5G System”.

[6]	3GPP TS 38.391: "Ambient IoT Medium Access Control Protocol specification".

[7]	3GPP TS 33.220: “Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA”

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

AIoT Device: as specified in TS 23.369 [2].

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ADM	AIoT Data Management

AIoT	Ambient Internet of Things

AIOTF	Ambient IoT Function

# 4	Security requirements for AIoT service

## 4.1	General

Two functional cases are supported: inventory, command.

The AIoT RAN reader is assumed to be trusted, i.e., authorized from network side to communicate with the AIoT device.

## 4.2	Security Requirements

### 4.2.1	Requirements on the device

#### 4.2.1.1	Secure storage and processing of credentials

The long-term credentials used for authentication shall be securely stored and processed on the AIoT device.

The long-term credentials shall be protected against cloning when stored or processed.

The long-term credentials shall be confidentiality and integrity protected when stored and processed.

In the present document, the AIoT system is defined as private network (isolated network deployment that does not interact with a public network) e.g. SNPN, and the AIoT device credentials storage follows 3GPP defined requirements, the exact mechanism is out of scope of 3GPP (similar to Annex I.2.2 of TS 33.501 [5]). This means that no interconnection exists between AIoT systems and PLMNs.

NOTE 1: In case UICC is used, the exact form factor and whether it is removable, non-removable or integrated is out of scope of 3GPP.

NOTE 2: UICC provides protection for long term  credentials against physical and logical attacks.

#### 4.2.1.2 	Requirements related to authentication between device and network

The AIoT device shall support:

-	a method for pseudo-random bit generation.

Editor’s Note: Further cryptographic primitives are FFS.

#### 4.2.1.3	Requirements for command protection

The AIoT device shall support confidentiality protection of AIoT NAS messages between the AIoT device and the AIOTF.

Confidentiality protection of AIoT NAS messages between the AIoT device and the AIOTF is optional to use.

The AIoT device shall support the following ciphering algorithms:

NEA0 and 128-NEA2 as specified in Annex D of TS 33.501 [5].

The AIoT device shall support integrity protection and replay protection of AIoT NAS messages between the AIoT device and the AIOTF.

Integrity protection of AIoT NAS messages between the AIoT device and the AIOTF is mandatory to use.

The AIoT device shall support the following integrity algorithms:

128-NIA2 as specified in Annex D of TS 33.501 [5].

#### 4.2.1.4	Requirements for identifier privacy

-	The device shall support a mechanism for the use of temporary IDs.

Editor’s Note: this requirement will be revisited.

### 4.2.2	Requirements on the AIOTF

#### 4.2.2.1	Requirement on Authentication

The AIOTF shall authenticate the AIoT device.

#### 4.2.2.2	Requirements on Communication Protection

The AIOTF shall support confidentiality protection of AIoT NAS Command request and response between the AIoT device and the AIOTF.

The AIOTF shall support the following ciphering algorithms:

NEA0, 128-NEA2 as defined in Annex D of the TS 33.501 [5].

Confidentiality protection of AIoT NAS Command request and response between the AIoT device and the AIOTF is optional to use.

The AIOTF shall support integrity protection of AIoT NAS Command request and response between the AIoT device and the AIOTF.

The AIOTF shall support the following integrity algorithms:

128-NIA2 as defined in Annex D of the TS 33.501 [5].

Integrity protection of AIoT NAS Command request and response between the AIoT device and the AIOTF is mandatory to use.

The AIOTF shall support selection of confidentiality and integrity algorithms for protecting AIoT NAS Command request and response between the AIoT device and the AIOTF based on operator’s local policy.

#### 4.2.2.3	Requirements on Privacy

The AIOTF shall support a mechanism for the use of temporary IDs and it is optional for network to use.

### 4.2.3	Requirements on the ADM

For network layer authentication between AIoT device and 5G core, credentials shall be securely stored in the ADM. In case of SNPN, AIoT device credential can be stored in the credential holder instead of ADM.

NOTE:	Security mechanisms for storage of AIoT device credentials in the ADM are left to implementation.

### 4.2.4	Security Requirements on the NG-RAN

AIOT2 is the reference point between the AIOTF and the NG-RAN.

NG-RAN shall support the use of integrity, confidentiality and replay protection with the AIOTF over the AIOT2 interface.

# 5	Security procedures for Ambient IoT service

## 5.1	General

This clause describes the security procedures for Ambient IoT service. The requirements can be found in clause 4.

## 5.2	Authentication procedure

### 5.2.1	General

This clause describes the authentication procedure for Ambient IoT devices for both Inventory procedure and Command procedure when authentication is triggered by the network.

Editor’s Note: The alignment with the ID privacy procedure is FFS.

NOTE: KAIOT_root is the long-term key.

### 5.2.2	Authentication procedure

The authentication procedure is aligned with inventory procedure and command procedure in 6.2.2 and 6.2.3 of TS 23.369[2].

Figure 5.2.1-1: Authentication procedure

0. Step 1-6 of clause 6.2.2 Procedure for Inventory or clause 6.2.3 Procedure for command in TS 23.369 [2] is performed.

1. ADM shall generate RANDAIOT_n. AIOTF shall retrieve RANDAIOT_n from ADM.

2. AIOTF shall send inventory request message including RANDAIOT_n to NG-RAN.

3. NG-RAN shall include RANDAIOT_n in  the paging request message to the AIoT device in addition to other device identification information.

NOTE 1: An active attack may send a new paging request to the device while there is an ongoing procedure in device. The device will abort the ongoing procedure and respond to the new paging. The security measure to such denial-of-service attack is not specified in present document.

NOTE 2: While a legitimate network is performing an inventory operation, an attacker may cause amplification of resource exhaustion at the legitimate network side by sending AIoT paging messages for all devices or to a large group of devices, which causes large number of devices sending D2R messages to the legitimate network that the legitimate network does not expect to receive. The security measure to such amplification of resource exhaustion attack is not specified in present document.

4. Upon receiving the paging request message, if the device determines it needs to respond based on the device identification information, AIoT device shall generate RANDAIOT_d, calculate RESAIOT using KAIoT_root and RANDAIOT_n (see Annex A.2) for network authenticating AIoT Device.

Editor’s Note: the randomness of RANDAIOT_d is FFS.

5. AIoT device sends D2R message to the NG-RAN, including RESAIOT and RANDAIOT_d from device.

6.  NG-RAN sends Inventory report message to AIOTF, including the RESAIOT and RANDAIOT_d.

7. AIOTF sends device identification information, RANDAIOT_n  and RANDAIOT_d to ADM.

NOTE 3: the authentication is expected to be run more often than normal UE, (e.g., during each inventory procedure), which has load impact to ADM.

8. ADM shall calculate XRESAIOT using the same method as in AIoT device (see Annex A.2).

9. ADM sends XRESAIOT to AIOTF.

10. AIOTF verifies RESAIOT. If the verification is successful, for command case, AIOTF shall acquire KAIoTF from ADM. ADM shall calculate KAIoTF if receiving request from AIOTF (see AnnexA.2). ADM sends KAIoTF to AIOTF.

The steps 12-14 in clause 6.2.2 for inventory procedure or the step 8-11of clause 6.2.3 for command procedure in TS 23.369 [2] continues.

For the command procedure, the AIoT device implicitly authenticates the network via the verification of MAC which is derived using the KCommand_int as specified in clause 5.2.3 of present document.

## 5.3	Protection of information during AIoT service communication

### 5.3.1	General

This clause describes the security procedures for the information protection in command message. The protection of information is provided as part of the AIoT NAS protocol between AIoT device and AIOTF. The AIOTF acts as the security termination point for AIoT information protection.

### 5.3.2	Security procedure on information protection during command procedure

Figure 5.3.2-1:  Security procedure on the information protection during command procedure

1.	The command procedure is initiated as specified in step 1-6 of clause 6.2.3 of TS 23.369 [2].

2.	The procedure as described in clause 5.2.2 shall be performed. The device and AIOTF acquire the KAIOTF key to be used for command protection. The derivation of KAIOTF key is specified in Annex A.3.

3.	The AIoTF shall construct a AIOT NAS Command Request and protect the message based on the KCommand_enc, KCommand_int, the confidentiality and integrity algorithms for the AIoT device. The AIoTF shall send the protected Command Request containing the selected protection algorithms in clear text to NG-RAN.

NOTE 1: The whole AIOT NAS Command Request message is integrity protected. If confidentiality algorithm is not null-scheme, the AIOT NAS Command Request message is partly ciphered with the exception that the selected protection algorithms are in clear text.

4.	The NG-RAN shall send a R2D message containing the protected AIOT NAS Command Request as specified in as specified in TS 38.300 [3] and TS 38.391 [6] .

5.	The device shall derive the KCommand_enc, KCommand_int and verify the integrity of the command message. If the verification of integrity is successful, the AIoT device shall decipher it in case it is confidentiality protected. The AIoT device shall construct a NAS Command Response and protect the message based on the KCommand_enc and KCommand_int key using the same algorithms.

6.	The AIoT device shall send a D2R message containing the protected AIOT NAS Command Response to the NG-RAN as specified in as specified in TS 38.300 [3] and TS 38.391 [6].

7.	The NG-RAN shall forward the AIOT NAS Command Response containing the protected AIOT NAS Command Response to the AIoTF.

8-9.	The AIoTF shall verify the integrity of the command message. If the verification of integrity is successful, the AIoT device shall decipher it in case it is confidentiality protected. Then, the AIOTF shall continue the procedure as specified in clause 6.2.3 of TS 23.369 [2].

NOTE 2: It is assumed that there is only one round of command procedure per device following an inventory procedure. Since the KAIOTF key is fresh, there is no need for additional freshness parameters for replay protection.

NOTE 3: It is assumed that no new algorithms will ever be introduced for information protection during command procedure.

### 5.3.3	Input parameters to integrity algorithm

The input parameters to the integrity algorithm as described in Annex D.3 in TS 33.501[5] shall be set as follows.

The KEY input is equal to the K Command_int key.

The DIRECTION bit is set to 0 for uplink and 1 for downlink.

The BEARER is set to all zeros.

The COUNT is set to all zeros.

### 5.3.4	Input parameters to ciphering algorithm

The input parameters for the ciphering algorithms shall be the same as the ones used for NAS integrity protection as described in clause 5.3.3, with the exception that there is an additional input parameter, namely the length of the key stream to be generated by the ciphering algorithms and the KEY input is equal to the KCommand_enc key.

## 5.4	Protection of AIoT device identifier privacy

### 5.4.1	General

This clause describes the mechanisms to protect AIoT device identifier privacy during the inventory procedure. The mechanism is based on the use of a Temporary ID (i.e., T-ID). The T-ID is generated based on the key (i.e., KAIoT_root) shared between AIoT device and ADM. Depending on the situation and deployment scenario, the network operator can choose which paging procedure to use.

When privacy protection is not used during the inventory procedure, the AIoT device includes its AIoT device permanent identifier as a device identification information in the procedure specified in clause 5.2.2.

### 5.4.2	The AIoT device identifier protection for inventory with filtering information

For the protection of AIoT device permanent ID during the inventory procedure described in clause 5.2.2, the following change shall apply:

- 	In step 4, the AIoT device determines it needs to reply to the NG-RAN based on the received filtering information.

NOTE 1: The attacker may obtain a AIoT device ID by performing a bitwise enumeration in multiple paging messages. To mitigate the attack, the AIoT device need to be configured with filtering information to match by limiting which bits of AIoT device identifier is allowed for filtering information (guidance would be to limit to the leftmost n bits of the permanent device identifier, e.g., only allow filtering information for the leftmost 64 bits and not respond otherwise).

-	In step 5 and 6, a device identification information is not included in the D2R message and Inventory Report message.

-	In step 7, filtering information is used as a device identification information if the AIOTF received it in step 0.

NOTE 2: The AIOTF identifies the AIoT device by checking the received RESAIoT. Therefore, device identification information is not needed in the D2R and Inventory Report message.

NOTE 3: When inventory with filtering information is used, after receiving the D2R message, the ADM has to exhaustively derive XRESAIoTs with all the long-term keys (i.e., KAIoT_root) of the AIoT devices in the group that was paged for every RANDAIoT_d received. The AIOTF then, need to check XRESAIoT with the received RESAIoT. Therefore, the size of the group should be chosen accordingly to reduce the energy consumption, inter NF interaction, and latency.

### 5.4.3	Procedure for AIoT Device identifier protection with Temp ID update during Individual inventory

For the protection of AIoT device permanent identifier during the inventory procedure with AIoT device identifier described in clause 5.2.2, the following changes shall apply:

-	In step 1, AIOTF shall retrieve a T-ID in addition to the RANDAIOT_n from ADM. The ADM shall, based on T-ID type, either fetch the stored T-ID in the AIoT device profile or generate the T-ID as specified in Annex B.1.

-	In step 2, 3 and 4, the T-ID shall be used as a device identification information.

-	In step 2 and 3 the AIOTF includes indication of type of T-ID handling. T-ID can be either concealed type or stored type. The concealed type can be based on either the stored T-ID or the permanent identifier. If needed the handling also indicates whether the stored T-ID type shall be updated with or without a command. NG-RAN includes the T-ID handling in the paging message.

-	In step 4, the AIoT device, based on the T-ID handling indication in the paging message, generates the T-ID in the same way as the ADM did in step 1. The AIoT device determines it needs to reply to the NG-RAN if the generated T-ID matches with the received T-ID. In case the stored T-ID update shall be done without a command, the AIoT Device generates a new Temp_ID_n+1 as specified in Annex B.1 and stores the new Temp ID_n+1.

-	In step 5 and 6, a device identification information is not included in the D2R message and Inventory Report message.

-	In step 7, the AIoT device permanent identifier is used as a device identification information. AIOTF requests the ADM to derive a new T-ID as specified in Annex B.1 and to store it in the AIoT Device profile.

NOTE 1:	The AIOTF identifies the AIoT device by checking the received RESAIoT parameter. Therefore, the device identification information is not needed in the D2R message and Inventory Report message.

NOTE 2: 	In case of concealed T-ID type, every AIoT devices that receive an Inventory Request with T-ID need to perform a T-ID matching by generating a T-ID based on the KAIoT_root and check if the generated T-ID is matched with the received T-ID. It is assumed that the AIoT device that receive the Inventory Request has enough energy to perform this T-ID matching in addition to the Inventory procedure specified in clause 5.2.2.

NOTE 3: 	In case of stored T-ID type, the stored T-IDs on the device side and network side can get out-of synch. The handling of such situation is described in clause 5.4.3.

### 5.4.4	Out-of-Synch detection and Resynchronization of T-ID

In case the network does not receive an Inventory Response from a AIoT Device after an Individual Inventory Request, then it can indicate that the AIoT Device and network is out-of-synch with the TIDs. The out-of-synch can happen if e.g.:

- The Inventory Response or Command Response from the Device was lost during transmission due to radio link issues e.g. interference, range, etc. in that case the AIoT Device would generate the T-ID_n+1, but the ADM would not generate the T-ID_n+1 or know that the device has received the T-ID_n+1 as it did not get any response.

- Something went wrong during the Inventory procedure e.g. the AIoT Device managed to write to the NVM but not send the inventory response or command response or the AIoT Device sent the inventory response or command response but was not able to write to the NVM.

This means that the ADM either has a T-ID that is older or newer than the T-ID in the AIoT Device. They can never be more than one off.

T-ID sequence recovery is possible if the network performs Individual Inventory with both T-ID_n-1 or T-ID_n+1. When the AIoT device responds to the network, the network adjusts the sequence, and both are in synch again.

Alternatively, the network can use concealed T-ID type using the permanent identifier and then send a command to provide a new T-ID to the device which it stores in the device.

## 5.5	Protection between AIoT network elements

For the interfaces specified in clause 4.3 of TS 23.369 [2], the security procedures specified in clause 13 in TS 33.501 [5] applies to the service-based interfaces within 5G core network for Ambient IoT. The mechanism described in clause 12.3 of TS 33.501 [5] applies to the NEF-AF interface.

The security mechanism specified for N2, between 5G-AN and AMF defined in clause 9.2 of TS 33.501 [5], applies to the AIOT2 interface between AIOTF and NG-RAN.

# Annex <A> (normative):

Key derivation functions

# A.1	KDF interface and input parameter construction

## A.1.1	General

All key derivations (including input parameter encoding) for 5GC shall be performed using the key derivation function (KDF) specified in Annex B.2.0 of TS 33.220 [yy].

This clause specifies how to construct the input string, S, and the input key, KEY, for each distinct use of the KDF. Note that "KEY" is denoted "Key" in TS 33.220 [yy].

## A.1.2	FC value allocations

The FC number space used is controlled by TS 33.220 [7], FC value allocated for the present document is 0xAA-0xZZ.

# A.2	RESAIOT and XRESAIOT derivation function

When deriving a RESAIOT and XRESAIOT from KAIOT_root, the following parameters shall be used to form the input S to the  KDF:

-	FC = 0xZZ,

-	P0 = RANDAIOT_n,

-	L0 = length of RANDAIOT_n (i.e. 0x00  0x10),

-	P1 = RANDAIOT_d.

-	L1 = length of RANDAIOT_d (i.e. 0x00  0x10),

-	P2 = AIoT device permanent identifier,

-	L2 = length of AIoT device permanent identifier,

The input key KEY shall be KAIOT_root.

# A.3	KAIOTF derivation function

When deriving a KAIOTF from KAIOT_root, the following parameters shall be used to form the input S to the  KDF:

-	FC = 0xZZ,

-	P0 = RANDAIOT_n,

-	L0 = length of RANDAIOT_n (i.e. 0x00  0x10),

-	P1 = RANDAIOT_d.

-	L1 = length of RANDAIOT_d (i.e. 0x00  0x10),

The input key KEY shall be the KAIOT_root.

# A.4	KCommand_enc and KCommand_int derivation function

When deriving a KCommand_enc or KCommand_int from KAIOTF, the following parameters shall be used to form the input S to the  KDF:

-	FC = 0xZZ,

-	P0 = algorithm identity as specified in TS 33.501[5].

-	L0 = length of algorithm identity (i.e. 0x00 0x01)

The input key KEY shall be the KAIOTF.

Annex <B> (normative):

Temporary Identifier generation functions

## B.1	T-ID generation

When generating a temporary ID (i.e., T-ID) from KAIOT_root, the following parameters shall be used to form the input S to the  KDF:

-	FC = 0xNN,

-	P0 = Temp_n,

-	L0 = length of Temp_n,

-	P1 = RANDAIOT_n,

-	L1 = length of RANDAIOT_n

The input key KEY shall be KAIOT_root. The P0 input is either the stored Temp ID_n or AIoT device Permanent ID.

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 04/2025 | SA3#121 |  |  |  |  | Initial version | 0.0.0 |
| 04/2025 | SA3#121 | S3-251706 |  |  |  | Incorporated accepted contributions: S3-251702, S3-251703, S3-251784 | 0.1.0 |
| 05/2025 | SA3#122 | S3-252287 |  |  |  | Incorporated accepted contributions: S3-252279, S3-252280, S3‑252281, S3‑252410, S3‑252283, S3‑252282, S3-252262, S3‑252308, S3‑252309, S3‑252412, S3-252310 | 0.2.0 |
| 08/2025 | SA3#123 | S3-252941 |  |  |  | Incorporated accepted contributions: S3-252934, S3-252942, S3-252943, S3-252944, S3-252945, S3-252996, S3-252997, S3-252998, S3-252999, S3-253051, S3-253059 | 0.3.0 |