3GPP TSG-SA3 Meeting #123	draft_S3-252942-r5

# Goteborg, Sweden, 25 – 29 August 2025						merge of S3-252863, S3-252726, S3-252655, S3-252848, S3-252558, S3-252611, S3-252827, S3-252806, S3-252837, S3-252852, S3-252853, S3-252865, S3-252847, S3-252754…

Source:	Xiaomi, Nokia, Huawei, Lenovo, OPPO, Vivo, ZTE, Ericsson, Qualcomm, CATT, Apple, Samsung

Title:	Addressing ENs in AIoT authentication procedure

Document for:	Approval

Agenda item:	4.1.1

Spec:	3GPP TS 33.369

Version:	0.2.0

Work Item:	AmbientIoT-SEC

Comments

It is proposed to address the following ENs in the AIoT authentication procedure.

Editor’s Note: Whether ADM or AIOTF generates RANDAIOT_n is FFS.

To reduce the interaction between the AIOTF and ADM, it is proposed that AIOTF generate RANDAIOT_n for authentication. Otherwise, one additional round of messages is necessary for transmitting the RANDAIOT_n, i.e. one message from the AIOTF to ADM for requesting the generation of RANDAIOT_n, and one message from the ADM to AIOTF for providing the RANDAIOT_n.

Editor’s Note: Whether replay attack is possible is FFS.

Editor’s Note: Whether RANDAIOT_d is required for inventory procedure is FFS.

Since the Inventory request is not protected, the replay attack can always be launched. However, the device authentication is determined by verifying the RES, which is generated at least by using RANDAIOT_n included in the Inventory request. If the verification of RES fails, the replay attack can be detected by the network side. Therefore, the attacker cannot manipulate the AIoT system by launching a replay attack during the inventory procedure.

During the authentication procedure, each peer entity should provide a random number to ensure freshness. Therefore, it is proposed to include RANDAIOT_d in the inventory response and use it as one of the input parameters for generating the RES. With RANDAIOT_d and RANDAIOT_n, the security of authentication can be further enhanced.

Editor’s Note: How RESAIOT is derived and whether it is derived from KAIoT or intermediate key is FFS.

Editor’s note: The impact of interaction between AIOTF and ADM is FFS. If the authentication is expected to be run more often than normal UE, (e.g., during each inventory procedure), the analysis of load of ADM is FFS.

Considering the storage limitation of AIoT device, it is proposed that the KAIOTF is refreshed during each inventory + command procedure. In this case, the AIoT device do not need to store the security context, which can meet the requirement from RAN. The KAIOTF can be generated by the ADM from KAIOT and provided to the AIOTF to protect the command message. Regarding the load of ADM, it can be left to the implementation of the private network.

Considering the security functionality of AIOTF and ADM, and the primary authentication procedure in SNPN (clause I.2.2.2.2 of TS 33.501), it is proposed that ADM calculate XRES by using the KAIoT and determine the authentication result by verifying RES. Therefore, the security functionality of AIOT system will be:

- The ADM is responsible for the authentication and credential management.

- The AIOTF is responsible for the communication protection.

Editor’s note: How to perform the mutual authentication for command procedure will be specified.

To save the calculation resources and transmission resources of AIoT device, the implicit authentication method can be used for mutual authentication. By verifying the security of command request message, the AIoT device can implicitly determine the authentication result to the network.

* * * First Change * * * *

### 5.2.2	Authentication procedure

The authentication procedure is aligned with inventory procedure and command procedure in 6.2.2 and 6.2.3 of TS 23.369[2].

Figure 5.2.1-1: Authentication procedure

0. Step 1-6 of clause 6.2.2 Procedure for Inventory or clause 6.2.3 Procedure for command in TS 23.369 [2] is performed.

1. ADM shall generate RANDAIOT_n. AIOTF shall retrieve RANDAIOT_n from ADM.

2. AIOTF shall include RANDAIOT_n in the inventory request message to NG-RAN, in addition to other device identification information.

Editor’s Note: The inclusion of RANDAIOT_n in Paging Request and the size of RANDAIOT_n needs RAN confirmation.

3. NG-RAN shall include RANDAIOT_n in the paging request message to the AIoT device in addition to other device identification information.

Editor’s Note: Whether replay attack is possible is FFS.

4. Upon receiving the paging request message, if the device determines it needs to respond based on the device identification information, AIoT device shall generate RANDAIOT_d using the pseudo-random bit generation. AIoT device shall calculate RESAIOT using KAIoT, RANDAIOT_d, and RANDAIOT_n (see Annex X) for network authenticating AIoT Device.

5. AIoT device sends D2R message to the NG-RAN, including RESAIOT and RANDAIOT_d from device.

Editor’s note: Device identification information is FFS.

6. NG-RAN sends Inventory report message to AIOTF, including the RESAIOT and RANDAIOT_d.

7. AIOTF sends device identification information, RANDAIOT_n and RANDAIOT_d to ADM.

8. ADM shall calculate XRESAIOT using the same method as in AIoT device (see Annex X).

9. ADM sends XRESAIOT to AIOTF.

10. AIOTF verifies RESAIOT. If the verification is successful, the steps 12-14 in clause 6.2.2 for inventory procedure or the step 8-11of clause 6.2.3 for command procedure in TS 23.369 [2] continues.

For the command procedure, the AIoT device implicitly authenticates the network via the verification of MAC which is derived using the KCommand_int as specified in clause 5.2.3 of present document.

* * * Second Change * * * *

Annex X (normative): 
Key derivation functions

# X.1	KDF interface and input parameter construction

## X.1.1	General

All key derivations (including input parameter encoding) for 5GC shall be performed using the key derivation function (KDF) specified in Annex B.2.0 of TS 33.220 [yy].

This clause specifies how to construct the input string, S, and the input key, KEY, for each distinct use of the KDF. Note that "KEY" is denoted "Key" in TS 33.220 [yy].

## X.1.2	FC value allocations

The FC number space used is controlled by TS 33.220 [yy], FC value allocated for the present document is 0xAA-0xZZ.

# X.Y	RESAIOT and XRESAIOT derivation function

When deriving a RESAIOT and XRESAIOT from KAIOT, the following parameters shall be used to form the input S to the  KDF:

-	FC = 0xZZ,

-	P0 = RANDAIOT_n,

-	L0 = length of RANDAIOT_n (i.e. 0x00  0x10),

-	P1 = RANDAIOT_d.

-	L1 = length of RANDAIOT_d (i.e. 0x00  0x10),

-	P2 = AIoT device permanent identifier,

-	L2 = length of AIoT device permanent identifier,

The input key KEY shall be KAIOT.

* * * End of Changes * * * *