3GPP TSG-SA3 Meeting #123	S3-253036-r1

Goteborg, Sweden, 25 – 29 August 2025

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 33.938 | CR | 0003 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial changes to TR 33.938 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S3 |  |  |  |  |  |  |  |  |  |
| Work item code: | FS_CryptoInv |  |  |  |  |  | Date: |  |  | 2025-08-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The Editor identified missing references in the TR 33.938. In addtition, editional changes are proposed to correct typos in the TR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of missing references and correction of typos. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incompleted TR |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 3.1, 4.2.12, 4.3.1, 4.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Merger of S3-252784, S3-252783, S3-252818, S3-252819 |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 33.210: "3G security; Network Domain Security (NDS); IP network layer security".

[3]	3GPP TS 33.310: "Network Domain Security (NDS); Authentication Framework (AF)".

[4]	3GPP TS 33.501: “Security architecture and procedures for 5G system”.

[5]	IETF RFC 9190: "EAP-TLS 1.3: Using the Extensible Authentication Protocol with TLS 1.3".

[6]	IETF RFC 5216: "The EAP-TLS Authentication Protocol".

[7]	SECG SEC 1: “Recommended Elliptic Curve Cryptography”, Version 2.0, 2009. Available at http://www.secg.org/sec1-v2.pdf.

[8]	SECG SEC 2: “Recommended Elliptic Curve Domain Parameters”, Version 2.0, 2010. Available at http://www.secg.org/sec2-v2.pdf.

[9]	IETF RFC 9001: "Using TLS to Secure QUIC".

[10]	IETF RFC 8152: "CBOR Object Signing and Encryption (COSE)".

[11]	3GPP TS 33.220: “Generic Authentication Architecture (GAA); Generic Bootstrapping Architecture (GBA)”.

[12]	IETF RFC 8613: "Object Security for Constrained RESTful Environments (OSCORE)".

[13]	3GPP TS 33.180: "Security of the Mission Critical (MC) service".

[14]	IETF RFC 6509: ''MIKEY-SAKKE: Sakai-Kasahara Key Encryption in Multimedia Internet KEYing (MIKEY)''.

[15]	IETF RFC 5448: "Improved Extensible Authentication Protocol Method for 3rd Generation Authentication and Key Agreement (EAP-AKA')".

[16]	3GPP TS 35.205: "3G Security; Specification of the MILENAGE algorithm set: An example algorithm set for the 3GPP authentication and key generation functions f1, f1*, f2, f3, f4, f5 and f5*".

[17]	3GPP TS 35.231: "Specification of the TUAK algorithm set: A second example algorithm set for the 3GPP authentication and key generation functions f1, f1*, f2, f3, f4, f5 and f5*; Document 1: Algorithm specification".

[18]	3GPP TS 35.234: "Specification of the MILENAGE-256 algorithm set; An example set of 256-bit 3GPP authentication and key generation functions f1, f1*, f2, f3, f4, f5, f5* and f5**; Document 1: General".

[19]	NIST IR 8547 ipd: “Transition to Post-Quantum Cryptography Standards”

[20]	IETF RFC 9147: "The Datagram Transport Layer Security (DTLS) Protocol Version 1.3".

[21]	IETF RFC 8446: "The Transport Layer Security (TLS) Protocol Version 1.3".

[22]	IETF RFC 6960: " X.509 Internet Public Key Infrastructure Online Certificate Status Protocol - OCSP".

[23]	IETF RFC 7296: " Internet Key Exchange Protocol Version 2 (IKEv2)".

[24]	IETF RFC 4303: "IP Encapsulating Security Payload (ESP)".

[25]	IETF RFC 8221: "Cryptographic Algorithm Implementation Requirements and Usage Guidance

for Encapsulating Security Payload (ESP) and Authentication Header (AH)".

[26]	IETF RFC 8750: "Implicit Initialization Vector (IV) for Counter-Based Ciphers in Encapsulating Security Payload (ESP)".`

[27]	IETF RFC 7516: "JSON Web Encryption".

[28]	IETF RFC 7515: "JSON Web Signature (JWS)".

[29]	IETF RFC 6507: “Elliptic Curve-Based Certificateless Signatures for Identity-Based Encryption (ECCSI)”

[30]	IETF RFC 6508: “Sakai-Kasahara Key Encryption (SAKKE)''

[31]	IETF RFC 5869: "HMAC-based Extract-and-Expand Key Derivation Function (HKDF)".

[32]	IETF RFC 4303: "IP Encapsulating Security Payload (ESP)".

[33]	IETF RFC 3602: "The AES-CBC Cipher Algorithm and Its Use with IPsec".

[34]	IETF RFC 4106: "The Use of Galois/Counter Mode (GCM) in IPsec Encapsulating Security Payload (ESP)".

[35]	IETF RFC 4543: "The Use of Galois Message Authentication Code (GMAC) in IPsec ESP and AH".

[36]	IETF RFC 4868: "Using HMAC-SHA-256, HMAC-SHA-384, and HMAC-SHA-512 with IPs".

[37]	IETF RFC 6347: “Datagram Transport Layer Security Version 1.2”.

[38]	IETF RFC 5246: “The Transport Layer Security (TLS) Protocol Version 1.2”.

[39]	IETF RFC 5281: "Extensible Authentication Protocol Tunnelled Transport Layer Security        Authenticated Protocol Version 0 (EAP-TTLSv0)".

[40]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[41]	IETF RFC 6750: "The OAuth 2.0 Authorization Framework: Bearer Token Usage".

[42]	IETF RFC 7519: "JSON Web Token (JWT)".

[43]	3GPP TS 29.500: "Technical Realization of Service Based Architecture".

[44]	3GPP TS 38.323: "Packet Data Convergence Protocol (PDCP) specification".

[45]	IETF RFC 8017: "PKCS#1: RSA Cryptography Specifications Version 2.2".

[46]	IETF RFC 4754: "IKE and IKEv2 Authentication Using the Elliptic Curve Digital Signature Algorithm (ECDSA)".

[47]	NIST FIPS PUB 180-4: "Secure Hash Standard (SHS)".

[48]	IETF RFC 8442: "ECDHE_PSK with AES-GCM and AES-CCM Cipher Suites for TLS 1.2 and DTLS 1.2”.

* * * Next Change * * * *

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Asymmetric Cryptography (NIST IR 8547 [19]): Also known as public-key cryptography, it is the cryptography that uses two separate keys to exchange data: one to encrypt or digitally sign the data and one to decrypt the data or verify the digital signature.

Key Agreement (NIST IR 8547 [19]): A (pair-wise) key-establishment procedure where the resultant secret keying material is a function of information contributed by two participants so that no party can predetermine the value of the secret keying material independently from the contributions of the other party.

Key Derivation (NIST IR 8547 [19]): The process of deriving a key in a non-reversible manner from shared information, some of which is secret.

Symmetric Key Cryptography (NIST IR 8547 [19]): A cryptographic algorithm that uses the same secret key for its operation and, if applicable, for reversing the effects of the operation.

* * * Next Change * * * *

### 4.2.12	NAS security

The NAS security mechanisms are to protect NAS signaling and data between the UE and the AMF over the N1 reference point in 5G system:

-	NAS signaling integrity and confidentiality protection between UE and AMF (see clause 6.4.3 and 6.4.4 of TS 33.501 [4]).

-	User plane data (SMS over NAS) integrity and confidentiality protection between UE and AMF (see clause 6.4.7 and 6.16 of TS 33.501 [4]).

NAS security protocol employs symmetric cryptography for confidentiality and integrity protection.

* * * Next Change * * * *

## 4.3	Summary Tables

### 4.3.1	3GPP Symmetric Cryptographic Algorithms

The following table summarizes the security related protocols used in 3GPP employing symmetric cryptographic algorithms including hash functions (5G System).

Table 4.3.1-1: Protocols Used in 3GPP Employing Symmetric Cryptographic Algorithms (5G System)

| Protocol/Function | Protocol Profile, Clauses | Cryptographic Algorithm(s) | Feature(s), Usage Type |
| --- | --- | --- | --- |
| MIKEY-SAKKE (IETF RFC 6509) [14] | IETF RFC 6509 [14], Appendix A | SHA-256 | Hash Function |
| KDF(TS 33.220, Clause B.2 [11]) | TS 33.220 [11], Clause B.2.0 | HMAC-SHA-256 | Session Key Derivation |
|  | TS 33.501 [4], Clause C.3 | ANSI-X9.63-KDF | Session Key Derivation |
| ECIES ([7], [8]) | TS 33.501 [4], Clause C.3 | SHA-256, HMAC-SHA-256, | Session Key Derivation |
|  |  | HMAC–SHA-256 | Integrity Protection |
|  |  | AES-128-CTR | Confidentiality Protection |
| PKI | TS 33.310 [3], Clause 6.1.1 | SHA-256 SHA-384 | Hash Function |
| OCSP (IETF RFC 6960 [22]) | TS 33.310 [3], Clause 6.1b | SHA-256 SHA-384 | Hash Function |
| COSE (IETF RFC 8152[10]) | TS 33.220 [11], Clause P.3.3 | HMAC-based KDF with SHA-256 [31] | Session Key Derivation / Hash Function |
|  |  | AES-CCM-16-64-128 | Confidentiality and Integrity Protection |
| EAP-TLS (IETF RFCs 9190 [5], 5216 [6]) | TS 33.501 [4], Clause B.2.1 | AEAD_AES_128_GCM | Confidentiality and Integrity Protection |
|  |  | HKDF (RFC5869 [31]) | Session Key Derivation |
| EAP TTLS (IETF RFC 5281 [39]) | TS 33.501 [4], Annex U TS 33.210 [2] clause 6.2 for TLS | See TLS in this table | Confidentiality and Integrity Protection |
|  |  |  | Session Key Derivation |
| OAuth 2.0 (IETF RFC 6749 [40], 6750 [41]) | TS 33.210 [2] clause 6.2 for TLS | See TLS 1.2 and TLS 1.3 in this table | Confidentiality and Integrity Protection |
|  |  |  | Hash Function |
|  | TS 33.210 [2] clause 6.3 for JWE/JWS | See JWE and JWS in this table | Confidentiality and Integrity Protection |
|  |  |  | Hash Function |
| IKEv2 (IETF RFC 7296 [23]) | TS 33.210 [2] clause 5.4 | 128-AES GCM SHA-256 (IETF RFC 8442 [48]) 256-AES GCM SHA-384 (IETF RFC 8442 [48]) | Confidentiality and Integrity Protection |
|  | TS 33.310 [3] clauses 5,6,7 | SHA2-256/384 [47] | Hash Function |
| PDCP security (TS 38.323 [44]) | TS 33.501 [4], Annex D | 128-NEA1, 128-NIA1 128-NEA2, 128-NIA2 128-NEA3, 128-NIA3 | Confidentiality and Integrity Protection |
| NAS security (TS 33.501 [4]) | TS 33.501 [4], Annex D | 128-NEA1, 128-NIA1 128-NEA2, 128-NIA2 128-NEA3, 128-NIA3 | Confidentiality and Integrity Protection |
| IPsec ESP (IETF RFCs 4303 [32], 8221 [25], 8750 [26]) | TS 33.210 [2] | ENCR_AES_CBC (IETF RFC 3602 [33]) | Confidentiality Protection |
|  |  | ENCR_AES_GCM_16 (IETF RFC 4106 [34]) ENCR_AES_GCM_16_IIV (IETF RFC 8750 [26]) | Confidentiality and Integrity Protection |
|  |  | AUTH_AES_128_GMAC (IETF RFC 4543 [35]) AUTH_HMAC_SHA2_256_128 (IETF RFC 4868 [36]) | Authentication |
| DTLS 1.2 (IETF RFC 6347 [37]) | TS 33.210 [2] clause 6.2.1 | See TLS 1.2 in this table | Confidentiality and Integrity Protection |
| DTLS 1.3 (IETF RFC 9147 [20]) | TS 33.210 [2] clause 6.2.1 | See TLS 1.3 in this table | Confidentiality and Integrity Protection |
| TLS 1.2 (IETF RFC 5246 [38]) | TS 33.210 [2] clauses 6.2.1, 6.2.3 | AES_128_GCM, AES_256_GCM | Confidentiality and Integrity Protection |
|  |  | SHA256, SHA384 | Hash Function |
| TLS 1.3 (IETF RFC 8446 [21]) | TS 33.210 [2] clauses 6.2.1, 6.2.2 | AES_128_GCM, AES_256_GCM, CHACHA20_POLY1305 | Confidentiality and Integrity Protection |
|  |  | SHA-256, SHA-384 | Hash Function |
| JWE (IETF RFC 7516 [27]) | TS 33.210 [2] clauses 6.3.1, 6.3.2 | AES_128_GCM, AES_256_GCM | Confidentiality and Integrity Protection |
| JWS (IETF RFC 7515 [28]) | TS 33.210 [2] clauses 6.3.1, 6.3.3 | SHA-256 | Hash Function |

| Protocol/Function | Protocol Profile, Clauses | Cryptographic Algorithm(s) | Feature(s), Usage Type |
| --- | --- | --- | --- |
| COSE (IETF RFC 8152[10]) | TS 33.220 [11], Clause P.3.3 | HMAC-based KDF with SHA-256 [31] | Session Key Derivation / Hash Function |
|  |  | AES-CCM-16-64-128 | Confidentiality and Integrity Protection |
| DTLS 1.2 (IETF RFC 6347 [37]) | TS 33.210 [2] clause 6.2.1 | See TLS 1.2 in this table | Confidentiality and Integrity Protection |
| DTLS 1.3 (IETF RFC 9147 [20]) | TS 33.210 [2] clause 6.2.1 | See TLS 1.3 in this table | Confidentiality and Integrity Protection |
| EAP-TLS (IETF RFCs 9190 [5], 5216 [6]) | TS 33.501 [4], Clause B.2.1 | AEAD_AES_128_GCM | Confidentiality and Integrity Protection |
|  |  | HKDF (RFC5869 [31]) | Session Key Derivation |
| EAP TTLS (IETF RFC 5281 [39]) | TS 33.501 [4], Annex U TS 33.210 [2] clause 6.2 for TLS | See TLS in this table | Confidentiality and Integrity Protection |
|  |  |  | Session Key Derivation |
| ECIES ([7], [8]) | TS 33.501 [4], Clause C.3 | SHA-256, HMAC-SHA-256, | Session Key Derivation |
|  |  | HMAC–SHA-256 | Integrity Protection |
|  |  | AES-128-CTR | Confidentiality Protection |
| IKEv2 (IETF RFC 7296 [23]) | TS 33.210 [2] clause 5.4 | 128-AES GCM SHA-256 (IETF RFC 8442 [48]) 256-AES GCM SHA-384 (IETF RFC 8442 [48]) | Confidentiality and Integrity Protection |
|  | TS 33.310 [3] clauses 5,6,7 | SHA2-256/384 [47] | Hash Function |
| IPsec ESP (IETF RFCs 4303 [32], 8221 [25], 8750 [26]) | TS 33.210 [2] | ENCR_AES_CBC (IETF RFC 3602 [33]) | Confidentiality Protection |
|  |  | ENCR_AES_GCM_16 (IETF RFC 4106 [34]) ENCR_AES_GCM_16_IIV (IETF RFC 8750 [26]) | Confidentiality and Integrity Protection |
|  |  | AUTH_AES_128_GMAC (IETF RFC 4543 [35]) AUTH_HMAC_SHA2_256_128 (IETF RFC 4868 [36]) | Authentication |
| JWE (IETF RFC 7516 [27]) | TS 33.210 [2] clauses 6.3.1, 6.3.2 | AES_128_GCM, AES_256_GCM | Confidentiality and Integrity Protection |
| JWS (IETF RFC 7515 [28]) | TS 33.210 [2] clauses 6.3.1, 6.3.3 | SHA-256 | Hash Function |
| KDF(TS 33.220, Clause B.2 [11]) | TS 33.220 [11], Clause B.2.0 | HMAC-SHA-256 | Session Key Derivation |
|  | TS 33.501 [4], Clause C.3 | ANSI-X9.63-KDF | Session Key Derivation |
| MIKEY-SAKKE (IETF RFC 6509) [14] | IETF RFC 6509 [14], Appendix A | SHA-256 | Hash Function |
| NAS security (TS 33.501 [4]) | TS 33.501 [4], Annex D | 128-NEA1, 128-NIA1 128-NEA2, 128-NIA2 128-NEA3, 128-NIA3 | Confidentiality and Integrity Protection |
| OAuth 2.0 (IETF RFC 6749 [40], 6750 [41]) | TS 33.210 [2] clause 6.2 for TLS | See TLS 1.2 and TLS 1.3 in this table | Confidentiality and Integrity Protection |
|  |  |  | Hash Function |
|  | TS 33.210 [2] clause 6.3 for JWE/JWS | See JWE and JWS in this table | Confidentiality and Integrity Protection |
|  |  |  | Hash Function |
| OCSP (IETF RFC 6960 [22]) | TS 33.310 [3], Clause 6.1b | SHA-256 SHA-384 | Hash Function |
| PDCP security (TS 38.323 [44]) | TS 33.501 [4], Annex D | 128-NEA1, 128-NIA1 128-NEA2, 128-NIA2 128-NEA3, 128-NIA3 | Confidentiality and Integrity Protection |
| PKI | TS 33.310 [3], Clause 6.1.1 | SHA-256 SHA-384 | Hash Function |
| TLS 1.2 (IETF RFC 5246 [38]) | TS 33.210 [2] clauses 6.2.1, 6.2.3 | AES_128_GCM, AES_256_GCM | Confidentiality and Integrity Protection |
|  |  | SHA256, SHA384 | Hash Function |
| TLS 1.3 (IETF RFC 8446 [21]) | TS 33.210 [2] clauses 6.2.1, 6.2.2 | AES_128_GCM, AES_256_GCM, CHACHA20_POLY1305 | Confidentiality and Integrity Protection |
|  |  | SHA-256, SHA-384 | Hash Function |

* * * Next Change * * * *

### 4.3.2	3GPP Asymmetric Cryptographic Algorithms

The following table summarizes the security related protocols used in 3GPP employing asymmetric cryptographic algorithms (5G System).

Table 4.3.2-1: Protocols Used in 3GPP Employing Asymmetric Cryptographic Algorithms (5G System)

| Protocol/Function | Protocol Profile, Clauses | Cryptographic Algorithm(s) | Feature(s), Usage Type |
| --- | --- | --- | --- |
| MIKEY-SAKKE (IETF RFC 6509) [14] | IETF RFC 6507 [29] | ECCSI | Digital signature |
|  | IETF RFC 6508 [30] | SAKKE | Key agreement |
| ECIES ([7], [8]) | TS 33.501 [4], Clause C.3 | ECDH | Key Agreement |
| PKI | TS 33.310 [3], Clause 6.1.1 | RSA, ECDSA | Authentication / Digital Signature |
| OCSP (IETF RFC 6960 [22]) | TS 33.310 [3], Clause 6.1b | RSA ECDSA | Authentication / Digital Signature |
| EAP-TLS (IETF RFCs 9190 [5], 5216 [6]) | TS 33.501 [4], Clause B.2.1 | See TLS in this table. | Authentication / Digital Signature / Confidentiality Protection / Hash Function |
|  | TS 33.501 [4] RFC 9190 (TLS1.3) [5] | ECDHE | Key Agreement |
| EAP-TTLS (IETF RFC 5281 [39]) | TS 33.501 [4], Annex U TS 33.210 [2] clause 6.2 for TLS | See TLS in this table | Key Agreement |
|  |  | See TLS in this table | Authentication / Digital Signature / Confidentiality Protection / Hash Function |
| OAuth 2.0 (IETF RFC 6749 [40], 6750 [41]) | TS 33.210 [2] clause 6.2 for TLS | See TLS 1.2 and TLS 1.3 in this table | Key Agreement |
|  |  |  | Digital Signature |
|  | TS 33.210 [2] clause 6.3 for JWE/JWS | See JWE and JWS in this table | Key Agreement |
|  |  |  | Digital Signature |
| IKEv2 (IETF RFC 7296 [23]) | TS 33.210 [2] clause 5.4 | DH | Key Agreement |
|  | TS 33.310 [3] clauses 5,6,7 | RSA Sha-256/384 (IETF RFC 8017 [45]) ECDSA SHA-256/384/512 (IETF RFC 4754 [46]) RSASSA-PSS SHA-256 [47] | Digital Signature |
|  |  | SHA2-256/384 [47] | Hash Function |
| DTLS 1.2 (IETF RFC 6347 [37]) | TS 33.210 [2] clause 6.2.1 | See TLS 1.2 in this table | Confidentiality and Integrity Protection |
| DTLS 1.3 (IETF RFC 9147 [20]) | TS 33.210 [2] clause 6.2.1 | See TLS 1.2 in this table | Confidentiality and Integrity Protection |
| TLS 1.2 (IETF RFC 5246 [38]) | TS 33.210 [2] clauses 6.2.1, 6.2.3 | ECDHE | Key Agreement |
|  |  | ECDSA, RSA | Digital Signature |
| TLS 1.3 (IETF RFC 8446 [21]) | TS 33.210 [2] clauses 6.2.1, 6.2.2 | ECDHE | Key Agreement |
|  |  | ECDSA, RSA | Digital Signature |
| JWE (IETF RFC 7516 [27]) | TS 33.210 [2] clauses 6.3.1, 6.3.2 | ECDH-ES | Key Agreement |
| JWS (IETF RFC 7515 [28]) | TS 33.210 [2] clauses 6.3.1, 6.3.3 | ECDSA | Digital Signature |

| Protocol/Function | Protocol Profile, Clauses | Cryptographic Algorithm(s) | Feature(s), Usage Type |
| --- | --- | --- | --- |
| DTLS 1.2 (IETF RFC 6347 [37]) | TS 33.210 [2] clause 6.2.1 | See TLS 1.2 in this table | Confidentiality and Integrity Protection |
| DTLS 1.3 (IETF RFC 9147 [20]) | TS 33.210 [2] clause 6.2.1 | See TLS 1.2 in this table | Confidentiality and Integrity Protection |
| EAP-TLS (IETF RFCs 9190 [5], 5216 [6]) | TS 33.501 [4], Clause B.2.1 | See TLS in this table. | Authentication / Digital Signature / Confidentiality Protection / Hash Function |
|  | TS 33.501 [4] RFC 9190 (TLS1.3) [5] | ECDHE | Key Agreement |
| EAP-TTLS (IETF RFC 5281 [39]) | TS 33.501 [4], Annex U TS 33.210 [2] clause 6.2 for TLS | See TLS in this table | Key Agreement |
|  |  | See TLS in this table | Authentication / Digital Signature / Confidentiality Protection / Hash Function |
| ECIES ([7], [8]) | TS 33.501 [4], Clause C.3 | ECDH | Key Agreement |
| IKEv2 (IETF RFC 7296 [23]) | TS 33.210 [2] clause 5.4 | DH | Key Agreement |
|  | TS 33.310 [3] clauses 5,6,7 | RSA Sha-256/384 (IETF RFC 8017 [45]) ECDSA SHA-256/384/512 (IETF RFC 4754 [46]) RSASSA-PSS SHA-256 [47] | Digital Signature |
|  |  | SHA2-256/384 [47] | Hash Function |
| JWE (IETF RFC 7516 [27]) | TS 33.210 [2] clauses 6.3.1, 6.3.2 | ECDH-ES | Key Agreement |
| JWS (IETF RFC 7515 [28]) | TS 33.210 [2] clauses 6.3.1, 6.3.3 | ECDSA | Digital Signature |
| MIKEY-SAKKE (IETF RFC 6509) [14] | IETF RFC 6507 [29] | ECCSI | Digital signature |
|  | IETF RFC 6508 [30] | SAKKE | Key agreement |
| OAuth 2.0 (IETF RFC 6749 [40], 6750 [41]) | TS 33.210 [2] clause 6.2 for TLS | See TLS 1.2 and TLS 1.3 in this table | Key Agreement |
|  |  |  | Digital Signature |
|  | TS 33.210 [2] clause 6.3 for JWE/JWS | See JWE and JWS in this table | Key Agreement |
|  |  |  | Digital Signature |
| OCSP (IETF RFC 6960 [22]) | TS 33.310 [3], Clause 6.1b | RSA ECDSA | Authentication / Digital Signature |
| PKI | TS 33.310 [3], Clause 6.1.1 | RSA, ECDSA | Authentication / Digital Signature |
| TLS 1.2 (IETF RFC 5246 [38]) | TS 33.210 [2] clauses 6.2.1, 6.2.3 | ECDHE | Key Agreement |
|  |  | ECDSA, RSA | Digital Signature |
| TLS 1.3 (IETF RFC 8446 [21]) | TS 33.210 [2] clauses 6.2.1, 6.2.2 | ECDHE | Key Agreement |
|  |  | ECDSA, RSA | Digital Signature |

* * * End of Changes * * * *