3GPP TSG-SA3 Meeting #125



















S3-254555
Dallas, USA, 17 – 21 Novmenber 2025












(revision of S3-254188)
	CR-Form-v12.1

	CHANGE REQUEST

	

	
	33.369
	CR
	0020
	rev
	3
	Current version:
	19.0.0
	

	

	For HELP on using this form: comprehensive instructions can be found at 
http://www.3gpp.org/Change-Requests.

	


	Proposed change affects:
	UICC apps
	
	ME
	X
	Radio Access Network
	
	Core Network
	X


	

	Title:

	Correction on privacy

	
	

	Source to WG:
	vivo, Lenovo, ZTE, OPPO, Qualcomm, Huawei, InterDigital, CATT

	Source to TSG:
	S3

	
	

	Work item code:
	AmbientIoT-SEC
	
	Date:
	2025-11-10

	
	
	
	
	

	Category:
	F
	
	Release:
	Rel-19

	
	Use one of the following categories:
F  (correction)
A  (mirror corresponding to a change in an earlier 












release)
B  (addition of feature), 
C  (functional modification of feature)
D  (editorial modification)

Detailed explanations of the above categories can
be found in 3GPP TR 21.900.
	Use one of the following releases:
Rel-8
(Release 8)
Rel-9
(Release 9)
Rel-10
(Release 10)
Rel-11
(Release 11)
…
Rel-15
(Release 15)
Rel-16
(Release 16)
Rel-17
(Release 17)
Rel-18
(Release 18)

	
	

	Reason for change:
	1. It is unclear how AIOTF retrieve TID from ADM
2. It is unclear how to update the T-ID when the stored T-ID shall be updated with a command

	
	

	Summary of change:
	1. Clarify that device permanent ID is used when privacy is not required.
2. Clarify AIOTF actions in case the stored T-ID update shall be done with a command.
3. Align interaction between AIOTF and ADM with service operations.
4. For stored T-ID update with command, the storing of new T-ID is triggered after command response is received.

	
	

	Consequences if not approved:
	Incomplete specification. 

	
	

	Clauses affected:
	5.4.2, 5.4.3, 5.4.4

	
	

	
	Y
	N
	
	

	Other specs
	
	X
	 Other core specifications

	TS/TR ... CR ... 

	affected:
	
	X
	 Test specifications
	TS/TR ... CR ... 

	(show related CRs)
	
	X
	 O&M Specifications
	TS/TR ... CR ... 

	
	

	Other comments:
	

	
	

	This CR's revision history:
	


***** START OF 1st CHANGE *****
5.4.2
The AIoT Device Identifier protection for inventory with filtering information

For the protection of AIoT Device Permanent Identifier during the inventory procedure described in clause 5.2.2, the following change shall apply: 

-
In step 4, the AIoT Device determines it needs to reply to the NG-RAN based on the received filtering information.

NOTE 1: The attacker may obtain an AIoT Device ID by performing a bitwise enumeration in multiple paging messages. To mitigate the attack, the AIoT Device need to be configured with filtering information to match by limiting which bits of AIoT Device Identifier is allowed for filtering information (guidance would be to limit to the leftmost n bits of the AIoT Permanent Device Identifier, e.g., only allow filtering information for the leftmost 64 bits and not respond otherwise).
-
In steps 5 and 6, the AIoT Identification Information is not included in the D2R message and Inventory Report message.
-
In step 7, Filtering Information is used as the AIoT Identification Information.
-
In step 9, ADM shall send the AIoT Device Permanent Identifier corresponding to XRESAIOT to AIOTF.
NOTE 2: The AIOTF identifies the AIoT device by checking the received RESAIoT. Therefore, AIoT Identification Information is not needed in the D2R and Inventory Report message.

NOTE 3: When inventory with filtering information is used, after receiving the D2R message, the ADM has to exhaustively derive XRESAIoTs with all the long-term keys (i.e., KAIoT_root) of the AIoT devices in the group that was paged for every RANDAIoT_d received. The AIOTF then, need to check XRESAIoT with the received RESAIoT. Therefore, the size of the group should be chosen accordingly to reduce the energy consumption, inter NF interaction, and latency.

***** START OF 2nd CHANGE *****
5.4.3
Procedure for AIoT Device Identifier protection with T-ID update during Individual inventory
For the protection of the AIoT Device Permanent Identifier during the inventory procedure described in clause 5.2.2, the following changes shall apply: 
-
In step 1, AIOTF shall invoke Nadm_SecTID_Get service operation to retrieve a T-ID in addition to the RANDAIOT_n from ADM using the AIoT Device Permanent Identifier. The ADM shall, based on T-ID handling information stored in the AIoT device profile, either fetch the stored T-ID in the AIoT Device profile or generate the T-ID based on the AIoT Device Permanent Identifier as specified in Annex B.1, and send the T-ID to the AIOTF.
-
In steps 2, 3, and 4, the T-ID shall be used as the AIoT Identification Information.

-
In step 2, the AIOTF includes the T-ID handling information in the Inventory Request message. T-ID handling information includes:
-
T-ID type that can be either a concealed type or a stored type. If the T-ID type is the concealed type, the T-ID is generated based on the AIoT Device Permanent Identifier. If the T-ID is the stored type, the T-ID is generated based on the stored T-ID as specified in Annex B.1.
-
Whether the stored T-ID type is updated with a command via a Command procedure or without a command during step 4.
-
In step 3, NG-RAN includes the T-ID handling information in the paging message.

-
In step 4, the AIoT device, based on the T-ID handling information in the paging message, generates the T-ID in the same way as the ADM did in step 1 if the T-ID type is concealed type, or retrieves the T-ID if the T-ID type is stored type. The AIoT device determines it needs to reply to the NG-RAN if the generated or retrieved T-ID matches the received T-ID. In case the stored T-ID update shall be done without a command, the AIoT Device generates a new T-ID_n+1 as specified in Annex B.1 and stores the new T-ID_n+1.

-
In steps 5 and 6, the AIoT Identification Information is not included in the D2R message and Inventory Report message.

-
In step 7, the AIoT Device Permanent Identifier is used as the AIoT Identification Information. If the T-ID type is stored type and the stored T-ID update is performed without the Command procedure, the AIOTF requests the ADM to derive a new T-ID as specified in Annex B.1 and to store it in the AIoT Device profile.
-
In step 10, if the T-ID type is stored type and the stored T-ID update is performed with the Command procedure, the AIOTF requests the ADM to generate and return a new T-ID.
NOTE 1:
The AIOTF identifies the AIoT Device by checking the received RESAIoT parameter. Therefore, the AIoT Identification Information is not needed in the D2R message and the Inventory Report message.

NOTE 2: 
In case of concealed T-ID type, every AIoT Device that receives an Inventory Request with T-ID needs to perform a T-ID matching by generating a T-ID based on the KAIoT_root and check if the generated T-ID matches the received T-ID. It is assumed that the AIoT Device that receives the Inventory Request has enough energy to perform this T-ID matching in addition to the Inventory procedure specified in clause 5.2.2.

NOTE 3: 
In the case of stored T-ID type, the stored T-IDs on the device side and network side can get out of synch. The handling of such situation is described in clause 5.4.4.
-
After step 10, if the stored T-ID update is performed via the Command procedure and the AIoT Device supports this, the AIOTF sends an encrypted AIOT NAS Command Request message containing the new T-ID requested from the ADM in step 10 to the AIoT Device. Upon receipt of the Command Request message, the AIoT Device stores the new T-ID. Upon receipt of the Command Response message, the AIOTF requests the ADM to store the new T-ID in the AIoT Device profile.
***** START OF 3rd CHANGE *****
5.4.4
Out-of-Synch detection and Resynchronization of T-ID

In case the network does not receive an Inventory Response from an AIoT Device after an Individual Inventory Request, it may occur because that the AIoT Device and the network are out of synch with the T-IDs. The out-of-synch can happen if e.g.:

-
The Inventory Response or Command Response from the Device was lost during transmission due to radio link issues e.g. interference, range, etc. in that case the AIoT Device would generate the T-ID_n+1, but the ADM would not generate the T-ID_n+1 or know that the device has received the T-ID_n+1 as it did not get any response.

-
Something went wrong during the Inventory or Command procedure e.g., the AIoT Device managed to write to the NVM but not send the inventory response or command response, or the AIoT Device sent the inventory response or command response but was not able to write to the NVM.

This means that the ADM either has a T-ID that is older or newer than the T-ID in the AIoT Device. They can never be more than one off.

T-ID sequence recovery is possible if the network performs Individual Inventory with T-ID_n-1 or T-ID_n+1. When the AIOTF invokes Nadm_SecTID_Get service operation with Resynchronization indicator to indicate T-ID sequence recovery to the ADM, the ADM provides T-ID_n-1 and/or T-ID_n+1 of the AIoT device to the AIOTF. When the AIoT device responds to the network, the network adjusts the sequence, and both are in synch again. 

Alternatively, the network can use concealed T-ID type using the permanent identifier and then send a command to provide a new T-ID to the device which it stores in the device.

***** END OF CHANGES *****
