3GPP TSG-SA3 Meeting #123	S3-252674

# Goteborg, Sweden, 25 – 29 August 2025

Source:	Huawei, HiSilicon

Title:	New key issue on Security between network and Sensing service consumer

Document for:	Approval

Agenda item:	6.1.8

Spec:	3GPP TS/TR <TS/TR number>

Version:	<TS version>

Work Item:	FS_Sensing_SEC

Comments

This contribution aims to provide a potential Key Issue on security between network and Sensing service consumer for the Sensing study.

* * * First Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

[a1]	3GPP TR 23.700-14: " Study on Integrated Sensing and Communication; Stage 2".

[a2]	3GPP TS 22.137: "Service requirements for Integrated Sensing and Communication; Stage 1".

* * * Second Change * * * *

Y.X	Key Issue #X: Security between network and Sensing service consumer

Y.X.1	Key issue details

The core network entities (e.g. Sensing Function (SF)) decides how to perform the Sensing operations based on the parameters in the Sensing request from the Sensing service consumer [a1], including discovery and selection of sensing entities/functions, sensing data collection. Furthermore, the core network entity exposes the Sensing result to the Sensing service consumer. The Sensing service consumer may use the Sensing result for e.g. UAV collision avoidance. The Sensing service consumer may be treated as an entity in 3GPP system, or entity belongs to a third party, depending on different scenarios [a2].

How to perform the Sensing operations is decided based on the parameters in the Sensing request from the Sensing service consumer. Thus the Sensing service consumer shall be authenticated to prevent illegal access to the Sensing services provided by 3GPP systems. The 3GPP system also need to check whether the consumer is authorised to perform specific Sensing request.

The Sensing result corresponding to the Sensing request is exposed to the Sensing service consumer. The authentication and authorisation of the Sensing service consumer also needs to be assured by the 3GPP system to prevent the Sensing result is exposed to false consumer.

Also, 3GPP system shall be able to secure the message exchange between Sensing service consumer and the network entity, this involves the Sensing request and Sensing result exposure.

Y.X.2	Security threats

If the Sensing request and Sensing result are not integrity and replay protected, the parameters included can be modified, or replayed by an attacker. Consequently, the sensing operation may be executed with incorrect purpose/service requirements, or unauthorised triggered, the modified Sensing Result may lead to potential risks to public safety (e.g. UAV collision, or hiding problematic/misbehaving UAS).

The Sensing result may be eavesdropped by an attacker if the Sensing data exposure procedures are not confidentiality protected.

Without authentication and authorisation of the Sensing service consumer, the sensing operations can be unauthorised triggered by an untrusted consumer which may be a malicious entity, or the sensing may be executed with a sensing request, including incorrect purpose/service requirements, forged by an attacker.

If authentication and authorisation are not performed, unauthorised Sensing service consumer may access the Sensing services provided by 3GPP and consume resources. The Sensing service resource is critical as it may serves for the purpose of public safety (e.g. UAV collision, or hiding problematic/misbehaving UAS).

The Sensing data may be consumed by an unauthorised entity if the Sensing service consumer is not authenticated and authorised for data exposure.

Y.X.3	Potential security requirements

The 5G System shall provide the means to offer integrity protection, confidentiality protection and replay protection of the connection between network and Sensing service consumer.

The Sensing service consumer shall be authenticated and authorised by the 3GPP systems.

* * * End of Changes * * * *